STM32 Offline AI Product Development Handbook

100 factory and business products • cost-optimized MCU selection • offline step-by-step workflow

Uses STM32U5 for sensor/time-series AI where possible.

Uses STM32H7 for controlled camera/DSP/OCR workloads.

Uses STM32N6 only where accelerated vision is justified.

Every project is designed to be developed without Internet access after the initial software/document download stage.

Prepared for offline-first embedded AI development • 17 October 2026

# How to Use This Handbook

Each product section gives a cost-first STM32 class, sensor stack, AI formulation, offline workflow, validation targets, cost-down notes and the condition that would justify moving to a more powerful MCU. The recommendations are starting architectures, not final component selections. Final MCU/package selection must be based on measured memory, latency, peripheral and environmental requirements.

Safety and compliance: Projects involving worker safety, vehicles, access control, fire/gas detection, medical/health interpretation, or other safety/security functions require domain-specific engineering and may require certification. AI output should not be treated as a sole protective function unless the complete system is designed and certified accordingly.

# 1. Offline Development Foundation for All 100 Products

## 1.1 MCU selection rule

| Family | Use first when | Typical AI formulation | Escalate when |
| --- | --- | --- | --- |
| STM32U5 | Inputs are vibration/audio/current/temp/pressure/ToF or a tiny fixed-state classifier | 1D CNN, MLP, anomaly detector, tiny spectrogram CNN, sensor fusion | DSP/model latency or memory no longer has margin |
| STM32H7 | Camera is fixed, scene is controlled, ROI is known, or image rate is modest | Image classifier, anomaly model, keypoints, compact OCR, tiny detector | Need complex full-scene detection/segmentation/pose/tracking |
| STM32N6 | Scene is variable or model needs accelerated computer vision | Detector, segmentation, pose, depth, re-ID, larger audio/vision network | Only cost-down if the task can be reformulated for H7 |

## 1.2 One-time download bundle before going offline

STM32CubeIDE, STM32CubeMX and STM32CubeProgrammer installers.

STM32Cube AI Studio / ST Edge AI Core local installer and documentation.

STM32CubeU5, STM32CubeH7 and STM32CubeN6 firmware packages for the boards/products you may use.

Board manuals, schematics, MCU datasheets, reference manuals and current errata PDFs.

Local copies of AI examples/model repositories you will actually use.

Python installer plus an offline wheelhouse for NumPy, SciPy, OpenCV, PyTorch/TensorFlow, ONNX/TFLite tooling, plotting, data-loading and project-specific packages.

Annotation software installers / local Docker images.

Pretrained weights and public datasets that you are legally allowed to use, stored locally with license files.

## 1.3 Universal offline project lifecycle

Product requirement: exact input, output, response time, accuracy, environment, interfaces and safety role.

Bench fixture: final-like sensor mounting, lens, lighting, mechanical coupling and power.

Raw data acquisition: collect from the exact prototype sensor stack; never train only on Internet images if deployment conditions are industrial.

Dataset governance: immutable raw data, reproducible preprocessing, labels, metadata, train/validation/final-test split.

Smallest-model-first training: begin with deterministic rules or classical features; add a neural model only where it materially improves the result.

Quantization: INT8 where supported; compare metrics before/after.

Target analysis: use AI Studio / ST Edge AI Core offline for memory/operator/performance analysis.

Firmware integration: acquisition, preprocessing, inference, postprocessing, hysteresis/state machine, watchdog, self-test and communications.

Target verification: compare MCU output to PC reference on identical test vectors.

Pilot field trial: run in shadow/advisory mode first; collect hard cases.

Retrain/fix: add hard cases without contaminating the final test set.

Release: lock firmware/model/config/tool versions and hashes.

Custom PCB: select the cheapest STM32 SKU with sufficient measured margin.

Production test: program offline, verify sensor IDs/calibration, run known inference vectors and log serial-numbered test results.

## 1.4 Common offline folder structure

D:\STM32_AI_PRODUCTS\
  installers\
  official_docs\
  cube_packages\
  edge_ai\
  python_wheelhouse\
  pretrained_weights\
  tools\
  products\
    P001_defect_inspection\
      raw_data\
      labels\
      processed\
      models\
      firmware\
      tests\
      releases\
    ...
  production\
  backups\

## 1.5 Development boards when you have no writer/programmer

| Workload | Recommended development board | Why |
| --- | --- | --- |
| Sensor AI / U5 | B-U585I-IOT02A | On-board STLINK-V3E and many sensors; convenient for vibration/audio/environmental prototypes. |
| Camera / H7 | STM32H747I-DISCO + B-CAMS-OMV | On-board STLINK-V3E, camera connector, external memory and official camera bundle. |
| Accelerated vision / N6 | STM32N6570-DK | On-board ST-LINK and camera/AI-oriented hardware for N6 prototypes. |

These boards eliminate the need for an external programmer during early development. A standalone ST-LINK is needed once you move to your own PCB.

# 2. 100-Project Index

| # | Product | Category | Cost-first MCU |
| --- | --- | --- | --- |
| 1 | AI defect inspection camera | Manufacturing quality | STM32H7 |
| 2 | AI assembly verification camera | Manufacturing quality | STM32H7 |
| 3 | AI label inspection camera | Packaging | STM32H7 |
| 4 | AI cap/seal inspection camera | Packaging | STM32H7 |
| 5 | AI barcode/OCR inspection unit | Packaging/logistics | STM32H7 |
| 6 | AI PCB inspection camera | Electronics | STM32H7 |
| 7 | AI weld inspection camera | Metal fabrication | STM32H7 |
| 8 | AI surface scratch/dent detector | Manufacturing quality | STM32H7 |
| 9 | AI dimensional/shape inspection sensor | Machining | STM32H7 |
| 10 | AI foreign-object detector | Food/pharma | STM32H7 |
| 11 | AI product counter | Conveyor | STM32H7 |
| 12 | AI conveyor jam detector | Conveyor | STM32H7 |
| 13 | AI conveyor sorting sensor | Conveyor/logistics | STM32H7 |
| 14 | AI product orientation detector | Conveyor | STM32H7 |
| 15 | AI missing-part detector | Assembly | STM32H7 |
| 16 | AI screw/bolt verification camera | Assembly | STM32H7 |
| 17 | AI cable/connector verification camera | Assembly | STM32H7 |
| 18 | AI tool verification camera | Assembly | STM32H7 |
| 19 | AI process-step verification camera | Assembly | STM32N6 |
| 20 | AI worker PPE detector | Industrial safety | STM32N6 |
| 21 | AI restricted-zone safety camera | Industrial safety | STM32N6 |
| 22 | AI hand/arm danger-zone detector | Industrial safety | STM32N6 |
| 23 | AI worker fall detector | Industrial safety | STM32N6 |
| 24 | AI forklift pedestrian detector | Warehouse safety | STM32N6 |
| 25 | AI forklift collision-warning unit | Warehouse safety | STM32N6 |
| 26 | AI AGV obstacle detector | Robotics | STM32N6 |
| 27 | AI robot/cobot safety camera | Robotics safety | STM32N6 |
| 28 | AI robot object-location camera | Robotics | STM32N6 |
| 29 | AI robotic bin-picking camera | Robotics | STM32N6 |
| 30 | AI pallet detector | Warehouse | STM32H7 |
| 31 | AI pallet-condition inspection camera | Warehouse | STM32H7 |
| 32 | AI package damage detector | Logistics | STM32H7 |
| 33 | AI parcel dimension/classification unit | Logistics | STM32H7 |
| 34 | AI loading-dock monitoring camera | Logistics | STM32N6 |
| 35 | AI truck-loading verification camera | Logistics | STM32N6 |
| 36 | AI container loading verifier | Logistics | STM32N6 |
| 37 | AI warehouse shelf inventory camera | Warehouse | STM32H7 |
| 38 | AI empty-bin detector | Warehouse/production | STM32U5 |
| 39 | AI machine-health sensor | Predictive maintenance | STM32U5 |
| 40 | AI bearing-failure sensor | Predictive maintenance | STM32U5 |
| 41 | AI motor anomaly detector | Predictive maintenance | STM32U5 |
| 42 | AI pump anomaly detector | Predictive maintenance | STM32U5 |
| 43 | AI fan/HVAC health detector | Building/industrial | STM32U5 |
| 44 | AI compressor monitoring sensor | Predictive maintenance | STM32U5 |
| 45 | AI gearbox health sensor | Predictive maintenance | STM32U5 |
| 46 | AI machine sound analyzer | Predictive maintenance | STM32U5 |
| 47 | AI vibration-monitoring node | Predictive maintenance | STM32U5 |
| 48 | AI leak detection sensor | Industrial monitoring | STM32U5 |
| 49 | AI smoke/flame visual detector | Industrial safety | STM32H7 |
| 50 | AI gas-anomaly sensor | Industrial safety | STM32U5 |
| 51 | AI people counter | Buildings/retail | STM32N6 |
| 52 | AI occupancy sensor | Buildings | STM32H7 |
| 53 | AI queue-length camera | Retail/service | STM32N6 |
| 54 | AI parking-space sensor | Parking | STM32H7 |
| 55 | AI vehicle counting unit | Transport/logistics | STM32N6 |
| 56 | AI license-plate edge camera | Access/parking | STM32N6 |
| 57 | AI access-control terminal | Buildings | STM32N6 |
| 58 | AI face + liveness access terminal | Security | STM32N6 |
| 59 | AI elevator occupancy sensor | Buildings | STM32H7 |
| 60 | AI escalator safety camera | Public facilities | STM32N6 |
| 61 | AI smart-retail shelf camera | Retail | STM32H7 |
| 62 | AI refrigerator inventory camera | Retail | STM32H7 |
| 63 | AI vending-machine inventory sensor | Retail | STM32H7 |
| 64 | AI automatic checkout camera | Retail | STM32N6 |
| 65 | AI produce recognition unit | Retail | STM32H7 |
| 66 | AI food quality grader | Food processing | STM32H7 |
| 67 | AI fruit sorting/grading camera | Agriculture/food | STM32H7 |
| 68 | AI vegetable grading camera | Agriculture/food | STM32H7 |
| 69 | AI egg quality inspection camera | Food | STM32H7 |
| 70 | AI fish grading/counting camera | Food/aquaculture | STM32N6 |
| 71 | AI meat inspection camera | Food | STM32H7 |
| 72 | AI bakery product inspection camera | Food | STM32H7 |
| 73 | AI crop disease camera | Agriculture | STM32H7 |
| 74 | AI weed-vs-crop detector | Agriculture | STM32N6 |
| 75 | AI fruit-picking vision unit | Agriculture robotics | STM32N6 |
| 76 | AI livestock counting camera | Agriculture | STM32N6 |
| 77 | AI livestock behavior monitor | Agriculture | STM32N6 |
| 78 | AI feed-level camera | Agriculture | STM32H7 |
| 79 | AI waste-sorting camera | Recycling | STM32N6 |
| 80 | AI plastic classification sensor | Recycling | STM32H7 |
| 81 | AI metal/non-metal sorting sensor | Recycling | STM32U5 |
| 82 | AI contamination detector for recycling | Recycling | STM32H7 |
| 83 | AI smart trash-bin sensor | Waste management | STM32U5 |
| 84 | AI solar-panel defect camera | Energy | STM32N6 |
| 85 | AI wind-turbine monitoring camera | Energy | STM32H7 |
| 86 | AI electrical-panel inspection camera | Electrical maintenance | STM32H7 |
| 87 | AI meter-reading camera | Utilities | STM32H7 |
| 88 | AI pipe/gauge-reading camera | Industrial monitoring | STM32H7 |
| 89 | AI analog-dial monitoring unit | Industrial retrofit | STM32H7 |
| 90 | AI tank/hopper level camera | Industrial process | STM32H7 |
| 91 | AI construction safety camera | Construction | STM32N6 |
| 92 | AI hard-hat/vest detector | Construction/mining | STM32N6 |
| 93 | AI crane-zone safety detector | Construction/ports | STM32N6 |
| 94 | AI mining equipment proximity unit | Mining | STM32N6 |
| 95 | AI port container inspection camera | Ports | STM32N6 |
| 96 | AI railway track/equipment inspection unit | Rail | STM32N6 |
| 97 | AI bus/truck blind-spot camera | Fleet safety | STM32N6 |
| 98 | AI fleet driver-monitoring unit | Fleet safety | STM32N6 |
| 99 | AI commercial vehicle cargo monitor | Logistics fleets | STM32H7 |
| 100 | AI machine-operator monitoring unit | Industrial operations | STM32N6 |

# Category — Manufacturing quality

## 1. AI defect inspection camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Factories; machine OEMs |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | Fixed RGB camera + controlled lighting + trigger |
| AI formulation | Visual defect classification or anomaly detection |
| Product output | PASS/FAIL + defect code |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Use a rigid fixture and controlled illumination. Prefer classification/anomaly detection over full object detection.

### Offline step-by-step development

Write a one-page requirement for “AI defect inspection camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using Fixed RGB camera + controlled lighting + trigger. Fix sensor mounting before collecting the main dataset.

Capture raw images from the real prototype under the full expected range of normal conditions. Keep originals immutable and store metadata (batch, lighting, product type, camera settings, date/site).

Define the smallest useful region of interest and input resolution. If the object position is fixed, crop it before inference instead of asking the network to search the whole frame.

Label/prepare data for Visual defect classification or anomaly detection. Split train/validation/final-test sets by batch/site when possible.

Train locally on the workstation. Start with a compact CNN or keypoint/OCR pipeline; only move to a detector/segmenter if the simpler formulation cannot meet the requirement.

Quantize to INT8 and compare final-test metrics before and after quantization. Save the calibration image list.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (PASS/FAIL + defect code).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Factories; machine OEMs). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Final-test images come from a different batch/day/site where possible.

Measure false accept and false reject separately; do not report accuracy alone.

Test lighting high/low limits, dirty lens, small position shifts and camera restart.

Measure trigger-to-output latency under worst-case processing load.

Run at least a multi-hour or multi-thousand-cycle soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

## 2. AI assembly verification camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Assembly plants |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB camera + controlled lighting |
| AI formulation | Part presence/orientation classifier; optional tiny detector |
| Product output | Assembly complete / missing / wrong part |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Use a rigid fixture and controlled illumination. Prefer classification/anomaly detection over full object detection.

### Offline step-by-step development

Write a one-page requirement for “AI assembly verification camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera + controlled lighting. Fix sensor mounting before collecting the main dataset.

Capture raw images from the real prototype under the full expected range of normal conditions. Keep originals immutable and store metadata (batch, lighting, product type, camera settings, date/site).

Define the smallest useful region of interest and input resolution. If the object position is fixed, crop it before inference instead of asking the network to search the whole frame.

Label/prepare data for Part presence/orientation classifier; optional tiny detector. Split train/validation/final-test sets by batch/site when possible.

Train locally on the workstation. Start with a compact CNN or keypoint/OCR pipeline; only move to a detector/segmenter if the simpler formulation cannot meet the requirement.

Quantize to INT8 and compare final-test metrics before and after quantization. Save the calibration image list.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Assembly complete / missing / wrong part).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Assembly plants). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Final-test images come from a different batch/day/site where possible.

Measure false accept and false reject separately; do not report accuracy alone.

Test lighting high/low limits, dirty lens, small position shifts and camera restart.

Measure trigger-to-output latency under worst-case processing load.

Run at least a multi-hour or multi-thousand-cycle soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

# Category — Packaging

## 3. AI label inspection camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Food, beverage, cosmetics, pharma |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB camera + trigger + fixed lighting |
| AI formulation | Label presence/orientation/print classifier; optional OCR |
| Product output | Label OK / missing / skew / wrong |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Synchronize capture with the package trigger. Keep label/cap/seal ROI fixed.

### Offline step-by-step development

Write a one-page requirement for “AI label inspection camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera + trigger + fixed lighting. Fix sensor mounting before collecting the main dataset.

Capture raw images from the real prototype under the full expected range of normal conditions. Keep originals immutable and store metadata (batch, lighting, product type, camera settings, date/site).

Define the smallest useful region of interest and input resolution. If the object position is fixed, crop it before inference instead of asking the network to search the whole frame.

Label/prepare data for Label presence/orientation/print classifier; optional OCR. Split train/validation/final-test sets by batch/site when possible.

Train locally on the workstation. Start with a compact CNN or keypoint/OCR pipeline; only move to a detector/segmenter if the simpler formulation cannot meet the requirement.

Quantize to INT8 and compare final-test metrics before and after quantization. Save the calibration image list.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Label OK / missing / skew / wrong).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Food, beverage, cosmetics, pharma). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Final-test images come from a different batch/day/site where possible.

Measure false accept and false reject separately; do not report accuracy alone.

Test lighting high/low limits, dirty lens, small position shifts and camera restart.

Measure trigger-to-output latency under worst-case processing load.

Run at least a multi-hour or multi-thousand-cycle soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

## 4. AI cap/seal inspection camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Bottling/packaging factories |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB camera + backlight/ring light |
| AI formulation | Binary/multiclass classifier; anomaly detection |
| Product output | Cap/seal OK or reject |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Synchronize capture with the package trigger. Keep label/cap/seal ROI fixed.

### Offline step-by-step development

Write a one-page requirement for “AI cap/seal inspection camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera + backlight/ring light. Fix sensor mounting before collecting the main dataset.

Capture raw images from the real prototype under the full expected range of normal conditions. Keep originals immutable and store metadata (batch, lighting, product type, camera settings, date/site).

Define the smallest useful region of interest and input resolution. If the object position is fixed, crop it before inference instead of asking the network to search the whole frame.

Label/prepare data for Binary/multiclass classifier; anomaly detection. Split train/validation/final-test sets by batch/site when possible.

Train locally on the workstation. Start with a compact CNN or keypoint/OCR pipeline; only move to a detector/segmenter if the simpler formulation cannot meet the requirement.

Quantize to INT8 and compare final-test metrics before and after quantization. Save the calibration image list.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Cap/seal OK or reject).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Bottling/packaging factories). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Final-test images come from a different batch/day/site where possible.

Measure false accept and false reject separately; do not report accuracy alone.

Test lighting high/low limits, dirty lens, small position shifts and camera restart.

Measure trigger-to-output latency under worst-case processing load.

Run at least a multi-hour or multi-thousand-cycle soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

# Category — Packaging/logistics

## 5. AI barcode/OCR inspection unit

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Packaging, logistics, manufacturing |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB camera + controlled lighting |
| AI formulation | ROI detector + compact OCR/barcode decoding |
| Product output | Code read + quality/validity result |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Constrain the product problem and choose the smallest model that solves it.

### Offline step-by-step development

Write a one-page requirement for “AI barcode/OCR inspection unit”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera + controlled lighting. Fix sensor mounting before collecting the main dataset.

Collect representative data locally.

Prepare labels and train the smallest suitable model.

Quantize and validate on the final test set.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Code read + quality/validity result).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Packaging, logistics, manufacturing). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test on machines/sessions not represented in training.

Sweep operating speed/load/temperature and verify the model does not confuse operating condition with failure.

Compare MCU preprocessing/inference with the offline Python reference on identical windows.

Test sensor disconnect, saturation and mounting changes.

Run a long-duration data-logging and inference soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

# Category — Electronics

## 6. AI PCB inspection camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Electronics manufacturers |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | High-quality RGB camera + fixture + light |
| AI formulation | Component presence/orientation classifier; anomaly detection |
| Product output | Missing/wrong/reversed component |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Constrain the product problem and choose the smallest model that solves it.

### Offline step-by-step development

Write a one-page requirement for “AI PCB inspection camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using High-quality RGB camera + fixture + light. Fix sensor mounting before collecting the main dataset.

Collect representative data locally.

Prepare labels and train the smallest suitable model.

Quantize and validate on the final test set.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Missing/wrong/reversed component).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Electronics manufacturers). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test on machines/sessions not represented in training.

Sweep operating speed/load/temperature and verify the model does not confuse operating condition with failure.

Compare MCU preprocessing/inference with the offline Python reference on identical windows.

Test sensor disconnect, saturation and mounting changes.

Run a long-duration data-logging and inference soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

# Category — Metal fabrication

## 7. AI weld inspection camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Automotive; welding lines |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB camera + stable illumination |
| AI formulation | Surface anomaly/classification model |
| Product output | Weld quality class / anomaly score |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Constrain the product problem and choose the smallest model that solves it.

### Offline step-by-step development

Write a one-page requirement for “AI weld inspection camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera + stable illumination. Fix sensor mounting before collecting the main dataset.

Collect representative data locally.

Prepare labels and train the smallest suitable model.

Quantize and validate on the final test set.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Weld quality class / anomaly score).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Automotive; welding lines). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test on machines/sessions not represented in training.

Sweep operating speed/load/temperature and verify the model does not confuse operating condition with failure.

Compare MCU preprocessing/inference with the offline Python reference on identical windows.

Test sensor disconnect, saturation and mounting changes.

Run a long-duration data-logging and inference soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

# Category — Manufacturing quality

## 8. AI surface scratch/dent detector

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Metal, glass, plastics, automotive |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB camera + directional light |
| AI formulation | Anomaly detection or segmentation |
| Product output | Defect score/location |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Use a rigid fixture and controlled illumination. Prefer classification/anomaly detection over full object detection.

### Offline step-by-step development

Write a one-page requirement for “AI surface scratch/dent detector”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera + directional light. Fix sensor mounting before collecting the main dataset.

Capture raw images from the real prototype under the full expected range of normal conditions. Keep originals immutable and store metadata (batch, lighting, product type, camera settings, date/site).

Define the smallest useful region of interest and input resolution. If the object position is fixed, crop it before inference instead of asking the network to search the whole frame.

Label/prepare data for Anomaly detection or segmentation. Split train/validation/final-test sets by batch/site when possible.

Train locally on the workstation. Start with a compact CNN or keypoint/OCR pipeline; only move to a detector/segmenter if the simpler formulation cannot meet the requirement.

Quantize to INT8 and compare final-test metrics before and after quantization. Save the calibration image list.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Defect score/location).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Metal, glass, plastics, automotive). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Final-test images come from a different batch/day/site where possible.

Measure false accept and false reject separately; do not report accuracy alone.

Test lighting high/low limits, dirty lens, small position shifts and camera restart.

Measure trigger-to-output latency under worst-case processing load.

Run at least a multi-hour or multi-thousand-cycle soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

# Category — Machining

## 9. AI dimensional/shape inspection sensor

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | CNC, injection molding |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | Camera + reference scale / optional laser/ToF |
| AI formulation | Keypoint/contour model + geometry |
| Product output | Dimension/shape tolerance result |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Calibrate pixels-to-mm or reference geometry; separate AI feature detection from deterministic dimensional calculations.

### Offline step-by-step development

Write a one-page requirement for “AI dimensional/shape inspection sensor”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using Camera + reference scale / optional laser/ToF. Fix sensor mounting before collecting the main dataset.

Capture raw images from the real prototype under the full expected range of normal conditions. Keep originals immutable and store metadata (batch, lighting, product type, camera settings, date/site).

Define the smallest useful region of interest and input resolution. If the object position is fixed, crop it before inference instead of asking the network to search the whole frame.

Label/prepare data for Keypoint/contour model + geometry. Split train/validation/final-test sets by batch/site when possible.

Train locally on the workstation. Start with a compact CNN or keypoint/OCR pipeline; only move to a detector/segmenter if the simpler formulation cannot meet the requirement.

Quantize to INT8 and compare final-test metrics before and after quantization. Save the calibration image list.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Dimension/shape tolerance result).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (CNC, injection molding). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Final-test images come from a different batch/day/site where possible.

Measure false accept and false reject separately; do not report accuracy alone.

Test lighting high/low limits, dirty lens, small position shifts and camera restart.

Measure trigger-to-output latency under worst-case processing load.

Run at least a multi-hour or multi-thousand-cycle soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

# Category — Food/pharma

## 10. AI foreign-object detector

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Food, pharma, production lines |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB camera + controlled background |
| AI formulation | Anomaly detection / object classifier |
| Product output | Foreign object alarm |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Control background and illumination; include hygiene/cleaning changes in the dataset.

### Offline step-by-step development

Write a one-page requirement for “AI foreign-object detector”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera + controlled background. Fix sensor mounting before collecting the main dataset.

Capture raw images from the real prototype under the full expected range of normal conditions. Keep originals immutable and store metadata (batch, lighting, product type, camera settings, date/site).

Define the smallest useful region of interest and input resolution. If the object position is fixed, crop it before inference instead of asking the network to search the whole frame.

Label/prepare data for Anomaly detection / object classifier. Split train/validation/final-test sets by batch/site when possible.

Train locally on the workstation. Start with a compact CNN or keypoint/OCR pipeline; only move to a detector/segmenter if the simpler formulation cannot meet the requirement.

Quantize to INT8 and compare final-test metrics before and after quantization. Save the calibration image list.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Foreign object alarm).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Food, pharma, production lines). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Final-test images come from a different batch/day/site where possible.

Measure false accept and false reject separately; do not report accuracy alone.

Test lighting high/low limits, dirty lens, small position shifts and camera restart.

Measure trigger-to-output latency under worst-case processing load.

Run at least a multi-hour or multi-thousand-cycle soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

# Category — Conveyor

## 11. AI product counter

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Factories/packaging |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | Camera or ToF + trigger |
| AI formulation | Tiny object detector or classifier+tracking |
| Product output | Count / rate |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Synchronize camera frames with conveyor speed/encoder/trigger. Model timing and reject position.

### Offline step-by-step development

Write a one-page requirement for “AI product counter”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using Camera or ToF + trigger. Fix sensor mounting before collecting the main dataset.

Capture raw images from the real prototype under the full expected range of normal conditions. Keep originals immutable and store metadata (batch, lighting, product type, camera settings, date/site).

Define the smallest useful region of interest and input resolution. If the object position is fixed, crop it before inference instead of asking the network to search the whole frame.

Label/prepare data for Tiny object detector or classifier+tracking. Split train/validation/final-test sets by batch/site when possible.

Train locally on the workstation. Start with a compact CNN or keypoint/OCR pipeline; only move to a detector/segmenter if the simpler formulation cannot meet the requirement.

Quantize to INT8 and compare final-test metrics before and after quantization. Save the calibration image list.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Count / rate).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Factories/packaging). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Final-test images come from a different batch/day/site where possible.

Measure false accept and false reject separately; do not report accuracy alone.

Test lighting high/low limits, dirty lens, small position shifts and camera restart.

Measure trigger-to-output latency under worst-case processing load.

Run at least a multi-hour or multi-thousand-cycle soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

## 12. AI conveyor jam detector

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Factories/distribution centers |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | Camera + motor current optional |
| AI formulation | Scene anomaly / motion state model |
| Product output | Jam / slow flow / normal |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Synchronize camera frames with conveyor speed/encoder/trigger. Model timing and reject position.

### Offline step-by-step development

Write a one-page requirement for “AI conveyor jam detector”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using Camera + motor current optional. Fix sensor mounting before collecting the main dataset.

Capture raw images from the real prototype under the full expected range of normal conditions. Keep originals immutable and store metadata (batch, lighting, product type, camera settings, date/site).

Define the smallest useful region of interest and input resolution. If the object position is fixed, crop it before inference instead of asking the network to search the whole frame.

Label/prepare data for Scene anomaly / motion state model. Split train/validation/final-test sets by batch/site when possible.

Train locally on the workstation. Start with a compact CNN or keypoint/OCR pipeline; only move to a detector/segmenter if the simpler formulation cannot meet the requirement.

Quantize to INT8 and compare final-test metrics before and after quantization. Save the calibration image list.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Jam / slow flow / normal).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Factories/distribution centers). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Final-test images come from a different batch/day/site where possible.

Measure false accept and false reject separately; do not report accuracy alone.

Test lighting high/low limits, dirty lens, small position shifts and camera restart.

Measure trigger-to-output latency under worst-case processing load.

Run at least a multi-hour or multi-thousand-cycle soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

# Category — Conveyor/logistics

## 13. AI conveyor sorting sensor

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Manufacturing/logistics |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB camera + encoder/trigger |
| AI formulation | Object classification/detection |
| Product output | Class + position for diverter |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Synchronize detection with conveyor motion and diverter timing.

### Offline step-by-step development

Write a one-page requirement for “AI conveyor sorting sensor”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera + encoder/trigger. Fix sensor mounting before collecting the main dataset.

Capture raw images from the real prototype under the full expected range of normal conditions. Keep originals immutable and store metadata (batch, lighting, product type, camera settings, date/site).

Define the smallest useful region of interest and input resolution. If the object position is fixed, crop it before inference instead of asking the network to search the whole frame.

Label/prepare data for Object classification/detection. Split train/validation/final-test sets by batch/site when possible.

Train locally on the workstation. Start with a compact CNN or keypoint/OCR pipeline; only move to a detector/segmenter if the simpler formulation cannot meet the requirement.

Quantize to INT8 and compare final-test metrics before and after quantization. Save the calibration image list.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Class + position for diverter).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Manufacturing/logistics). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Final-test images come from a different batch/day/site where possible.

Measure false accept and false reject separately; do not report accuracy alone.

Test lighting high/low limits, dirty lens, small position shifts and camera restart.

Measure trigger-to-output latency under worst-case processing load.

Run at least a multi-hour or multi-thousand-cycle soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

# Category — Conveyor

## 14. AI product orientation detector

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Packaging/robotic handling |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB camera + fixed background |
| AI formulation | Orientation classifier/keypoints |
| Product output | Angle/orientation class |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Synchronize camera frames with conveyor speed/encoder/trigger. Model timing and reject position.

### Offline step-by-step development

Write a one-page requirement for “AI product orientation detector”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera + fixed background. Fix sensor mounting before collecting the main dataset.

Capture raw images from the real prototype under the full expected range of normal conditions. Keep originals immutable and store metadata (batch, lighting, product type, camera settings, date/site).

Define the smallest useful region of interest and input resolution. If the object position is fixed, crop it before inference instead of asking the network to search the whole frame.

Label/prepare data for Orientation classifier/keypoints. Split train/validation/final-test sets by batch/site when possible.

Train locally on the workstation. Start with a compact CNN or keypoint/OCR pipeline; only move to a detector/segmenter if the simpler formulation cannot meet the requirement.

Quantize to INT8 and compare final-test metrics before and after quantization. Save the calibration image list.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Angle/orientation class).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Packaging/robotic handling). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Final-test images come from a different batch/day/site where possible.

Measure false accept and false reject separately; do not report accuracy alone.

Test lighting high/low limits, dirty lens, small position shifts and camera restart.

Measure trigger-to-output latency under worst-case processing load.

Run at least a multi-hour or multi-thousand-cycle soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

# Category — Assembly

## 15. AI missing-part detector

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Assembly factories |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB camera + fixed ROI |
| AI formulation | Binary/multiclass classifier |
| Product output | Missing part / present |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Fix camera and workstation geometry. Use the smallest ROI/class set possible.

### Offline step-by-step development

Write a one-page requirement for “AI missing-part detector”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera + fixed ROI. Fix sensor mounting before collecting the main dataset.

Capture raw images from the real prototype under the full expected range of normal conditions. Keep originals immutable and store metadata (batch, lighting, product type, camera settings, date/site).

Define the smallest useful region of interest and input resolution. If the object position is fixed, crop it before inference instead of asking the network to search the whole frame.

Label/prepare data for Binary/multiclass classifier. Split train/validation/final-test sets by batch/site when possible.

Train locally on the workstation. Start with a compact CNN or keypoint/OCR pipeline; only move to a detector/segmenter if the simpler formulation cannot meet the requirement.

Quantize to INT8 and compare final-test metrics before and after quantization. Save the calibration image list.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Missing part / present).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Assembly factories). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Final-test images come from a different batch/day/site where possible.

Measure false accept and false reject separately; do not report accuracy alone.

Test lighting high/low limits, dirty lens, small position shifts and camera restart.

Measure trigger-to-output latency under worst-case processing load.

Run at least a multi-hour or multi-thousand-cycle soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

## 16. AI screw/bolt verification camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Automotive/electronics |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB camera + controlled lighting |
| AI formulation | Keypoint/object classifier |
| Product output | Count/presence/tightening mark state |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Fix camera and workstation geometry. Use the smallest ROI/class set possible.

### Offline step-by-step development

Write a one-page requirement for “AI screw/bolt verification camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera + controlled lighting. Fix sensor mounting before collecting the main dataset.

Capture raw images from the real prototype under the full expected range of normal conditions. Keep originals immutable and store metadata (batch, lighting, product type, camera settings, date/site).

Define the smallest useful region of interest and input resolution. If the object position is fixed, crop it before inference instead of asking the network to search the whole frame.

Label/prepare data for Keypoint/object classifier. Split train/validation/final-test sets by batch/site when possible.

Train locally on the workstation. Start with a compact CNN or keypoint/OCR pipeline; only move to a detector/segmenter if the simpler formulation cannot meet the requirement.

Quantize to INT8 and compare final-test metrics before and after quantization. Save the calibration image list.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Count/presence/tightening mark state).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Automotive/electronics). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Final-test images come from a different batch/day/site where possible.

Measure false accept and false reject separately; do not report accuracy alone.

Test lighting high/low limits, dirty lens, small position shifts and camera restart.

Measure trigger-to-output latency under worst-case processing load.

Run at least a multi-hour or multi-thousand-cycle soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

## 17. AI cable/connector verification camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Electronics/wiring |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB camera |
| AI formulation | Presence/orientation/routing classifier |
| Product output | Connected / unconnected / wrong route |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Fix camera and workstation geometry. Use the smallest ROI/class set possible.

### Offline step-by-step development

Write a one-page requirement for “AI cable/connector verification camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera. Fix sensor mounting before collecting the main dataset.

Capture raw images from the real prototype under the full expected range of normal conditions. Keep originals immutable and store metadata (batch, lighting, product type, camera settings, date/site).

Define the smallest useful region of interest and input resolution. If the object position is fixed, crop it before inference instead of asking the network to search the whole frame.

Label/prepare data for Presence/orientation/routing classifier. Split train/validation/final-test sets by batch/site when possible.

Train locally on the workstation. Start with a compact CNN or keypoint/OCR pipeline; only move to a detector/segmenter if the simpler formulation cannot meet the requirement.

Quantize to INT8 and compare final-test metrics before and after quantization. Save the calibration image list.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Connected / unconnected / wrong route).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Electronics/wiring). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Final-test images come from a different batch/day/site where possible.

Measure false accept and false reject separately; do not report accuracy alone.

Test lighting high/low limits, dirty lens, small position shifts and camera restart.

Measure trigger-to-output latency under worst-case processing load.

Run at least a multi-hour or multi-thousand-cycle soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

## 18. AI tool verification camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Manual workstations |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB camera |
| AI formulation | Tool classification |
| Product output | Correct tool / wrong tool |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Fix camera and workstation geometry. Use the smallest ROI/class set possible.

### Offline step-by-step development

Write a one-page requirement for “AI tool verification camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera. Fix sensor mounting before collecting the main dataset.

Capture raw images from the real prototype under the full expected range of normal conditions. Keep originals immutable and store metadata (batch, lighting, product type, camera settings, date/site).

Define the smallest useful region of interest and input resolution. If the object position is fixed, crop it before inference instead of asking the network to search the whole frame.

Label/prepare data for Tool classification. Split train/validation/final-test sets by batch/site when possible.

Train locally on the workstation. Start with a compact CNN or keypoint/OCR pipeline; only move to a detector/segmenter if the simpler formulation cannot meet the requirement.

Quantize to INT8 and compare final-test metrics before and after quantization. Save the calibration image list.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Correct tool / wrong tool).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Manual workstations). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Final-test images come from a different batch/day/site where possible.

Measure false accept and false reject separately; do not report accuracy alone.

Test lighting high/low limits, dirty lens, small position shifts and camera restart.

Measure trigger-to-output latency under worst-case processing load.

Run at least a multi-hour or multi-thousand-cycle soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

## 19. AI process-step verification camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Manufacturing workstations |
| Cost-first MCU | STM32N6 — STM32N6 accelerated target |
| Sensors / inputs | RGB camera + optional tool sensor |
| AI formulation | Action/state classifier over short sequence |
| Product output | Step completed / skipped / wrong order |

Use STM32N657-class devices when the workload genuinely needs detection/segmentation/pose/depth/tracking or more complex variable scenes. Prototype on STM32N6570-DK. Re-evaluate H7 only if the problem can be constrained substantially.

Cost-control strategy: Fix camera and workstation geometry. Use the smallest ROI/class set possible.

### Offline step-by-step development

Write a one-page requirement for “AI process-step verification camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera + optional tool sensor. Fix sensor mounting before collecting the main dataset.

Capture raw images from the real prototype under the full expected range of normal conditions. Keep originals immutable and store metadata (batch, lighting, product type, camera settings, date/site).

Define the smallest useful region of interest and input resolution. If the object position is fixed, crop it before inference instead of asking the network to search the whole frame.

Label/prepare data for Action/state classifier over short sequence. Split train/validation/final-test sets by batch/site when possible.

Train locally on the workstation. Start with a compact CNN or keypoint/OCR pipeline; only move to a detector/segmenter if the simpler formulation cannot meet the requirement.

Quantize to INT8 and compare final-test metrics before and after quantization. Save the calibration image list.

Analyze/generate the model for STM32N6 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Step completed / skipped / wrong order).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Manufacturing workstations). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Final-test images come from a different batch/day/site where possible.

Measure false accept and false reject separately; do not report accuracy alone.

Test lighting high/low limits, dirty lens, small position shifts and camera restart.

Measure trigger-to-output latency under worst-case processing load.

Run at least a multi-hour or multi-thousand-cycle soak test.

### Cost-down / upgrade decision

N6 is selected because this product needs more complex variable-scene perception. Cost-down is possible only if the requirement can be constrained: fixed zones, fewer classes, lower resolution, reduced frame rate, external range sensor, or a simpler classifier. Re-test H7 only after that reformulation.

# Category — Industrial safety

## 20. AI worker PPE detector

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Factories, mines, construction |
| Cost-first MCU | STM32N6 — STM32N6 accelerated target |
| Sensors / inputs | RGB camera |
| AI formulation | Person + PPE detector |
| Product output | PPE compliance event |

Use STM32N657-class devices when the workload genuinely needs detection/segmentation/pose/depth/tracking or more complex variable scenes. Prototype on STM32N6570-DK. Re-evaluate H7 only if the problem can be constrained substantially.

Cost-control strategy: Collect difficult human poses and negatives. AI should be a warning layer unless full safety engineering/certification is performed.

### Offline step-by-step development

Write a one-page requirement for “AI worker PPE detector”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera. Fix sensor mounting before collecting the main dataset.

Collect representative video/images at the final mounting height and field of view. Include occlusion, backlight, low light, reflections and difficult negatives.

Label the minimum classes/targets needed for Person + PPE detector; do not add classes that the product does not use.

Train locally with a model sized for the selected MCU. For N6 targets, use compact detector/segmentation/pose models; for H7, simplify resolution/classes and use fixed zones where possible.

Quantize to INT8 and evaluate per-condition accuracy, not only global mAP/accuracy.

If range sensing is present, synchronize camera frames with ToF/radar/IMU timestamps and tune fusion logic offline from recorded sessions.

Analyze/generate the model for STM32N6 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (PPE compliance event).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Factories, mines, construction). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test each major environmental condition separately: day/night, backlight, occlusion, crowd density, dust/weather where applicable.

Measure missed-object/person rate and nuisance-alarm rate by scenario.

Measure end-to-end latency including camera exposure/capture, inference, fusion and output.

Force sensor disconnection/obstruction and verify a clear degraded/fault state.

Run a controlled pilot in advisory mode before enabling operational actions.

### Cost-down / upgrade decision

N6 is selected because this product needs more complex variable-scene perception. Cost-down is possible only if the requirement can be constrained: fixed zones, fewer classes, lower resolution, reduced frame rate, external range sensor, or a simpler classifier. Re-test H7 only after that reformulation.

## 21. AI restricted-zone safety camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Factories/warehouses |
| Cost-first MCU | STM32N6 — STM32N6 accelerated target |
| Sensors / inputs | RGB camera + optional ToF |
| AI formulation | Person detection/segmentation + zone logic |
| Product output | Intrusion warning |

Use STM32N657-class devices when the workload genuinely needs detection/segmentation/pose/depth/tracking or more complex variable scenes. Prototype on STM32N6570-DK. Re-evaluate H7 only if the problem can be constrained substantially.

Cost-control strategy: Collect difficult human poses and negatives. AI should be a warning layer unless full safety engineering/certification is performed.

### Offline step-by-step development

Write a one-page requirement for “AI restricted-zone safety camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera + optional ToF. Fix sensor mounting before collecting the main dataset.

Collect representative video/images at the final mounting height and field of view. Include occlusion, backlight, low light, reflections and difficult negatives.

Label the minimum classes/targets needed for Person detection/segmentation + zone logic; do not add classes that the product does not use.

Train locally with a model sized for the selected MCU. For N6 targets, use compact detector/segmentation/pose models; for H7, simplify resolution/classes and use fixed zones where possible.

Quantize to INT8 and evaluate per-condition accuracy, not only global mAP/accuracy.

If range sensing is present, synchronize camera frames with ToF/radar/IMU timestamps and tune fusion logic offline from recorded sessions.

Analyze/generate the model for STM32N6 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Intrusion warning).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Factories/warehouses). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test each major environmental condition separately: day/night, backlight, occlusion, crowd density, dust/weather where applicable.

Measure missed-object/person rate and nuisance-alarm rate by scenario.

Measure end-to-end latency including camera exposure/capture, inference, fusion and output.

Force sensor disconnection/obstruction and verify a clear degraded/fault state.

Run a controlled pilot in advisory mode before enabling operational actions.

### Cost-down / upgrade decision

N6 is selected because this product needs more complex variable-scene perception. Cost-down is possible only if the requirement can be constrained: fixed zones, fewer classes, lower resolution, reduced frame rate, external range sensor, or a simpler classifier. Re-test H7 only after that reformulation.

## 22. AI hand/arm danger-zone detector

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Presses/cutters/machinery |
| Cost-first MCU | STM32N6 — STM32N6 accelerated target |
| Sensors / inputs | RGB or ToF camera |
| AI formulation | Hand/pose detector + zone logic |
| Product output | Hand-in-danger-zone warning |

Use STM32N657-class devices when the workload genuinely needs detection/segmentation/pose/depth/tracking or more complex variable scenes. Prototype on STM32N6570-DK. Re-evaluate H7 only if the problem can be constrained substantially.

Cost-control strategy: Collect difficult human poses and negatives. AI should be a warning layer unless full safety engineering/certification is performed.

### Offline step-by-step development

Write a one-page requirement for “AI hand/arm danger-zone detector”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB or ToF camera. Fix sensor mounting before collecting the main dataset.

Collect representative video/images at the final mounting height and field of view. Include occlusion, backlight, low light, reflections and difficult negatives.

Label the minimum classes/targets needed for Hand/pose detector + zone logic; do not add classes that the product does not use.

Train locally with a model sized for the selected MCU. For N6 targets, use compact detector/segmentation/pose models; for H7, simplify resolution/classes and use fixed zones where possible.

Quantize to INT8 and evaluate per-condition accuracy, not only global mAP/accuracy.

If range sensing is present, synchronize camera frames with ToF/radar/IMU timestamps and tune fusion logic offline from recorded sessions.

Analyze/generate the model for STM32N6 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Hand-in-danger-zone warning).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Presses/cutters/machinery). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test each major environmental condition separately: day/night, backlight, occlusion, crowd density, dust/weather where applicable.

Measure missed-object/person rate and nuisance-alarm rate by scenario.

Measure end-to-end latency including camera exposure/capture, inference, fusion and output.

Force sensor disconnection/obstruction and verify a clear degraded/fault state.

Run a controlled pilot in advisory mode before enabling operational actions.

### Cost-down / upgrade decision

N6 is selected because this product needs more complex variable-scene perception. Cost-down is possible only if the requirement can be constrained: fixed zones, fewer classes, lower resolution, reduced frame rate, external range sensor, or a simpler classifier. Re-test H7 only after that reformulation.

## 23. AI worker fall detector

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Warehouses/factories |
| Cost-first MCU | STM32N6 — STM32N6 accelerated target |
| Sensors / inputs | RGB camera or low-res thermal + IMU optional |
| AI formulation | Pose/action classifier |
| Product output | Fall suspected event |

Use STM32N657-class devices when the workload genuinely needs detection/segmentation/pose/depth/tracking or more complex variable scenes. Prototype on STM32N6570-DK. Re-evaluate H7 only if the problem can be constrained substantially.

Cost-control strategy: Collect difficult human poses and negatives. AI should be a warning layer unless full safety engineering/certification is performed.

### Offline step-by-step development

Write a one-page requirement for “AI worker fall detector”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera or low-res thermal + IMU optional. Fix sensor mounting before collecting the main dataset.

Collect representative video/images at the final mounting height and field of view. Include occlusion, backlight, low light, reflections and difficult negatives.

Label the minimum classes/targets needed for Pose/action classifier; do not add classes that the product does not use.

Train locally with a model sized for the selected MCU. For N6 targets, use compact detector/segmentation/pose models; for H7, simplify resolution/classes and use fixed zones where possible.

Quantize to INT8 and evaluate per-condition accuracy, not only global mAP/accuracy.

If range sensing is present, synchronize camera frames with ToF/radar/IMU timestamps and tune fusion logic offline from recorded sessions.

Analyze/generate the model for STM32N6 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Fall suspected event).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Warehouses/factories). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test each major environmental condition separately: day/night, backlight, occlusion, crowd density, dust/weather where applicable.

Measure missed-object/person rate and nuisance-alarm rate by scenario.

Measure end-to-end latency including camera exposure/capture, inference, fusion and output.

Force sensor disconnection/obstruction and verify a clear degraded/fault state.

Run a controlled pilot in advisory mode before enabling operational actions.

### Cost-down / upgrade decision

N6 is selected because this product needs more complex variable-scene perception. Cost-down is possible only if the requirement can be constrained: fixed zones, fewer classes, lower resolution, reduced frame rate, external range sensor, or a simpler classifier. Re-test H7 only after that reformulation.

# Category — Warehouse safety

## 24. AI forklift pedestrian detector

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Warehouses/factories |
| Cost-first MCU | STM32N6 — STM32N6 accelerated target |
| Sensors / inputs | RGB camera + ToF |
| AI formulation | Person detector + distance fusion |
| Product output | Pedestrian proximity warning |

Use STM32N657-class devices when the workload genuinely needs detection/segmentation/pose/depth/tracking or more complex variable scenes. Prototype on STM32N6570-DK. Re-evaluate H7 only if the problem can be constrained substantially.

Cost-control strategy: Fuse semantic camera output with ToF/radar distance and vehicle motion.

### Offline step-by-step development

Write a one-page requirement for “AI forklift pedestrian detector”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera + ToF. Fix sensor mounting before collecting the main dataset.

Collect representative video/images at the final mounting height and field of view. Include occlusion, backlight, low light, reflections and difficult negatives.

Label the minimum classes/targets needed for Person detector + distance fusion; do not add classes that the product does not use.

Train locally with a model sized for the selected MCU. For N6 targets, use compact detector/segmentation/pose models; for H7, simplify resolution/classes and use fixed zones where possible.

Quantize to INT8 and evaluate per-condition accuracy, not only global mAP/accuracy.

If range sensing is present, synchronize camera frames with ToF/radar/IMU timestamps and tune fusion logic offline from recorded sessions.

Analyze/generate the model for STM32N6 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Pedestrian proximity warning).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Warehouses/factories). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test each major environmental condition separately: day/night, backlight, occlusion, crowd density, dust/weather where applicable.

Measure missed-object/person rate and nuisance-alarm rate by scenario.

Measure end-to-end latency including camera exposure/capture, inference, fusion and output.

Force sensor disconnection/obstruction and verify a clear degraded/fault state.

Run a controlled pilot in advisory mode before enabling operational actions.

### Cost-down / upgrade decision

N6 is selected because this product needs more complex variable-scene perception. Cost-down is possible only if the requirement can be constrained: fixed zones, fewer classes, lower resolution, reduced frame rate, external range sensor, or a simpler classifier. Re-test H7 only after that reformulation.

## 25. AI forklift collision-warning unit

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Logistics fleets |
| Cost-first MCU | STM32N6 — STM32N6 accelerated target |
| Sensors / inputs | RGB + ToF/radar + IMU |
| AI formulation | Object detection + closing-risk estimation |
| Product output | Risk level |

Use STM32N657-class devices when the workload genuinely needs detection/segmentation/pose/depth/tracking or more complex variable scenes. Prototype on STM32N6570-DK. Re-evaluate H7 only if the problem can be constrained substantially.

Cost-control strategy: Fuse semantic camera output with ToF/radar distance and vehicle motion.

### Offline step-by-step development

Write a one-page requirement for “AI forklift collision-warning unit”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB + ToF/radar + IMU. Fix sensor mounting before collecting the main dataset.

Collect representative video/images at the final mounting height and field of view. Include occlusion, backlight, low light, reflections and difficult negatives.

Label the minimum classes/targets needed for Object detection + closing-risk estimation; do not add classes that the product does not use.

Train locally with a model sized for the selected MCU. For N6 targets, use compact detector/segmentation/pose models; for H7, simplify resolution/classes and use fixed zones where possible.

Quantize to INT8 and evaluate per-condition accuracy, not only global mAP/accuracy.

If range sensing is present, synchronize camera frames with ToF/radar/IMU timestamps and tune fusion logic offline from recorded sessions.

Analyze/generate the model for STM32N6 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Risk level).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Logistics fleets). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test each major environmental condition separately: day/night, backlight, occlusion, crowd density, dust/weather where applicable.

Measure missed-object/person rate and nuisance-alarm rate by scenario.

Measure end-to-end latency including camera exposure/capture, inference, fusion and output.

Force sensor disconnection/obstruction and verify a clear degraded/fault state.

Run a controlled pilot in advisory mode before enabling operational actions.

### Cost-down / upgrade decision

N6 is selected because this product needs more complex variable-scene perception. Cost-down is possible only if the requirement can be constrained: fixed zones, fewer classes, lower resolution, reduced frame rate, external range sensor, or a simpler classifier. Re-test H7 only after that reformulation.

# Category — Robotics

## 26. AI AGV obstacle detector

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Automated warehouses |
| Cost-first MCU | STM32N6 — STM32N6 accelerated target |
| Sensors / inputs | RGB + ToF/depth + IMU |
| AI formulation | Obstacle detection/segmentation |
| Product output | Obstacle class + free path |

Use STM32N657-class devices when the workload genuinely needs detection/segmentation/pose/depth/tracking or more complex variable scenes. Prototype on STM32N6570-DK. Re-evaluate H7 only if the problem can be constrained substantially.

Cost-control strategy: Calibrate camera-to-robot coordinates and depth. Validate repeatability, not just detection accuracy.

### Offline step-by-step development

Write a one-page requirement for “AI AGV obstacle detector”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB + ToF/depth + IMU. Fix sensor mounting before collecting the main dataset.

Collect representative video/images at the final mounting height and field of view. Include occlusion, backlight, low light, reflections and difficult negatives.

Label the minimum classes/targets needed for Obstacle detection/segmentation; do not add classes that the product does not use.

Train locally with a model sized for the selected MCU. For N6 targets, use compact detector/segmentation/pose models; for H7, simplify resolution/classes and use fixed zones where possible.

Quantize to INT8 and evaluate per-condition accuracy, not only global mAP/accuracy.

If range sensing is present, synchronize camera frames with ToF/radar/IMU timestamps and tune fusion logic offline from recorded sessions.

Analyze/generate the model for STM32N6 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Obstacle class + free path).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Automated warehouses). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test each major environmental condition separately: day/night, backlight, occlusion, crowd density, dust/weather where applicable.

Measure missed-object/person rate and nuisance-alarm rate by scenario.

Measure end-to-end latency including camera exposure/capture, inference, fusion and output.

Force sensor disconnection/obstruction and verify a clear degraded/fault state.

Run a controlled pilot in advisory mode before enabling operational actions.

### Cost-down / upgrade decision

N6 is selected because this product needs more complex variable-scene perception. Cost-down is possible only if the requirement can be constrained: fixed zones, fewer classes, lower resolution, reduced frame rate, external range sensor, or a simpler classifier. Re-test H7 only after that reformulation.

# Category — Robotics safety

## 27. AI robot/cobot safety camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Robotic cells |
| Cost-first MCU | STM32N6 — STM32N6 accelerated target |
| Sensors / inputs | RGB + ToF |
| AI formulation | Person/pose/zone segmentation |
| Product output | Human proximity risk |

Use STM32N657-class devices when the workload genuinely needs detection/segmentation/pose/depth/tracking or more complex variable scenes. Prototype on STM32N6570-DK. Re-evaluate H7 only if the problem can be constrained substantially.

Cost-control strategy: Combine person perception with zones/depth; use independent certified protection for safety functions.

### Offline step-by-step development

Write a one-page requirement for “AI robot/cobot safety camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB + ToF. Fix sensor mounting before collecting the main dataset.

Collect representative video/images at the final mounting height and field of view. Include occlusion, backlight, low light, reflections and difficult negatives.

Label the minimum classes/targets needed for Person/pose/zone segmentation; do not add classes that the product does not use.

Train locally with a model sized for the selected MCU. For N6 targets, use compact detector/segmentation/pose models; for H7, simplify resolution/classes and use fixed zones where possible.

Quantize to INT8 and evaluate per-condition accuracy, not only global mAP/accuracy.

If range sensing is present, synchronize camera frames with ToF/radar/IMU timestamps and tune fusion logic offline from recorded sessions.

Analyze/generate the model for STM32N6 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Human proximity risk).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Robotic cells). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test each major environmental condition separately: day/night, backlight, occlusion, crowd density, dust/weather where applicable.

Measure missed-object/person rate and nuisance-alarm rate by scenario.

Measure end-to-end latency including camera exposure/capture, inference, fusion and output.

Force sensor disconnection/obstruction and verify a clear degraded/fault state.

Run a controlled pilot in advisory mode before enabling operational actions.

### Cost-down / upgrade decision

N6 is selected because this product needs more complex variable-scene perception. Cost-down is possible only if the requirement can be constrained: fixed zones, fewer classes, lower resolution, reduced frame rate, external range sensor, or a simpler classifier. Re-test H7 only after that reformulation.

# Category — Robotics

## 28. AI robot object-location camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Pick-and-place robots |
| Cost-first MCU | STM32N6 — STM32N6 accelerated target |
| Sensors / inputs | RGB + optional ToF |
| AI formulation | Object detector/keypoints |
| Product output | Object x/y/class/orientation |

Use STM32N657-class devices when the workload genuinely needs detection/segmentation/pose/depth/tracking or more complex variable scenes. Prototype on STM32N6570-DK. Re-evaluate H7 only if the problem can be constrained substantially.

Cost-control strategy: Calibrate camera-to-robot coordinates and depth. Validate repeatability, not just detection accuracy.

### Offline step-by-step development

Write a one-page requirement for “AI robot object-location camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB + optional ToF. Fix sensor mounting before collecting the main dataset.

Collect representative video/images at the final mounting height and field of view. Include occlusion, backlight, low light, reflections and difficult negatives.

Label the minimum classes/targets needed for Object detector/keypoints; do not add classes that the product does not use.

Train locally with a model sized for the selected MCU. For N6 targets, use compact detector/segmentation/pose models; for H7, simplify resolution/classes and use fixed zones where possible.

Quantize to INT8 and evaluate per-condition accuracy, not only global mAP/accuracy.

If range sensing is present, synchronize camera frames with ToF/radar/IMU timestamps and tune fusion logic offline from recorded sessions.

Analyze/generate the model for STM32N6 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Object x/y/class/orientation).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Pick-and-place robots). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test each major environmental condition separately: day/night, backlight, occlusion, crowd density, dust/weather where applicable.

Measure missed-object/person rate and nuisance-alarm rate by scenario.

Measure end-to-end latency including camera exposure/capture, inference, fusion and output.

Force sensor disconnection/obstruction and verify a clear degraded/fault state.

Run a controlled pilot in advisory mode before enabling operational actions.

### Cost-down / upgrade decision

N6 is selected because this product needs more complex variable-scene perception. Cost-down is possible only if the requirement can be constrained: fixed zones, fewer classes, lower resolution, reduced frame rate, external range sensor, or a simpler classifier. Re-test H7 only after that reformulation.

## 29. AI robotic bin-picking camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Logistics/manufacturing |
| Cost-first MCU | STM32N6 — STM32N6 accelerated target |
| Sensors / inputs | RGB + depth/ToF |
| AI formulation | Instance segmentation + grasp point |
| Product output | Object mask + grasp target |

Use STM32N657-class devices when the workload genuinely needs detection/segmentation/pose/depth/tracking or more complex variable scenes. Prototype on STM32N6570-DK. Re-evaluate H7 only if the problem can be constrained substantially.

Cost-control strategy: Calibrate camera-to-robot coordinates and depth. Validate repeatability, not just detection accuracy.

### Offline step-by-step development

Write a one-page requirement for “AI robotic bin-picking camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB + depth/ToF. Fix sensor mounting before collecting the main dataset.

Collect representative video/images at the final mounting height and field of view. Include occlusion, backlight, low light, reflections and difficult negatives.

Label the minimum classes/targets needed for Instance segmentation + grasp point; do not add classes that the product does not use.

Train locally with a model sized for the selected MCU. For N6 targets, use compact detector/segmentation/pose models; for H7, simplify resolution/classes and use fixed zones where possible.

Quantize to INT8 and evaluate per-condition accuracy, not only global mAP/accuracy.

If range sensing is present, synchronize camera frames with ToF/radar/IMU timestamps and tune fusion logic offline from recorded sessions.

Analyze/generate the model for STM32N6 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Object mask + grasp target).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Logistics/manufacturing). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test each major environmental condition separately: day/night, backlight, occlusion, crowd density, dust/weather where applicable.

Measure missed-object/person rate and nuisance-alarm rate by scenario.

Measure end-to-end latency including camera exposure/capture, inference, fusion and output.

Force sensor disconnection/obstruction and verify a clear degraded/fault state.

Run a controlled pilot in advisory mode before enabling operational actions.

### Cost-down / upgrade decision

N6 is selected because this product needs more complex variable-scene perception. Cost-down is possible only if the requirement can be constrained: fixed zones, fewer classes, lower resolution, reduced frame rate, external range sensor, or a simpler classifier. Re-test H7 only after that reformulation.

# Category — Warehouse

## 30. AI pallet detector

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Warehouses/DCs |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB + ToF |
| AI formulation | Pallet detector |
| Product output | Pallet present/position |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Collect data at real shelf/dock heights and under warehouse lighting/occlusion.

### Offline step-by-step development

Write a one-page requirement for “AI pallet detector”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB + ToF. Fix sensor mounting before collecting the main dataset.

Collect representative video/images at the final mounting height and field of view. Include occlusion, backlight, low light, reflections and difficult negatives.

Label the minimum classes/targets needed for Pallet detector; do not add classes that the product does not use.

Train locally with a model sized for the selected MCU. For N6 targets, use compact detector/segmentation/pose models; for H7, simplify resolution/classes and use fixed zones where possible.

Quantize to INT8 and evaluate per-condition accuracy, not only global mAP/accuracy.

If range sensing is present, synchronize camera frames with ToF/radar/IMU timestamps and tune fusion logic offline from recorded sessions.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Pallet present/position).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Warehouses/DCs). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test each major environmental condition separately: day/night, backlight, occlusion, crowd density, dust/weather where applicable.

Measure missed-object/person rate and nuisance-alarm rate by scenario.

Measure end-to-end latency including camera exposure/capture, inference, fusion and output.

Force sensor disconnection/obstruction and verify a clear degraded/fault state.

Run a controlled pilot in advisory mode before enabling operational actions.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

## 31. AI pallet-condition inspection camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Warehouses/logistics |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB camera |
| AI formulation | Damage/anomaly classifier |
| Product output | Pallet OK/damaged |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Collect data at real shelf/dock heights and under warehouse lighting/occlusion.

### Offline step-by-step development

Write a one-page requirement for “AI pallet-condition inspection camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera. Fix sensor mounting before collecting the main dataset.

Collect representative video/images at the final mounting height and field of view. Include occlusion, backlight, low light, reflections and difficult negatives.

Label the minimum classes/targets needed for Damage/anomaly classifier; do not add classes that the product does not use.

Train locally with a model sized for the selected MCU. For N6 targets, use compact detector/segmentation/pose models; for H7, simplify resolution/classes and use fixed zones where possible.

Quantize to INT8 and evaluate per-condition accuracy, not only global mAP/accuracy.

If range sensing is present, synchronize camera frames with ToF/radar/IMU timestamps and tune fusion logic offline from recorded sessions.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Pallet OK/damaged).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Warehouses/logistics). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test each major environmental condition separately: day/night, backlight, occlusion, crowd density, dust/weather where applicable.

Measure missed-object/person rate and nuisance-alarm rate by scenario.

Measure end-to-end latency including camera exposure/capture, inference, fusion and output.

Force sensor disconnection/obstruction and verify a clear degraded/fault state.

Run a controlled pilot in advisory mode before enabling operational actions.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

# Category — Logistics

## 32. AI package damage detector

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Courier/fulfillment centers |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB camera + controlled station |
| AI formulation | Damage/anomaly classifier |
| Product output | Package damage class |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Constrain the product problem and choose the smallest model that solves it.

### Offline step-by-step development

Write a one-page requirement for “AI package damage detector”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera + controlled station. Fix sensor mounting before collecting the main dataset.

Collect representative data locally.

Prepare labels and train the smallest suitable model.

Quantize and validate on the final test set.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Package damage class).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Courier/fulfillment centers). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test on machines/sessions not represented in training.

Sweep operating speed/load/temperature and verify the model does not confuse operating condition with failure.

Compare MCU preprocessing/inference with the offline Python reference on identical windows.

Test sensor disconnect, saturation and mounting changes.

Run a long-duration data-logging and inference soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

## 33. AI parcel dimension/classification unit

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Sorting centers |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB + multi-zone ToF |
| AI formulation | Parcel classifier + geometric measurement |
| Product output | Class + dimensions |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Constrain the product problem and choose the smallest model that solves it.

### Offline step-by-step development

Write a one-page requirement for “AI parcel dimension/classification unit”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB + multi-zone ToF. Fix sensor mounting before collecting the main dataset.

Collect representative data locally.

Prepare labels and train the smallest suitable model.

Quantize and validate on the final test set.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Class + dimensions).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Sorting centers). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test on machines/sessions not represented in training.

Sweep operating speed/load/temperature and verify the model does not confuse operating condition with failure.

Compare MCU preprocessing/inference with the offline Python reference on identical windows.

Test sensor disconnect, saturation and mounting changes.

Run a long-duration data-logging and inference soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

## 34. AI loading-dock monitoring camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Warehouses |
| Cost-first MCU | STM32N6 — STM32N6 accelerated target |
| Sensors / inputs | RGB + ToF |
| AI formulation | Truck/person/pallet detector |
| Product output | Dock state/events |

Use STM32N657-class devices when the workload genuinely needs detection/segmentation/pose/depth/tracking or more complex variable scenes. Prototype on STM32N6570-DK. Re-evaluate H7 only if the problem can be constrained substantially.

Cost-control strategy: Constrain the product problem and choose the smallest model that solves it.

### Offline step-by-step development

Write a one-page requirement for “AI loading-dock monitoring camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB + ToF. Fix sensor mounting before collecting the main dataset.

Collect representative data locally.

Prepare labels and train the smallest suitable model.

Quantize and validate on the final test set.

Analyze/generate the model for STM32N6 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Dock state/events).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Warehouses). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test on machines/sessions not represented in training.

Sweep operating speed/load/temperature and verify the model does not confuse operating condition with failure.

Compare MCU preprocessing/inference with the offline Python reference on identical windows.

Test sensor disconnect, saturation and mounting changes.

Run a long-duration data-logging and inference soak test.

### Cost-down / upgrade decision

N6 is selected because this product needs more complex variable-scene perception. Cost-down is possible only if the requirement can be constrained: fixed zones, fewer classes, lower resolution, reduced frame rate, external range sensor, or a simpler classifier. Re-test H7 only after that reformulation.

## 35. AI truck-loading verification camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Distribution centers |
| Cost-first MCU | STM32N6 — STM32N6 accelerated target |
| Sensors / inputs | RGB camera |
| AI formulation | Pallet/package detection and count |
| Product output | Loading completeness |

Use STM32N657-class devices when the workload genuinely needs detection/segmentation/pose/depth/tracking or more complex variable scenes. Prototype on STM32N6570-DK. Re-evaluate H7 only if the problem can be constrained substantially.

Cost-control strategy: Constrain the product problem and choose the smallest model that solves it.

### Offline step-by-step development

Write a one-page requirement for “AI truck-loading verification camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera. Fix sensor mounting before collecting the main dataset.

Collect representative data locally.

Prepare labels and train the smallest suitable model.

Quantize and validate on the final test set.

Analyze/generate the model for STM32N6 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Loading completeness).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Distribution centers). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test on machines/sessions not represented in training.

Sweep operating speed/load/temperature and verify the model does not confuse operating condition with failure.

Compare MCU preprocessing/inference with the offline Python reference on identical windows.

Test sensor disconnect, saturation and mounting changes.

Run a long-duration data-logging and inference soak test.

### Cost-down / upgrade decision

N6 is selected because this product needs more complex variable-scene perception. Cost-down is possible only if the requirement can be constrained: fixed zones, fewer classes, lower resolution, reduced frame rate, external range sensor, or a simpler classifier. Re-test H7 only after that reformulation.

## 36. AI container loading verifier

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Ports/shipping |
| Cost-first MCU | STM32N6 — STM32N6 accelerated target |
| Sensors / inputs | RGB camera + optional depth |
| AI formulation | Object/pallet detection + occupancy estimate |
| Product output | Container load state |

Use STM32N657-class devices when the workload genuinely needs detection/segmentation/pose/depth/tracking or more complex variable scenes. Prototype on STM32N6570-DK. Re-evaluate H7 only if the problem can be constrained substantially.

Cost-control strategy: Constrain the product problem and choose the smallest model that solves it.

### Offline step-by-step development

Write a one-page requirement for “AI container loading verifier”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera + optional depth. Fix sensor mounting before collecting the main dataset.

Collect representative data locally.

Prepare labels and train the smallest suitable model.

Quantize and validate on the final test set.

Analyze/generate the model for STM32N6 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Container load state).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Ports/shipping). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test on machines/sessions not represented in training.

Sweep operating speed/load/temperature and verify the model does not confuse operating condition with failure.

Compare MCU preprocessing/inference with the offline Python reference on identical windows.

Test sensor disconnect, saturation and mounting changes.

Run a long-duration data-logging and inference soak test.

### Cost-down / upgrade decision

N6 is selected because this product needs more complex variable-scene perception. Cost-down is possible only if the requirement can be constrained: fixed zones, fewer classes, lower resolution, reduced frame rate, external range sensor, or a simpler classifier. Re-test H7 only after that reformulation.

# Category — Warehouse

## 37. AI warehouse shelf inventory camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Warehouses |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB camera |
| AI formulation | Product/bin presence classifier/detector |
| Product output | Stock state |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Collect data at real shelf/dock heights and under warehouse lighting/occlusion.

### Offline step-by-step development

Write a one-page requirement for “AI warehouse shelf inventory camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera. Fix sensor mounting before collecting the main dataset.

Collect representative video/images at the final mounting height and field of view. Include occlusion, backlight, low light, reflections and difficult negatives.

Label the minimum classes/targets needed for Product/bin presence classifier/detector; do not add classes that the product does not use.

Train locally with a model sized for the selected MCU. For N6 targets, use compact detector/segmentation/pose models; for H7, simplify resolution/classes and use fixed zones where possible.

Quantize to INT8 and evaluate per-condition accuracy, not only global mAP/accuracy.

If range sensing is present, synchronize camera frames with ToF/radar/IMU timestamps and tune fusion logic offline from recorded sessions.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Stock state).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Warehouses). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test each major environmental condition separately: day/night, backlight, occlusion, crowd density, dust/weather where applicable.

Measure missed-object/person rate and nuisance-alarm rate by scenario.

Measure end-to-end latency including camera exposure/capture, inference, fusion and output.

Force sensor disconnection/obstruction and verify a clear degraded/fault state.

Run a controlled pilot in advisory mode before enabling operational actions.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

# Category — Warehouse/production

## 38. AI empty-bin detector

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Manufacturing/warehouses |
| Cost-first MCU | STM32U5 — STM32U5 cost-first target |
| Sensors / inputs | RGB or ToF |
| AI formulation | Binary fill-state classifier |
| Product output | Empty/low/normal |

Use U575/U585/U595-class parts as starting points. Best for low-power sensor AI, audio/vibration, simple fusion and small models. Prototype with B-U585I-IOT02A when its onboard sensors are useful.

Cost-control strategy: Use ToF or simple camera classification when the state space is small.

### Offline step-by-step development

Write a one-page requirement for “AI empty-bin detector”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB or ToF. Fix sensor mounting before collecting the main dataset.

Record synchronized raw sensor streams across the full healthy operating envelope; log sample rate, sensor orientation, machine/load/state and temperature.

Segment data into fixed windows and evaluate both engineered features (RMS, kurtosis, FFT bands, temperature trends) and a small 1D CNN/MLP/anomaly model.

Prepare labels or normal/anomaly targets for Binary fill-state classifier. Keep entire machines/sites out of the training set for a stronger final test where possible.

Train locally, quantize, and verify that the model is learning the intended physical condition rather than a shortcut such as RPM or background noise.

Reproduce exactly the same preprocessing in C on the MCU and compare window-by-window outputs against the offline Python reference.

Analyze/generate the model for STM32U5 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Empty/low/normal).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Manufacturing/warehouses). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test on machines/sessions not represented in training.

Sweep operating speed/load/temperature and verify the model does not confuse operating condition with failure.

Compare MCU preprocessing/inference with the offline Python reference on identical windows.

Test sensor disconnect, saturation and mounting changes.

Run a long-duration data-logging and inference soak test.

### Cost-down / upgrade decision

If measured CPU/RAM usage has large margin, evaluate a smaller U5 SKU or lower STM32 family during a formal cost-down phase. Move upward to H7 only when sample rate, channels, DSP or model size cannot meet timing with margin.

# Category — Predictive maintenance

## 39. AI machine-health sensor

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Factories |
| Cost-first MCU | STM32U5 — STM32U5 cost-first target |
| Sensors / inputs | Accelerometer + microphone + temperature |
| AI formulation | 1D CNN / anomaly model |
| Product output | Health score / anomaly |

Use U575/U585/U595-class parts as starting points. Best for low-power sensor AI, audio/vibration, simple fusion and small models. Prototype with B-U585I-IOT02A when its onboard sensors are useful.

Cost-control strategy: Mount sensors repeatably; collect healthy operating envelopes before fault data.

### Offline step-by-step development

Write a one-page requirement for “AI machine-health sensor”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using Accelerometer + microphone + temperature. Fix sensor mounting before collecting the main dataset.

Record synchronized raw sensor streams across the full healthy operating envelope; log sample rate, sensor orientation, machine/load/state and temperature.

Segment data into fixed windows and evaluate both engineered features (RMS, kurtosis, FFT bands, temperature trends) and a small 1D CNN/MLP/anomaly model.

Prepare labels or normal/anomaly targets for 1D CNN / anomaly model. Keep entire machines/sites out of the training set for a stronger final test where possible.

Train locally, quantize, and verify that the model is learning the intended physical condition rather than a shortcut such as RPM or background noise.

Reproduce exactly the same preprocessing in C on the MCU and compare window-by-window outputs against the offline Python reference.

Analyze/generate the model for STM32U5 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Health score / anomaly).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Factories). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test on machines/sessions not represented in training.

Sweep operating speed/load/temperature and verify the model does not confuse operating condition with failure.

Compare MCU preprocessing/inference with the offline Python reference on identical windows.

Test sensor disconnect, saturation and mounting changes.

Run a long-duration data-logging and inference soak test.

### Cost-down / upgrade decision

If measured CPU/RAM usage has large margin, evaluate a smaller U5 SKU or lower STM32 family during a formal cost-down phase. Move upward to H7 only when sample rate, channels, DSP or model size cannot meet timing with margin.

## 40. AI bearing-failure sensor

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Factories/heavy industry |
| Cost-first MCU | STM32U5 — STM32U5 cost-first target |
| Sensors / inputs | High-bandwidth accelerometer + temp |
| AI formulation | Spectral/1D CNN classifier |
| Product output | Bearing fault probability |

Use U575/U585/U595-class parts as starting points. Best for low-power sensor AI, audio/vibration, simple fusion and small models. Prototype with B-U585I-IOT02A when its onboard sensors are useful.

Cost-control strategy: Mount sensors repeatably; collect healthy operating envelopes before fault data.

### Offline step-by-step development

Write a one-page requirement for “AI bearing-failure sensor”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using High-bandwidth accelerometer + temp. Fix sensor mounting before collecting the main dataset.

Record synchronized raw sensor streams across the full healthy operating envelope; log sample rate, sensor orientation, machine/load/state and temperature.

Segment data into fixed windows and evaluate both engineered features (RMS, kurtosis, FFT bands, temperature trends) and a small 1D CNN/MLP/anomaly model.

Prepare labels or normal/anomaly targets for Spectral/1D CNN classifier. Keep entire machines/sites out of the training set for a stronger final test where possible.

Train locally, quantize, and verify that the model is learning the intended physical condition rather than a shortcut such as RPM or background noise.

Reproduce exactly the same preprocessing in C on the MCU and compare window-by-window outputs against the offline Python reference.

Analyze/generate the model for STM32U5 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Bearing fault probability).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Factories/heavy industry). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test on machines/sessions not represented in training.

Sweep operating speed/load/temperature and verify the model does not confuse operating condition with failure.

Compare MCU preprocessing/inference with the offline Python reference on identical windows.

Test sensor disconnect, saturation and mounting changes.

Run a long-duration data-logging and inference soak test.

### Cost-down / upgrade decision

If measured CPU/RAM usage has large margin, evaluate a smaller U5 SKU or lower STM32 family during a formal cost-down phase. Move upward to H7 only when sample rate, channels, DSP or model size cannot meet timing with margin.

## 41. AI motor anomaly detector

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Manufacturing |
| Cost-first MCU | STM32U5 — STM32U5 cost-first target |
| Sensors / inputs | Vibration + current + audio |
| AI formulation | Anomaly model / classifier |
| Product output | Motor health state |

Use U575/U585/U595-class parts as starting points. Best for low-power sensor AI, audio/vibration, simple fusion and small models. Prototype with B-U585I-IOT02A when its onboard sensors are useful.

Cost-control strategy: Mount sensors repeatably; collect healthy operating envelopes before fault data.

### Offline step-by-step development

Write a one-page requirement for “AI motor anomaly detector”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using Vibration + current + audio. Fix sensor mounting before collecting the main dataset.

Record synchronized raw sensor streams across the full healthy operating envelope; log sample rate, sensor orientation, machine/load/state and temperature.

Segment data into fixed windows and evaluate both engineered features (RMS, kurtosis, FFT bands, temperature trends) and a small 1D CNN/MLP/anomaly model.

Prepare labels or normal/anomaly targets for Anomaly model / classifier. Keep entire machines/sites out of the training set for a stronger final test where possible.

Train locally, quantize, and verify that the model is learning the intended physical condition rather than a shortcut such as RPM or background noise.

Reproduce exactly the same preprocessing in C on the MCU and compare window-by-window outputs against the offline Python reference.

Analyze/generate the model for STM32U5 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Motor health state).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Manufacturing). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test on machines/sessions not represented in training.

Sweep operating speed/load/temperature and verify the model does not confuse operating condition with failure.

Compare MCU preprocessing/inference with the offline Python reference on identical windows.

Test sensor disconnect, saturation and mounting changes.

Run a long-duration data-logging and inference soak test.

### Cost-down / upgrade decision

If measured CPU/RAM usage has large margin, evaluate a smaller U5 SKU or lower STM32 family during a formal cost-down phase. Move upward to H7 only when sample rate, channels, DSP or model size cannot meet timing with margin.

## 42. AI pump anomaly detector

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Water/chemical/manufacturing |
| Cost-first MCU | STM32U5 — STM32U5 cost-first target |
| Sensors / inputs | Vibration + audio + pressure optional |
| AI formulation | Anomaly/classifier |
| Product output | Cavitation/imbalance/anomaly |

Use U575/U585/U595-class parts as starting points. Best for low-power sensor AI, audio/vibration, simple fusion and small models. Prototype with B-U585I-IOT02A when its onboard sensors are useful.

Cost-control strategy: Mount sensors repeatably; collect healthy operating envelopes before fault data.

### Offline step-by-step development

Write a one-page requirement for “AI pump anomaly detector”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using Vibration + audio + pressure optional. Fix sensor mounting before collecting the main dataset.

Record synchronized raw sensor streams across the full healthy operating envelope; log sample rate, sensor orientation, machine/load/state and temperature.

Segment data into fixed windows and evaluate both engineered features (RMS, kurtosis, FFT bands, temperature trends) and a small 1D CNN/MLP/anomaly model.

Prepare labels or normal/anomaly targets for Anomaly/classifier. Keep entire machines/sites out of the training set for a stronger final test where possible.

Train locally, quantize, and verify that the model is learning the intended physical condition rather than a shortcut such as RPM or background noise.

Reproduce exactly the same preprocessing in C on the MCU and compare window-by-window outputs against the offline Python reference.

Analyze/generate the model for STM32U5 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Cavitation/imbalance/anomaly).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Water/chemical/manufacturing). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test on machines/sessions not represented in training.

Sweep operating speed/load/temperature and verify the model does not confuse operating condition with failure.

Compare MCU preprocessing/inference with the offline Python reference on identical windows.

Test sensor disconnect, saturation and mounting changes.

Run a long-duration data-logging and inference soak test.

### Cost-down / upgrade decision

If measured CPU/RAM usage has large margin, evaluate a smaller U5 SKU or lower STM32 family during a formal cost-down phase. Move upward to H7 only when sample rate, channels, DSP or model size cannot meet timing with margin.

# Category — Building/industrial

## 43. AI fan/HVAC health detector

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Commercial buildings/factories |
| Cost-first MCU | STM32U5 — STM32U5 cost-first target |
| Sensors / inputs | Vibration + audio + current |
| AI formulation | Anomaly classifier |
| Product output | Fan health / imbalance |

Use U575/U585/U595-class parts as starting points. Best for low-power sensor AI, audio/vibration, simple fusion and small models. Prototype with B-U585I-IOT02A when its onboard sensors are useful.

Cost-control strategy: Use vibration/audio/current features and low-power duty cycling.

### Offline step-by-step development

Write a one-page requirement for “AI fan/HVAC health detector”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using Vibration + audio + current. Fix sensor mounting before collecting the main dataset.

Record synchronized raw sensor streams across the full healthy operating envelope; log sample rate, sensor orientation, machine/load/state and temperature.

Segment data into fixed windows and evaluate both engineered features (RMS, kurtosis, FFT bands, temperature trends) and a small 1D CNN/MLP/anomaly model.

Prepare labels or normal/anomaly targets for Anomaly classifier. Keep entire machines/sites out of the training set for a stronger final test where possible.

Train locally, quantize, and verify that the model is learning the intended physical condition rather than a shortcut such as RPM or background noise.

Reproduce exactly the same preprocessing in C on the MCU and compare window-by-window outputs against the offline Python reference.

Analyze/generate the model for STM32U5 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Fan health / imbalance).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Commercial buildings/factories). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test on machines/sessions not represented in training.

Sweep operating speed/load/temperature and verify the model does not confuse operating condition with failure.

Compare MCU preprocessing/inference with the offline Python reference on identical windows.

Test sensor disconnect, saturation and mounting changes.

Run a long-duration data-logging and inference soak test.

### Cost-down / upgrade decision

If measured CPU/RAM usage has large margin, evaluate a smaller U5 SKU or lower STM32 family during a formal cost-down phase. Move upward to H7 only when sample rate, channels, DSP or model size cannot meet timing with margin.

# Category — Predictive maintenance

## 44. AI compressor monitoring sensor

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Factories/refrigeration |
| Cost-first MCU | STM32U5 — STM32U5 cost-first target |
| Sensors / inputs | Vibration + audio + temperature/pressure |
| AI formulation | Anomaly/fault classifier |
| Product output | Compressor health |

Use U575/U585/U595-class parts as starting points. Best for low-power sensor AI, audio/vibration, simple fusion and small models. Prototype with B-U585I-IOT02A when its onboard sensors are useful.

Cost-control strategy: Mount sensors repeatably; collect healthy operating envelopes before fault data.

### Offline step-by-step development

Write a one-page requirement for “AI compressor monitoring sensor”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using Vibration + audio + temperature/pressure. Fix sensor mounting before collecting the main dataset.

Record synchronized raw sensor streams across the full healthy operating envelope; log sample rate, sensor orientation, machine/load/state and temperature.

Segment data into fixed windows and evaluate both engineered features (RMS, kurtosis, FFT bands, temperature trends) and a small 1D CNN/MLP/anomaly model.

Prepare labels or normal/anomaly targets for Anomaly/fault classifier. Keep entire machines/sites out of the training set for a stronger final test where possible.

Train locally, quantize, and verify that the model is learning the intended physical condition rather than a shortcut such as RPM or background noise.

Reproduce exactly the same preprocessing in C on the MCU and compare window-by-window outputs against the offline Python reference.

Analyze/generate the model for STM32U5 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Compressor health).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Factories/refrigeration). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test on machines/sessions not represented in training.

Sweep operating speed/load/temperature and verify the model does not confuse operating condition with failure.

Compare MCU preprocessing/inference with the offline Python reference on identical windows.

Test sensor disconnect, saturation and mounting changes.

Run a long-duration data-logging and inference soak test.

### Cost-down / upgrade decision

If measured CPU/RAM usage has large margin, evaluate a smaller U5 SKU or lower STM32 family during a formal cost-down phase. Move upward to H7 only when sample rate, channels, DSP or model size cannot meet timing with margin.

## 45. AI gearbox health sensor

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Production machinery |
| Cost-first MCU | STM32U5 — STM32U5 cost-first target |
| Sensors / inputs | Vibration + speed/RPM |
| AI formulation | Spectral CNN / anomaly |
| Product output | Gear fault score |

Use U575/U585/U595-class parts as starting points. Best for low-power sensor AI, audio/vibration, simple fusion and small models. Prototype with B-U585I-IOT02A when its onboard sensors are useful.

Cost-control strategy: Mount sensors repeatably; collect healthy operating envelopes before fault data.

### Offline step-by-step development

Write a one-page requirement for “AI gearbox health sensor”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using Vibration + speed/RPM. Fix sensor mounting before collecting the main dataset.

Record synchronized raw sensor streams across the full healthy operating envelope; log sample rate, sensor orientation, machine/load/state and temperature.

Segment data into fixed windows and evaluate both engineered features (RMS, kurtosis, FFT bands, temperature trends) and a small 1D CNN/MLP/anomaly model.

Prepare labels or normal/anomaly targets for Spectral CNN / anomaly. Keep entire machines/sites out of the training set for a stronger final test where possible.

Train locally, quantize, and verify that the model is learning the intended physical condition rather than a shortcut such as RPM or background noise.

Reproduce exactly the same preprocessing in C on the MCU and compare window-by-window outputs against the offline Python reference.

Analyze/generate the model for STM32U5 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Gear fault score).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Production machinery). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test on machines/sessions not represented in training.

Sweep operating speed/load/temperature and verify the model does not confuse operating condition with failure.

Compare MCU preprocessing/inference with the offline Python reference on identical windows.

Test sensor disconnect, saturation and mounting changes.

Run a long-duration data-logging and inference soak test.

### Cost-down / upgrade decision

If measured CPU/RAM usage has large margin, evaluate a smaller U5 SKU or lower STM32 family during a formal cost-down phase. Move upward to H7 only when sample rate, channels, DSP or model size cannot meet timing with margin.

## 46. AI machine sound analyzer

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | CNC/motors/presses |
| Cost-first MCU | STM32U5 — STM32U5 cost-first target |
| Sensors / inputs | Microphone + optional vibration |
| AI formulation | Audio event/anomaly CNN |
| Product output | Sound fault class |

Use U575/U585/U595-class parts as starting points. Best for low-power sensor AI, audio/vibration, simple fusion and small models. Prototype with B-U585I-IOT02A when its onboard sensors are useful.

Cost-control strategy: Mount sensors repeatably; collect healthy operating envelopes before fault data.

### Offline step-by-step development

Write a one-page requirement for “AI machine sound analyzer”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using Microphone + optional vibration. Fix sensor mounting before collecting the main dataset.

Record synchronized raw sensor streams across the full healthy operating envelope; log sample rate, sensor orientation, machine/load/state and temperature.

Segment data into fixed windows and evaluate both engineered features (RMS, kurtosis, FFT bands, temperature trends) and a small 1D CNN/MLP/anomaly model.

Prepare labels or normal/anomaly targets for Audio event/anomaly CNN. Keep entire machines/sites out of the training set for a stronger final test where possible.

Train locally, quantize, and verify that the model is learning the intended physical condition rather than a shortcut such as RPM or background noise.

Reproduce exactly the same preprocessing in C on the MCU and compare window-by-window outputs against the offline Python reference.

Analyze/generate the model for STM32U5 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Sound fault class).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (CNC/motors/presses). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test on machines/sessions not represented in training.

Sweep operating speed/load/temperature and verify the model does not confuse operating condition with failure.

Compare MCU preprocessing/inference with the offline Python reference on identical windows.

Test sensor disconnect, saturation and mounting changes.

Run a long-duration data-logging and inference soak test.

### Cost-down / upgrade decision

If measured CPU/RAM usage has large margin, evaluate a smaller U5 SKU or lower STM32 family during a formal cost-down phase. Move upward to H7 only when sample rate, channels, DSP or model size cannot meet timing with margin.

## 47. AI vibration-monitoring node

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Industrial machinery |
| Cost-first MCU | STM32U5 — STM32U5 cost-first target |
| Sensors / inputs | Accelerometer |
| AI formulation | 1D CNN / feature MLP |
| Product output | Anomaly score |

Use U575/U585/U595-class parts as starting points. Best for low-power sensor AI, audio/vibration, simple fusion and small models. Prototype with B-U585I-IOT02A when its onboard sensors are useful.

Cost-control strategy: Mount sensors repeatably; collect healthy operating envelopes before fault data.

### Offline step-by-step development

Write a one-page requirement for “AI vibration-monitoring node”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using Accelerometer. Fix sensor mounting before collecting the main dataset.

Record synchronized raw sensor streams across the full healthy operating envelope; log sample rate, sensor orientation, machine/load/state and temperature.

Segment data into fixed windows and evaluate both engineered features (RMS, kurtosis, FFT bands, temperature trends) and a small 1D CNN/MLP/anomaly model.

Prepare labels or normal/anomaly targets for 1D CNN / feature MLP. Keep entire machines/sites out of the training set for a stronger final test where possible.

Train locally, quantize, and verify that the model is learning the intended physical condition rather than a shortcut such as RPM or background noise.

Reproduce exactly the same preprocessing in C on the MCU and compare window-by-window outputs against the offline Python reference.

Analyze/generate the model for STM32U5 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Anomaly score).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Industrial machinery). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test on machines/sessions not represented in training.

Sweep operating speed/load/temperature and verify the model does not confuse operating condition with failure.

Compare MCU preprocessing/inference with the offline Python reference on identical windows.

Test sensor disconnect, saturation and mounting changes.

Run a long-duration data-logging and inference soak test.

### Cost-down / upgrade decision

If measured CPU/RAM usage has large margin, evaluate a smaller U5 SKU or lower STM32 family during a formal cost-down phase. Move upward to H7 only when sample rate, channels, DSP or model size cannot meet timing with margin.

# Category — Industrial monitoring

## 48. AI leak detection sensor

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Water/gas/industrial facilities |
| Cost-first MCU | STM32U5 — STM32U5 cost-first target |
| Sensors / inputs | Acoustic sensor/mic + pressure optional |
| AI formulation | Acoustic event/anomaly model |
| Product output | Leak probability |

Use U575/U585/U595-class parts as starting points. Best for low-power sensor AI, audio/vibration, simple fusion and small models. Prototype with B-U585I-IOT02A when its onboard sensors are useful.

Cost-control strategy: Prefer sensor fusion and anomaly models where labeled faults are rare.

### Offline step-by-step development

Write a one-page requirement for “AI leak detection sensor”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using Acoustic sensor/mic + pressure optional. Fix sensor mounting before collecting the main dataset.

Record synchronized raw sensor streams across the full healthy operating envelope; log sample rate, sensor orientation, machine/load/state and temperature.

Segment data into fixed windows and evaluate both engineered features (RMS, kurtosis, FFT bands, temperature trends) and a small 1D CNN/MLP/anomaly model.

Prepare labels or normal/anomaly targets for Acoustic event/anomaly model. Keep entire machines/sites out of the training set for a stronger final test where possible.

Train locally, quantize, and verify that the model is learning the intended physical condition rather than a shortcut such as RPM or background noise.

Reproduce exactly the same preprocessing in C on the MCU and compare window-by-window outputs against the offline Python reference.

Analyze/generate the model for STM32U5 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Leak probability).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Water/gas/industrial facilities). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test on machines/sessions not represented in training.

Sweep operating speed/load/temperature and verify the model does not confuse operating condition with failure.

Compare MCU preprocessing/inference with the offline Python reference on identical windows.

Test sensor disconnect, saturation and mounting changes.

Run a long-duration data-logging and inference soak test.

### Cost-down / upgrade decision

If measured CPU/RAM usage has large margin, evaluate a smaller U5 SKU or lower STM32 family during a formal cost-down phase. Move upward to H7 only when sample rate, channels, DSP or model size cannot meet timing with margin.

# Category — Industrial safety

## 49. AI smoke/flame visual detector

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Factories/warehouses |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB/IR camera + temp/smoke sensor |
| AI formulation | Visual flame/smoke classifier + sensor fusion |
| Product output | Early warning event |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Collect difficult human poses and negatives. AI should be a warning layer unless full safety engineering/certification is performed.

### Offline step-by-step development

Write a one-page requirement for “AI smoke/flame visual detector”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB/IR camera + temp/smoke sensor. Fix sensor mounting before collecting the main dataset.

Collect representative video/images at the final mounting height and field of view. Include occlusion, backlight, low light, reflections and difficult negatives.

Label the minimum classes/targets needed for Visual flame/smoke classifier + sensor fusion; do not add classes that the product does not use.

Train locally with a model sized for the selected MCU. For N6 targets, use compact detector/segmentation/pose models; for H7, simplify resolution/classes and use fixed zones where possible.

Quantize to INT8 and evaluate per-condition accuracy, not only global mAP/accuracy.

If range sensing is present, synchronize camera frames with ToF/radar/IMU timestamps and tune fusion logic offline from recorded sessions.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Early warning event).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Factories/warehouses). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test each major environmental condition separately: day/night, backlight, occlusion, crowd density, dust/weather where applicable.

Measure missed-object/person rate and nuisance-alarm rate by scenario.

Measure end-to-end latency including camera exposure/capture, inference, fusion and output.

Force sensor disconnection/obstruction and verify a clear degraded/fault state.

Run a controlled pilot in advisory mode before enabling operational actions.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

## 50. AI gas-anomaly sensor

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Chemical plants/factories |
| Cost-first MCU | STM32U5 — STM32U5 cost-first target |
| Sensors / inputs | Gas sensor array + temp/humidity |
| AI formulation | Multivariate anomaly/classifier |
| Product output | Gas pattern alarm |

Use U575/U585/U595-class parts as starting points. Best for low-power sensor AI, audio/vibration, simple fusion and small models. Prototype with B-U585I-IOT02A when its onboard sensors are useful.

Cost-control strategy: Collect difficult human poses and negatives. AI should be a warning layer unless full safety engineering/certification is performed.

### Offline step-by-step development

Write a one-page requirement for “AI gas-anomaly sensor”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using Gas sensor array + temp/humidity. Fix sensor mounting before collecting the main dataset.

Collect representative video/images at the final mounting height and field of view. Include occlusion, backlight, low light, reflections and difficult negatives.

Label the minimum classes/targets needed for Multivariate anomaly/classifier; do not add classes that the product does not use.

Train locally with a model sized for the selected MCU. For N6 targets, use compact detector/segmentation/pose models; for H7, simplify resolution/classes and use fixed zones where possible.

Quantize to INT8 and evaluate per-condition accuracy, not only global mAP/accuracy.

If range sensing is present, synchronize camera frames with ToF/radar/IMU timestamps and tune fusion logic offline from recorded sessions.

Analyze/generate the model for STM32U5 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Gas pattern alarm).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Chemical plants/factories). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test each major environmental condition separately: day/night, backlight, occlusion, crowd density, dust/weather where applicable.

Measure missed-object/person rate and nuisance-alarm rate by scenario.

Measure end-to-end latency including camera exposure/capture, inference, fusion and output.

Force sensor disconnection/obstruction and verify a clear degraded/fault state.

Run a controlled pilot in advisory mode before enabling operational actions.

### Cost-down / upgrade decision

If measured CPU/RAM usage has large margin, evaluate a smaller U5 SKU or lower STM32 family during a formal cost-down phase. Move upward to H7 only when sample rate, channels, DSP or model size cannot meet timing with margin.

# Category — Buildings/retail

## 51. AI people counter

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Stores, malls, offices, factories |
| Cost-first MCU | STM32N6 — STM32N6 accelerated target |
| Sensors / inputs | Overhead RGB or ToF |
| AI formulation | Person detection/counting |
| Product output | Enter/exit count |

Use STM32N657-class devices when the workload genuinely needs detection/segmentation/pose/depth/tracking or more complex variable scenes. Prototype on STM32N6570-DK. Re-evaluate H7 only if the problem can be constrained substantially.

Cost-control strategy: Use privacy-respecting camera placement where possible; validate count error at peak density.

### Offline step-by-step development

Write a one-page requirement for “AI people counter”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using Overhead RGB or ToF. Fix sensor mounting before collecting the main dataset.

Collect representative video/images at the final mounting height and field of view. Include occlusion, backlight, low light, reflections and difficult negatives.

Label the minimum classes/targets needed for Person detection/counting; do not add classes that the product does not use.

Train locally with a model sized for the selected MCU. For N6 targets, use compact detector/segmentation/pose models; for H7, simplify resolution/classes and use fixed zones where possible.

Quantize to INT8 and evaluate per-condition accuracy, not only global mAP/accuracy.

If range sensing is present, synchronize camera frames with ToF/radar/IMU timestamps and tune fusion logic offline from recorded sessions.

Analyze/generate the model for STM32N6 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Enter/exit count).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Stores, malls, offices, factories). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test each major environmental condition separately: day/night, backlight, occlusion, crowd density, dust/weather where applicable.

Measure missed-object/person rate and nuisance-alarm rate by scenario.

Measure end-to-end latency including camera exposure/capture, inference, fusion and output.

Force sensor disconnection/obstruction and verify a clear degraded/fault state.

Run a controlled pilot in advisory mode before enabling operational actions.

### Cost-down / upgrade decision

N6 is selected because this product needs more complex variable-scene perception. Cost-down is possible only if the requirement can be constrained: fixed zones, fewer classes, lower resolution, reduced frame rate, external range sensor, or a simpler classifier. Re-test H7 only after that reformulation.

# Category — Buildings

## 52. AI occupancy sensor

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Offices/hotels/commercial |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | Low-res thermal/ToF/RGB |
| AI formulation | Presence/count classifier |
| Product output | Occupancy count/state |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Prefer ToF/thermal or low-resolution vision if identity is not required.

### Offline step-by-step development

Write a one-page requirement for “AI occupancy sensor”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using Low-res thermal/ToF/RGB. Fix sensor mounting before collecting the main dataset.

Collect representative video/images at the final mounting height and field of view. Include occlusion, backlight, low light, reflections and difficult negatives.

Label the minimum classes/targets needed for Presence/count classifier; do not add classes that the product does not use.

Train locally with a model sized for the selected MCU. For N6 targets, use compact detector/segmentation/pose models; for H7, simplify resolution/classes and use fixed zones where possible.

Quantize to INT8 and evaluate per-condition accuracy, not only global mAP/accuracy.

If range sensing is present, synchronize camera frames with ToF/radar/IMU timestamps and tune fusion logic offline from recorded sessions.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Occupancy count/state).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Offices/hotels/commercial). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test each major environmental condition separately: day/night, backlight, occlusion, crowd density, dust/weather where applicable.

Measure missed-object/person rate and nuisance-alarm rate by scenario.

Measure end-to-end latency including camera exposure/capture, inference, fusion and output.

Force sensor disconnection/obstruction and verify a clear degraded/fault state.

Run a controlled pilot in advisory mode before enabling operational actions.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

# Category — Retail/service

## 53. AI queue-length camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Banks, airports, retail |
| Cost-first MCU | STM32N6 — STM32N6 accelerated target |
| Sensors / inputs | RGB camera |
| AI formulation | Person detection/tracking |
| Product output | Queue length/wait proxy |

Use STM32N657-class devices when the workload genuinely needs detection/segmentation/pose/depth/tracking or more complex variable scenes. Prototype on STM32N6570-DK. Re-evaluate H7 only if the problem can be constrained substantially.

Cost-control strategy: Collect crowds, occlusion and seasonal clothing conditions.

### Offline step-by-step development

Write a one-page requirement for “AI queue-length camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera. Fix sensor mounting before collecting the main dataset.

Collect representative video/images at the final mounting height and field of view. Include occlusion, backlight, low light, reflections and difficult negatives.

Label the minimum classes/targets needed for Person detection/tracking; do not add classes that the product does not use.

Train locally with a model sized for the selected MCU. For N6 targets, use compact detector/segmentation/pose models; for H7, simplify resolution/classes and use fixed zones where possible.

Quantize to INT8 and evaluate per-condition accuracy, not only global mAP/accuracy.

If range sensing is present, synchronize camera frames with ToF/radar/IMU timestamps and tune fusion logic offline from recorded sessions.

Analyze/generate the model for STM32N6 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Queue length/wait proxy).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Banks, airports, retail). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test each major environmental condition separately: day/night, backlight, occlusion, crowd density, dust/weather where applicable.

Measure missed-object/person rate and nuisance-alarm rate by scenario.

Measure end-to-end latency including camera exposure/capture, inference, fusion and output.

Force sensor disconnection/obstruction and verify a clear degraded/fault state.

Run a controlled pilot in advisory mode before enabling operational actions.

### Cost-down / upgrade decision

N6 is selected because this product needs more complex variable-scene perception. Cost-down is possible only if the requirement can be constrained: fixed zones, fewer classes, lower resolution, reduced frame rate, external range sensor, or a simpler classifier. Re-test H7 only after that reformulation.

# Category — Parking

## 54. AI parking-space sensor

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Malls/offices/parking operators |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB or ToF |
| AI formulation | Per-space occupancy classifier |
| Product output | Space free/occupied |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Treat each parking space as a fixed ROI to reduce compute cost.

### Offline step-by-step development

Write a one-page requirement for “AI parking-space sensor”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB or ToF. Fix sensor mounting before collecting the main dataset.

Capture raw images from the real prototype under the full expected range of normal conditions. Keep originals immutable and store metadata (batch, lighting, product type, camera settings, date/site).

Define the smallest useful region of interest and input resolution. If the object position is fixed, crop it before inference instead of asking the network to search the whole frame.

Label/prepare data for Per-space occupancy classifier. Split train/validation/final-test sets by batch/site when possible.

Train locally on the workstation. Start with a compact CNN or keypoint/OCR pipeline; only move to a detector/segmenter if the simpler formulation cannot meet the requirement.

Quantize to INT8 and compare final-test metrics before and after quantization. Save the calibration image list.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Space free/occupied).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Malls/offices/parking operators). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Final-test images come from a different batch/day/site where possible.

Measure false accept and false reject separately; do not report accuracy alone.

Test lighting high/low limits, dirty lens, small position shifts and camera restart.

Measure trigger-to-output latency under worst-case processing load.

Run at least a multi-hour or multi-thousand-cycle soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

# Category — Transport/logistics

## 55. AI vehicle counting unit

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Parking/campuses |
| Cost-first MCU | STM32N6 — STM32N6 accelerated target |
| Sensors / inputs | RGB camera |
| AI formulation | Vehicle detector/count |
| Product output | Vehicle count/class |

Use STM32N657-class devices when the workload genuinely needs detection/segmentation/pose/depth/tracking or more complex variable scenes. Prototype on STM32N6570-DK. Re-evaluate H7 only if the problem can be constrained substantially.

Cost-control strategy: Use fixed camera geometry and count lines; validate day/night and weather if outdoors.

### Offline step-by-step development

Write a one-page requirement for “AI vehicle counting unit”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera. Fix sensor mounting before collecting the main dataset.

Collect representative video/images at the final mounting height and field of view. Include occlusion, backlight, low light, reflections and difficult negatives.

Label the minimum classes/targets needed for Vehicle detector/count; do not add classes that the product does not use.

Train locally with a model sized for the selected MCU. For N6 targets, use compact detector/segmentation/pose models; for H7, simplify resolution/classes and use fixed zones where possible.

Quantize to INT8 and evaluate per-condition accuracy, not only global mAP/accuracy.

If range sensing is present, synchronize camera frames with ToF/radar/IMU timestamps and tune fusion logic offline from recorded sessions.

Analyze/generate the model for STM32N6 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Vehicle count/class).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Parking/campuses). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test each major environmental condition separately: day/night, backlight, occlusion, crowd density, dust/weather where applicable.

Measure missed-object/person rate and nuisance-alarm rate by scenario.

Measure end-to-end latency including camera exposure/capture, inference, fusion and output.

Force sensor disconnection/obstruction and verify a clear degraded/fault state.

Run a controlled pilot in advisory mode before enabling operational actions.

### Cost-down / upgrade decision

N6 is selected because this product needs more complex variable-scene perception. Cost-down is possible only if the requirement can be constrained: fixed zones, fewer classes, lower resolution, reduced frame rate, external range sensor, or a simpler classifier. Re-test H7 only after that reformulation.

# Category — Access/parking

## 56. AI license-plate edge camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Parking/business entrances |
| Cost-first MCU | STM32N6 — STM32N6 accelerated target |
| Sensors / inputs | RGB camera + IR illumination |
| AI formulation | Plate detector + OCR |
| Product output | Plate string/confidence |

Use STM32N657-class devices when the workload genuinely needs detection/segmentation/pose/depth/tracking or more complex variable scenes. Prototype on STM32N6570-DK. Re-evaluate H7 only if the problem can be constrained substantially.

Cost-control strategy: Use controlled illumination and constrained capture distance/angle for plate OCR.

### Offline step-by-step development

Write a one-page requirement for “AI license-plate edge camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera + IR illumination. Fix sensor mounting before collecting the main dataset.

Capture raw images from the real prototype under the full expected range of normal conditions. Keep originals immutable and store metadata (batch, lighting, product type, camera settings, date/site).

Define the smallest useful region of interest and input resolution. If the object position is fixed, crop it before inference instead of asking the network to search the whole frame.

Label/prepare data for Plate detector + OCR. Split train/validation/final-test sets by batch/site when possible.

Train locally on the workstation. Start with a compact CNN or keypoint/OCR pipeline; only move to a detector/segmenter if the simpler formulation cannot meet the requirement.

Quantize to INT8 and compare final-test metrics before and after quantization. Save the calibration image list.

Analyze/generate the model for STM32N6 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Plate string/confidence).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Parking/business entrances). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Final-test images come from a different batch/day/site where possible.

Measure false accept and false reject separately; do not report accuracy alone.

Test lighting high/low limits, dirty lens, small position shifts and camera restart.

Measure trigger-to-output latency under worst-case processing load.

Run at least a multi-hour or multi-thousand-cycle soak test.

### Cost-down / upgrade decision

N6 is selected because this product needs more complex variable-scene perception. Cost-down is possible only if the requirement can be constrained: fixed zones, fewer classes, lower resolution, reduced frame rate, external range sensor, or a simpler classifier. Re-test H7 only after that reformulation.

# Category — Buildings

## 57. AI access-control terminal

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Offices/factories/hotels |
| Cost-first MCU | STM32N6 — STM32N6 accelerated target |
| Sensors / inputs | RGB camera + keypad/card |
| AI formulation | Face recognition/verification |
| Product output | Identity match |

Use STM32N657-class devices when the workload genuinely needs detection/segmentation/pose/depth/tracking or more complex variable scenes. Prototype on STM32N6570-DK. Re-evaluate H7 only if the problem can be constrained substantially.

Cost-control strategy: Prefer ToF/thermal or low-resolution vision if identity is not required.

### Offline step-by-step development

Write a one-page requirement for “AI access-control terminal”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera + keypad/card. Fix sensor mounting before collecting the main dataset.

Collect representative video/images at the final mounting height and field of view. Include occlusion, backlight, low light, reflections and difficult negatives.

Label the minimum classes/targets needed for Face recognition/verification; do not add classes that the product does not use.

Train locally with a model sized for the selected MCU. For N6 targets, use compact detector/segmentation/pose models; for H7, simplify resolution/classes and use fixed zones where possible.

Quantize to INT8 and evaluate per-condition accuracy, not only global mAP/accuracy.

If range sensing is present, synchronize camera frames with ToF/radar/IMU timestamps and tune fusion logic offline from recorded sessions.

Analyze/generate the model for STM32N6 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Identity match).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Offices/factories/hotels). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test each major environmental condition separately: day/night, backlight, occlusion, crowd density, dust/weather where applicable.

Measure missed-object/person rate and nuisance-alarm rate by scenario.

Measure end-to-end latency including camera exposure/capture, inference, fusion and output.

Force sensor disconnection/obstruction and verify a clear degraded/fault state.

Run a controlled pilot in advisory mode before enabling operational actions.

### Cost-down / upgrade decision

N6 is selected because this product needs more complex variable-scene perception. Cost-down is possible only if the requirement can be constrained: fixed zones, fewer classes, lower resolution, reduced frame rate, external range sensor, or a simpler classifier. Re-test H7 only after that reformulation.

# Category — Security

## 58. AI face + liveness access terminal

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Secure factories/offices |
| Cost-first MCU | STM32N6 — STM32N6 accelerated target |
| Sensors / inputs | RGB + ToF |
| AI formulation | Face recognition + liveness |
| Product output | Authorized + live face |

Use STM32N657-class devices when the workload genuinely needs detection/segmentation/pose/depth/tracking or more complex variable scenes. Prototype on STM32N6570-DK. Re-evaluate H7 only if the problem can be constrained substantially.

Cost-control strategy: Use face verification plus liveness/depth; protect templates/credentials and implement secure provisioning.

### Offline step-by-step development

Write a one-page requirement for “AI face + liveness access terminal”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB + ToF. Fix sensor mounting before collecting the main dataset.

Collect representative video/images at the final mounting height and field of view. Include occlusion, backlight, low light, reflections and difficult negatives.

Label the minimum classes/targets needed for Face recognition + liveness; do not add classes that the product does not use.

Train locally with a model sized for the selected MCU. For N6 targets, use compact detector/segmentation/pose models; for H7, simplify resolution/classes and use fixed zones where possible.

Quantize to INT8 and evaluate per-condition accuracy, not only global mAP/accuracy.

If range sensing is present, synchronize camera frames with ToF/radar/IMU timestamps and tune fusion logic offline from recorded sessions.

Analyze/generate the model for STM32N6 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Authorized + live face).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Secure factories/offices). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test each major environmental condition separately: day/night, backlight, occlusion, crowd density, dust/weather where applicable.

Measure missed-object/person rate and nuisance-alarm rate by scenario.

Measure end-to-end latency including camera exposure/capture, inference, fusion and output.

Force sensor disconnection/obstruction and verify a clear degraded/fault state.

Run a controlled pilot in advisory mode before enabling operational actions.

### Cost-down / upgrade decision

N6 is selected because this product needs more complex variable-scene perception. Cost-down is possible only if the requirement can be constrained: fixed zones, fewer classes, lower resolution, reduced frame rate, external range sensor, or a simpler classifier. Re-test H7 only after that reformulation.

# Category — Buildings

## 59. AI elevator occupancy sensor

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Commercial buildings |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | Ceiling ToF/thermal/RGB |
| AI formulation | People count/occupancy |
| Product output | Occupancy level |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Prefer ToF/thermal or low-resolution vision if identity is not required.

### Offline step-by-step development

Write a one-page requirement for “AI elevator occupancy sensor”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using Ceiling ToF/thermal/RGB. Fix sensor mounting before collecting the main dataset.

Collect representative video/images at the final mounting height and field of view. Include occlusion, backlight, low light, reflections and difficult negatives.

Label the minimum classes/targets needed for People count/occupancy; do not add classes that the product does not use.

Train locally with a model sized for the selected MCU. For N6 targets, use compact detector/segmentation/pose models; for H7, simplify resolution/classes and use fixed zones where possible.

Quantize to INT8 and evaluate per-condition accuracy, not only global mAP/accuracy.

If range sensing is present, synchronize camera frames with ToF/radar/IMU timestamps and tune fusion logic offline from recorded sessions.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Occupancy level).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Commercial buildings). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test each major environmental condition separately: day/night, backlight, occlusion, crowd density, dust/weather where applicable.

Measure missed-object/person rate and nuisance-alarm rate by scenario.

Measure end-to-end latency including camera exposure/capture, inference, fusion and output.

Force sensor disconnection/obstruction and verify a clear degraded/fault state.

Run a controlled pilot in advisory mode before enabling operational actions.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

# Category — Public facilities

## 60. AI escalator safety camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Malls/airports/stations |
| Cost-first MCU | STM32N6 — STM32N6 accelerated target |
| Sensors / inputs | RGB camera |
| AI formulation | Person/fall/abnormal posture detection |
| Product output | Safety event |

Use STM32N657-class devices when the workload genuinely needs detection/segmentation/pose/depth/tracking or more complex variable scenes. Prototype on STM32N6570-DK. Re-evaluate H7 only if the problem can be constrained substantially.

Cost-control strategy: Validate crowd/occlusion and abnormal-pose events; treat as monitoring, not sole safety protection.

### Offline step-by-step development

Write a one-page requirement for “AI escalator safety camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera. Fix sensor mounting before collecting the main dataset.

Collect representative video/images at the final mounting height and field of view. Include occlusion, backlight, low light, reflections and difficult negatives.

Label the minimum classes/targets needed for Person/fall/abnormal posture detection; do not add classes that the product does not use.

Train locally with a model sized for the selected MCU. For N6 targets, use compact detector/segmentation/pose models; for H7, simplify resolution/classes and use fixed zones where possible.

Quantize to INT8 and evaluate per-condition accuracy, not only global mAP/accuracy.

If range sensing is present, synchronize camera frames with ToF/radar/IMU timestamps and tune fusion logic offline from recorded sessions.

Analyze/generate the model for STM32N6 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Safety event).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Malls/airports/stations). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test each major environmental condition separately: day/night, backlight, occlusion, crowd density, dust/weather where applicable.

Measure missed-object/person rate and nuisance-alarm rate by scenario.

Measure end-to-end latency including camera exposure/capture, inference, fusion and output.

Force sensor disconnection/obstruction and verify a clear degraded/fault state.

Run a controlled pilot in advisory mode before enabling operational actions.

### Cost-down / upgrade decision

N6 is selected because this product needs more complex variable-scene perception. Cost-down is possible only if the requirement can be constrained: fixed zones, fewer classes, lower resolution, reduced frame rate, external range sensor, or a simpler classifier. Re-test H7 only after that reformulation.

# Category — Retail

## 61. AI smart-retail shelf camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Supermarkets |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB camera |
| AI formulation | Product presence/count |
| Product output | Shelf stock state |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Use shelf/slot/scale constraints to reduce model complexity.

### Offline step-by-step development

Write a one-page requirement for “AI smart-retail shelf camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera. Fix sensor mounting before collecting the main dataset.

Capture raw images from the real prototype under the full expected range of normal conditions. Keep originals immutable and store metadata (batch, lighting, product type, camera settings, date/site).

Define the smallest useful region of interest and input resolution. If the object position is fixed, crop it before inference instead of asking the network to search the whole frame.

Label/prepare data for Product presence/count. Split train/validation/final-test sets by batch/site when possible.

Train locally on the workstation. Start with a compact CNN or keypoint/OCR pipeline; only move to a detector/segmenter if the simpler formulation cannot meet the requirement.

Quantize to INT8 and compare final-test metrics before and after quantization. Save the calibration image list.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Shelf stock state).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Supermarkets). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Final-test images come from a different batch/day/site where possible.

Measure false accept and false reject separately; do not report accuracy alone.

Test lighting high/low limits, dirty lens, small position shifts and camera restart.

Measure trigger-to-output latency under worst-case processing load.

Run at least a multi-hour or multi-thousand-cycle soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

## 62. AI refrigerator inventory camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Supermarkets/convenience |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB camera + door sensor |
| AI formulation | Product recognition/count |
| Product output | Inventory delta |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Use shelf/slot/scale constraints to reduce model complexity.

### Offline step-by-step development

Write a one-page requirement for “AI refrigerator inventory camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera + door sensor. Fix sensor mounting before collecting the main dataset.

Capture raw images from the real prototype under the full expected range of normal conditions. Keep originals immutable and store metadata (batch, lighting, product type, camera settings, date/site).

Define the smallest useful region of interest and input resolution. If the object position is fixed, crop it before inference instead of asking the network to search the whole frame.

Label/prepare data for Product recognition/count. Split train/validation/final-test sets by batch/site when possible.

Train locally on the workstation. Start with a compact CNN or keypoint/OCR pipeline; only move to a detector/segmenter if the simpler formulation cannot meet the requirement.

Quantize to INT8 and compare final-test metrics before and after quantization. Save the calibration image list.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Inventory delta).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Supermarkets/convenience). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Final-test images come from a different batch/day/site where possible.

Measure false accept and false reject separately; do not report accuracy alone.

Test lighting high/low limits, dirty lens, small position shifts and camera restart.

Measure trigger-to-output latency under worst-case processing load.

Run at least a multi-hour or multi-thousand-cycle soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

## 63. AI vending-machine inventory sensor

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Vending operators |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB camera + door sensor |
| AI formulation | Slot/product classifier |
| Product output | Stock count/state |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Use shelf/slot/scale constraints to reduce model complexity.

### Offline step-by-step development

Write a one-page requirement for “AI vending-machine inventory sensor”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera + door sensor. Fix sensor mounting before collecting the main dataset.

Capture raw images from the real prototype under the full expected range of normal conditions. Keep originals immutable and store metadata (batch, lighting, product type, camera settings, date/site).

Define the smallest useful region of interest and input resolution. If the object position is fixed, crop it before inference instead of asking the network to search the whole frame.

Label/prepare data for Slot/product classifier. Split train/validation/final-test sets by batch/site when possible.

Train locally on the workstation. Start with a compact CNN or keypoint/OCR pipeline; only move to a detector/segmenter if the simpler formulation cannot meet the requirement.

Quantize to INT8 and compare final-test metrics before and after quantization. Save the calibration image list.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Stock count/state).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Vending operators). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Final-test images come from a different batch/day/site where possible.

Measure false accept and false reject separately; do not report accuracy alone.

Test lighting high/low limits, dirty lens, small position shifts and camera restart.

Measure trigger-to-output latency under worst-case processing load.

Run at least a multi-hour or multi-thousand-cycle soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

## 64. AI automatic checkout camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Stores/cafeterias |
| Cost-first MCU | STM32N6 — STM32N6 accelerated target |
| Sensors / inputs | RGB camera + weight sensor optional |
| AI formulation | Multi-object/product recognition |
| Product output | Product list |

Use STM32N657-class devices when the workload genuinely needs detection/segmentation/pose/depth/tracking or more complex variable scenes. Prototype on STM32N6570-DK. Re-evaluate H7 only if the problem can be constrained substantially.

Cost-control strategy: Use shelf/slot/scale constraints to reduce model complexity.

### Offline step-by-step development

Write a one-page requirement for “AI automatic checkout camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera + weight sensor optional. Fix sensor mounting before collecting the main dataset.

Capture raw images from the real prototype under the full expected range of normal conditions. Keep originals immutable and store metadata (batch, lighting, product type, camera settings, date/site).

Define the smallest useful region of interest and input resolution. If the object position is fixed, crop it before inference instead of asking the network to search the whole frame.

Label/prepare data for Multi-object/product recognition. Split train/validation/final-test sets by batch/site when possible.

Train locally on the workstation. Start with a compact CNN or keypoint/OCR pipeline; only move to a detector/segmenter if the simpler formulation cannot meet the requirement.

Quantize to INT8 and compare final-test metrics before and after quantization. Save the calibration image list.

Analyze/generate the model for STM32N6 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Product list).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Stores/cafeterias). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Final-test images come from a different batch/day/site where possible.

Measure false accept and false reject separately; do not report accuracy alone.

Test lighting high/low limits, dirty lens, small position shifts and camera restart.

Measure trigger-to-output latency under worst-case processing load.

Run at least a multi-hour or multi-thousand-cycle soak test.

### Cost-down / upgrade decision

N6 is selected because this product needs more complex variable-scene perception. Cost-down is possible only if the requirement can be constrained: fixed zones, fewer classes, lower resolution, reduced frame rate, external range sensor, or a simpler classifier. Re-test H7 only after that reformulation.

## 65. AI produce recognition unit

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Supermarkets |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB camera + scale |
| AI formulation | Produce classifier |
| Product output | Produce class + confidence |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Use shelf/slot/scale constraints to reduce model complexity.

### Offline step-by-step development

Write a one-page requirement for “AI produce recognition unit”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera + scale. Fix sensor mounting before collecting the main dataset.

Capture raw images from the real prototype under the full expected range of normal conditions. Keep originals immutable and store metadata (batch, lighting, product type, camera settings, date/site).

Define the smallest useful region of interest and input resolution. If the object position is fixed, crop it before inference instead of asking the network to search the whole frame.

Label/prepare data for Produce classifier. Split train/validation/final-test sets by batch/site when possible.

Train locally on the workstation. Start with a compact CNN or keypoint/OCR pipeline; only move to a detector/segmenter if the simpler formulation cannot meet the requirement.

Quantize to INT8 and compare final-test metrics before and after quantization. Save the calibration image list.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Produce class + confidence).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Supermarkets). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Final-test images come from a different batch/day/site where possible.

Measure false accept and false reject separately; do not report accuracy alone.

Test lighting high/low limits, dirty lens, small position shifts and camera restart.

Measure trigger-to-output latency under worst-case processing load.

Run at least a multi-hour or multi-thousand-cycle soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

# Category — Food processing

## 66. AI food quality grader

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Food plants |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB camera + controlled light |
| AI formulation | Quality classifier/anomaly |
| Product output | Grade/defect |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Build cleanable optics/lighting fixtures and include normal product variability.

### Offline step-by-step development

Write a one-page requirement for “AI food quality grader”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera + controlled light. Fix sensor mounting before collecting the main dataset.

Capture raw images from the real prototype under the full expected range of normal conditions. Keep originals immutable and store metadata (batch, lighting, product type, camera settings, date/site).

Define the smallest useful region of interest and input resolution. If the object position is fixed, crop it before inference instead of asking the network to search the whole frame.

Label/prepare data for Quality classifier/anomaly. Split train/validation/final-test sets by batch/site when possible.

Train locally on the workstation. Start with a compact CNN or keypoint/OCR pipeline; only move to a detector/segmenter if the simpler formulation cannot meet the requirement.

Quantize to INT8 and compare final-test metrics before and after quantization. Save the calibration image list.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Grade/defect).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Food plants). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Final-test images come from a different batch/day/site where possible.

Measure false accept and false reject separately; do not report accuracy alone.

Test lighting high/low limits, dirty lens, small position shifts and camera restart.

Measure trigger-to-output latency under worst-case processing load.

Run at least a multi-hour or multi-thousand-cycle soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

# Category — Agriculture/food

## 67. AI fruit sorting/grading camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Packing facilities |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB camera + conveyor trigger |
| AI formulation | Fruit class/defect/ripeness classifier |
| Product output | Grade + diverter class |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Control packing-line background and trigger; include seasonal varieties.

### Offline step-by-step development

Write a one-page requirement for “AI fruit sorting/grading camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera + conveyor trigger. Fix sensor mounting before collecting the main dataset.

Capture raw images from the real prototype under the full expected range of normal conditions. Keep originals immutable and store metadata (batch, lighting, product type, camera settings, date/site).

Define the smallest useful region of interest and input resolution. If the object position is fixed, crop it before inference instead of asking the network to search the whole frame.

Label/prepare data for Fruit class/defect/ripeness classifier. Split train/validation/final-test sets by batch/site when possible.

Train locally on the workstation. Start with a compact CNN or keypoint/OCR pipeline; only move to a detector/segmenter if the simpler formulation cannot meet the requirement.

Quantize to INT8 and compare final-test metrics before and after quantization. Save the calibration image list.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Grade + diverter class).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Packing facilities). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Final-test images come from a different batch/day/site where possible.

Measure false accept and false reject separately; do not report accuracy alone.

Test lighting high/low limits, dirty lens, small position shifts and camera restart.

Measure trigger-to-output latency under worst-case processing load.

Run at least a multi-hour or multi-thousand-cycle soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

## 68. AI vegetable grading camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Packing facilities |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB camera |
| AI formulation | Quality/size classifier |
| Product output | Grade |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Control packing-line background and trigger; include seasonal varieties.

### Offline step-by-step development

Write a one-page requirement for “AI vegetable grading camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera. Fix sensor mounting before collecting the main dataset.

Capture raw images from the real prototype under the full expected range of normal conditions. Keep originals immutable and store metadata (batch, lighting, product type, camera settings, date/site).

Define the smallest useful region of interest and input resolution. If the object position is fixed, crop it before inference instead of asking the network to search the whole frame.

Label/prepare data for Quality/size classifier. Split train/validation/final-test sets by batch/site when possible.

Train locally on the workstation. Start with a compact CNN or keypoint/OCR pipeline; only move to a detector/segmenter if the simpler formulation cannot meet the requirement.

Quantize to INT8 and compare final-test metrics before and after quantization. Save the calibration image list.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Grade).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Packing facilities). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Final-test images come from a different batch/day/site where possible.

Measure false accept and false reject separately; do not report accuracy alone.

Test lighting high/low limits, dirty lens, small position shifts and camera restart.

Measure trigger-to-output latency under worst-case processing load.

Run at least a multi-hour or multi-thousand-cycle soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

# Category — Food

## 69. AI egg quality inspection camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Poultry industry |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | Backlight/RGB camera |
| AI formulation | Crack/shape/cleanliness classifier |
| Product output | Egg quality grade |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Control illumination/background and collect normal color/shape variability.

### Offline step-by-step development

Write a one-page requirement for “AI egg quality inspection camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using Backlight/RGB camera. Fix sensor mounting before collecting the main dataset.

Capture raw images from the real prototype under the full expected range of normal conditions. Keep originals immutable and store metadata (batch, lighting, product type, camera settings, date/site).

Define the smallest useful region of interest and input resolution. If the object position is fixed, crop it before inference instead of asking the network to search the whole frame.

Label/prepare data for Crack/shape/cleanliness classifier. Split train/validation/final-test sets by batch/site when possible.

Train locally on the workstation. Start with a compact CNN or keypoint/OCR pipeline; only move to a detector/segmenter if the simpler formulation cannot meet the requirement.

Quantize to INT8 and compare final-test metrics before and after quantization. Save the calibration image list.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Egg quality grade).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Poultry industry). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Final-test images come from a different batch/day/site where possible.

Measure false accept and false reject separately; do not report accuracy alone.

Test lighting high/low limits, dirty lens, small position shifts and camera restart.

Measure trigger-to-output latency under worst-case processing load.

Run at least a multi-hour or multi-thousand-cycle soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

# Category — Food/aquaculture

## 70. AI fish grading/counting camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Fisheries |
| Cost-first MCU | STM32N6 — STM32N6 accelerated target |
| Sensors / inputs | RGB camera |
| AI formulation | Fish detection/size class |
| Product output | Count + grade |

Use STM32N657-class devices when the workload genuinely needs detection/segmentation/pose/depth/tracking or more complex variable scenes. Prototype on STM32N6570-DK. Re-evaluate H7 only if the problem can be constrained substantially.

Cost-control strategy: Constrain the product problem and choose the smallest model that solves it.

### Offline step-by-step development

Write a one-page requirement for “AI fish grading/counting camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera. Fix sensor mounting before collecting the main dataset.

Collect representative data locally.

Prepare labels and train the smallest suitable model.

Quantize and validate on the final test set.

Analyze/generate the model for STM32N6 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Count + grade).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Fisheries). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test on machines/sessions not represented in training.

Sweep operating speed/load/temperature and verify the model does not confuse operating condition with failure.

Compare MCU preprocessing/inference with the offline Python reference on identical windows.

Test sensor disconnect, saturation and mounting changes.

Run a long-duration data-logging and inference soak test.

### Cost-down / upgrade decision

N6 is selected because this product needs more complex variable-scene perception. Cost-down is possible only if the requirement can be constrained: fixed zones, fewer classes, lower resolution, reduced frame rate, external range sensor, or a simpler classifier. Re-test H7 only after that reformulation.

# Category — Food

## 71. AI meat inspection camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Food processors |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB camera + controlled light |
| AI formulation | Surface quality/anomaly classifier |
| Product output | Quality/contamination flag |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Control illumination/background and collect normal color/shape variability.

### Offline step-by-step development

Write a one-page requirement for “AI meat inspection camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera + controlled light. Fix sensor mounting before collecting the main dataset.

Capture raw images from the real prototype under the full expected range of normal conditions. Keep originals immutable and store metadata (batch, lighting, product type, camera settings, date/site).

Define the smallest useful region of interest and input resolution. If the object position is fixed, crop it before inference instead of asking the network to search the whole frame.

Label/prepare data for Surface quality/anomaly classifier. Split train/validation/final-test sets by batch/site when possible.

Train locally on the workstation. Start with a compact CNN or keypoint/OCR pipeline; only move to a detector/segmenter if the simpler formulation cannot meet the requirement.

Quantize to INT8 and compare final-test metrics before and after quantization. Save the calibration image list.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Quality/contamination flag).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Food processors). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Final-test images come from a different batch/day/site where possible.

Measure false accept and false reject separately; do not report accuracy alone.

Test lighting high/low limits, dirty lens, small position shifts and camera restart.

Measure trigger-to-output latency under worst-case processing load.

Run at least a multi-hour or multi-thousand-cycle soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

## 72. AI bakery product inspection camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Commercial bakeries |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB camera |
| AI formulation | Shape/color/burn classifier |
| Product output | Grade/reject |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Control illumination/background and collect normal color/shape variability.

### Offline step-by-step development

Write a one-page requirement for “AI bakery product inspection camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera. Fix sensor mounting before collecting the main dataset.

Capture raw images from the real prototype under the full expected range of normal conditions. Keep originals immutable and store metadata (batch, lighting, product type, camera settings, date/site).

Define the smallest useful region of interest and input resolution. If the object position is fixed, crop it before inference instead of asking the network to search the whole frame.

Label/prepare data for Shape/color/burn classifier. Split train/validation/final-test sets by batch/site when possible.

Train locally on the workstation. Start with a compact CNN or keypoint/OCR pipeline; only move to a detector/segmenter if the simpler formulation cannot meet the requirement.

Quantize to INT8 and compare final-test metrics before and after quantization. Save the calibration image list.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Grade/reject).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Commercial bakeries). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Final-test images come from a different batch/day/site where possible.

Measure false accept and false reject separately; do not report accuracy alone.

Test lighting high/low limits, dirty lens, small position shifts and camera restart.

Measure trigger-to-output latency under worst-case processing load.

Run at least a multi-hour or multi-thousand-cycle soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

# Category — Agriculture

## 73. AI crop disease camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Farms/greenhouses |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB camera |
| AI formulation | Leaf disease classifier/segmentation |
| Product output | Disease class/severity |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Collect across season, cultivar, sunlight and soil/background conditions.

### Offline step-by-step development

Write a one-page requirement for “AI crop disease camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera. Fix sensor mounting before collecting the main dataset.

Collect data across multiple days, sunlight conditions, plant ages/varieties and backgrounds. Avoid a dataset from one greenhouse corner only.

Label the minimum visual targets for Leaf disease classifier/segmentation.

Train a compact local model; use color/texture normalization only if it remains stable on the target camera.

Quantize to INT8 and test on a site/date that was not used for training.

Build a hard-case set for shadows, wet leaves, soil, overlapping plants and disease look-alikes.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Disease class/severity).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Farms/greenhouses). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test on machines/sessions not represented in training.

Sweep operating speed/load/temperature and verify the model does not confuse operating condition with failure.

Compare MCU preprocessing/inference with the offline Python reference on identical windows.

Test sensor disconnect, saturation and mounting changes.

Run a long-duration data-logging and inference soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

## 74. AI weed-vs-crop detector

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Agricultural equipment |
| Cost-first MCU | STM32N6 — STM32N6 accelerated target |
| Sensors / inputs | RGB camera |
| AI formulation | Segmentation/detection |
| Product output | Crop/weed mask |

Use STM32N657-class devices when the workload genuinely needs detection/segmentation/pose/depth/tracking or more complex variable scenes. Prototype on STM32N6570-DK. Re-evaluate H7 only if the problem can be constrained substantially.

Cost-control strategy: Collect across season, cultivar, sunlight and soil/background conditions.

### Offline step-by-step development

Write a one-page requirement for “AI weed-vs-crop detector”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera. Fix sensor mounting before collecting the main dataset.

Collect data across multiple days, sunlight conditions, plant ages/varieties and backgrounds. Avoid a dataset from one greenhouse corner only.

Label the minimum visual targets for Segmentation/detection.

Train a compact local model; use color/texture normalization only if it remains stable on the target camera.

Quantize to INT8 and test on a site/date that was not used for training.

Build a hard-case set for shadows, wet leaves, soil, overlapping plants and disease look-alikes.

Analyze/generate the model for STM32N6 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Crop/weed mask).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Agricultural equipment). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test on machines/sessions not represented in training.

Sweep operating speed/load/temperature and verify the model does not confuse operating condition with failure.

Compare MCU preprocessing/inference with the offline Python reference on identical windows.

Test sensor disconnect, saturation and mounting changes.

Run a long-duration data-logging and inference soak test.

### Cost-down / upgrade decision

N6 is selected because this product needs more complex variable-scene perception. Cost-down is possible only if the requirement can be constrained: fixed zones, fewer classes, lower resolution, reduced frame rate, external range sensor, or a simpler classifier. Re-test H7 only after that reformulation.

# Category — Agriculture robotics

## 75. AI fruit-picking vision unit

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Agricultural robots |
| Cost-first MCU | STM32N6 — STM32N6 accelerated target |
| Sensors / inputs | RGB + depth/ToF |
| AI formulation | Fruit detector + depth/grasp point |
| Product output | Fruit position |

Use STM32N657-class devices when the workload genuinely needs detection/segmentation/pose/depth/tracking or more complex variable scenes. Prototype on STM32N6570-DK. Re-evaluate H7 only if the problem can be constrained substantially.

Cost-control strategy: Use depth and calibrated manipulator geometry; validate fruit occlusion and foliage.

### Offline step-by-step development

Write a one-page requirement for “AI fruit-picking vision unit”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB + depth/ToF. Fix sensor mounting before collecting the main dataset.

Collect representative video/images at the final mounting height and field of view. Include occlusion, backlight, low light, reflections and difficult negatives.

Label the minimum classes/targets needed for Fruit detector + depth/grasp point; do not add classes that the product does not use.

Train locally with a model sized for the selected MCU. For N6 targets, use compact detector/segmentation/pose models; for H7, simplify resolution/classes and use fixed zones where possible.

Quantize to INT8 and evaluate per-condition accuracy, not only global mAP/accuracy.

If range sensing is present, synchronize camera frames with ToF/radar/IMU timestamps and tune fusion logic offline from recorded sessions.

Analyze/generate the model for STM32N6 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Fruit position).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Agricultural robots). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test each major environmental condition separately: day/night, backlight, occlusion, crowd density, dust/weather where applicable.

Measure missed-object/person rate and nuisance-alarm rate by scenario.

Measure end-to-end latency including camera exposure/capture, inference, fusion and output.

Force sensor disconnection/obstruction and verify a clear degraded/fault state.

Run a controlled pilot in advisory mode before enabling operational actions.

### Cost-down / upgrade decision

N6 is selected because this product needs more complex variable-scene perception. Cost-down is possible only if the requirement can be constrained: fixed zones, fewer classes, lower resolution, reduced frame rate, external range sensor, or a simpler classifier. Re-test H7 only after that reformulation.

# Category — Agriculture

## 76. AI livestock counting camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Farms |
| Cost-first MCU | STM32N6 — STM32N6 accelerated target |
| Sensors / inputs | RGB camera |
| AI formulation | Animal detection/counting |
| Product output | Count/flow |

Use STM32N657-class devices when the workload genuinely needs detection/segmentation/pose/depth/tracking or more complex variable scenes. Prototype on STM32N6570-DK. Re-evaluate H7 only if the problem can be constrained substantially.

Cost-control strategy: Collect across season, cultivar, sunlight and soil/background conditions.

### Offline step-by-step development

Write a one-page requirement for “AI livestock counting camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera. Fix sensor mounting before collecting the main dataset.

Collect data across multiple days, sunlight conditions, plant ages/varieties and backgrounds. Avoid a dataset from one greenhouse corner only.

Label the minimum visual targets for Animal detection/counting.

Train a compact local model; use color/texture normalization only if it remains stable on the target camera.

Quantize to INT8 and test on a site/date that was not used for training.

Build a hard-case set for shadows, wet leaves, soil, overlapping plants and disease look-alikes.

Analyze/generate the model for STM32N6 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Count/flow).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Farms). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test on machines/sessions not represented in training.

Sweep operating speed/load/temperature and verify the model does not confuse operating condition with failure.

Compare MCU preprocessing/inference with the offline Python reference on identical windows.

Test sensor disconnect, saturation and mounting changes.

Run a long-duration data-logging and inference soak test.

### Cost-down / upgrade decision

N6 is selected because this product needs more complex variable-scene perception. Cost-down is possible only if the requirement can be constrained: fixed zones, fewer classes, lower resolution, reduced frame rate, external range sensor, or a simpler classifier. Re-test H7 only after that reformulation.

## 77. AI livestock behavior monitor

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Dairy/poultry/pig farms |
| Cost-first MCU | STM32N6 — STM32N6 accelerated target |
| Sensors / inputs | RGB camera + optional IMU/acoustic |
| AI formulation | Pose/action classifier |
| Product output | Behavior/anomaly |

Use STM32N657-class devices when the workload genuinely needs detection/segmentation/pose/depth/tracking or more complex variable scenes. Prototype on STM32N6570-DK. Re-evaluate H7 only if the problem can be constrained substantially.

Cost-control strategy: Collect across season, cultivar, sunlight and soil/background conditions.

### Offline step-by-step development

Write a one-page requirement for “AI livestock behavior monitor”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera + optional IMU/acoustic. Fix sensor mounting before collecting the main dataset.

Collect data across multiple days, sunlight conditions, plant ages/varieties and backgrounds. Avoid a dataset from one greenhouse corner only.

Label the minimum visual targets for Pose/action classifier.

Train a compact local model; use color/texture normalization only if it remains stable on the target camera.

Quantize to INT8 and test on a site/date that was not used for training.

Build a hard-case set for shadows, wet leaves, soil, overlapping plants and disease look-alikes.

Analyze/generate the model for STM32N6 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Behavior/anomaly).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Dairy/poultry/pig farms). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test on machines/sessions not represented in training.

Sweep operating speed/load/temperature and verify the model does not confuse operating condition with failure.

Compare MCU preprocessing/inference with the offline Python reference on identical windows.

Test sensor disconnect, saturation and mounting changes.

Run a long-duration data-logging and inference soak test.

### Cost-down / upgrade decision

N6 is selected because this product needs more complex variable-scene perception. Cost-down is possible only if the requirement can be constrained: fixed zones, fewer classes, lower resolution, reduced frame rate, external range sensor, or a simpler classifier. Re-test H7 only after that reformulation.

## 78. AI feed-level camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Farms |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB or ToF |
| AI formulation | Fill-level classifier/geometry |
| Product output | Feed level |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Collect across season, cultivar, sunlight and soil/background conditions.

### Offline step-by-step development

Write a one-page requirement for “AI feed-level camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB or ToF. Fix sensor mounting before collecting the main dataset.

Collect data across multiple days, sunlight conditions, plant ages/varieties and backgrounds. Avoid a dataset from one greenhouse corner only.

Label the minimum visual targets for Fill-level classifier/geometry.

Train a compact local model; use color/texture normalization only if it remains stable on the target camera.

Quantize to INT8 and test on a site/date that was not used for training.

Build a hard-case set for shadows, wet leaves, soil, overlapping plants and disease look-alikes.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Feed level).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Farms). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test on machines/sessions not represented in training.

Sweep operating speed/load/temperature and verify the model does not confuse operating condition with failure.

Compare MCU preprocessing/inference with the offline Python reference on identical windows.

Test sensor disconnect, saturation and mounting changes.

Run a long-duration data-logging and inference soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

# Category — Recycling

## 79. AI waste-sorting camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Recycling plants |
| Cost-first MCU | STM32N6 — STM32N6 accelerated target |
| Sensors / inputs | RGB camera + conveyor encoder |
| AI formulation | Material/object classification |
| Product output | Material class + position |

Use STM32N657-class devices when the workload genuinely needs detection/segmentation/pose/depth/tracking or more complex variable scenes. Prototype on STM32N6570-DK. Re-evaluate H7 only if the problem can be constrained substantially.

Cost-control strategy: Use conveyor encoder/trigger and collect dirty, crushed and mixed materials.

### Offline step-by-step development

Write a one-page requirement for “AI waste-sorting camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera + conveyor encoder. Fix sensor mounting before collecting the main dataset.

Capture raw images from the real prototype under the full expected range of normal conditions. Keep originals immutable and store metadata (batch, lighting, product type, camera settings, date/site).

Define the smallest useful region of interest and input resolution. If the object position is fixed, crop it before inference instead of asking the network to search the whole frame.

Label/prepare data for Material/object classification. Split train/validation/final-test sets by batch/site when possible.

Train locally on the workstation. Start with a compact CNN or keypoint/OCR pipeline; only move to a detector/segmenter if the simpler formulation cannot meet the requirement.

Quantize to INT8 and compare final-test metrics before and after quantization. Save the calibration image list.

Analyze/generate the model for STM32N6 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Material class + position).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Recycling plants). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Final-test images come from a different batch/day/site where possible.

Measure false accept and false reject separately; do not report accuracy alone.

Test lighting high/low limits, dirty lens, small position shifts and camera restart.

Measure trigger-to-output latency under worst-case processing load.

Run at least a multi-hour or multi-thousand-cycle soak test.

### Cost-down / upgrade decision

N6 is selected because this product needs more complex variable-scene perception. Cost-down is possible only if the requirement can be constrained: fixed zones, fewer classes, lower resolution, reduced frame rate, external range sensor, or a simpler classifier. Re-test H7 only after that reformulation.

## 80. AI plastic classification sensor

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Recycling machinery |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB + optional NIR sensor |
| AI formulation | Material classifier |
| Product output | Plastic type |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Use conveyor encoder/trigger and collect dirty, crushed and mixed materials.

### Offline step-by-step development

Write a one-page requirement for “AI plastic classification sensor”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB + optional NIR sensor. Fix sensor mounting before collecting the main dataset.

Capture raw images from the real prototype under the full expected range of normal conditions. Keep originals immutable and store metadata (batch, lighting, product type, camera settings, date/site).

Define the smallest useful region of interest and input resolution. If the object position is fixed, crop it before inference instead of asking the network to search the whole frame.

Label/prepare data for Material classifier. Split train/validation/final-test sets by batch/site when possible.

Train locally on the workstation. Start with a compact CNN or keypoint/OCR pipeline; only move to a detector/segmenter if the simpler formulation cannot meet the requirement.

Quantize to INT8 and compare final-test metrics before and after quantization. Save the calibration image list.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Plastic type).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Recycling machinery). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Final-test images come from a different batch/day/site where possible.

Measure false accept and false reject separately; do not report accuracy alone.

Test lighting high/low limits, dirty lens, small position shifts and camera restart.

Measure trigger-to-output latency under worst-case processing load.

Run at least a multi-hour or multi-thousand-cycle soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

## 81. AI metal/non-metal sorting sensor

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Recycling plants |
| Cost-first MCU | STM32U5 — STM32U5 cost-first target |
| Sensors / inputs | Inductive sensor + RGB optional |
| AI formulation | Sensor fusion classifier |
| Product output | Metal/non-metal class |

Use U575/U585/U595-class parts as starting points. Best for low-power sensor AI, audio/vibration, simple fusion and small models. Prototype with B-U585I-IOT02A when its onboard sensors are useful.

Cost-control strategy: Use conveyor encoder/trigger and collect dirty, crushed and mixed materials.

### Offline step-by-step development

Write a one-page requirement for “AI metal/non-metal sorting sensor”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using Inductive sensor + RGB optional. Fix sensor mounting before collecting the main dataset.

Capture raw images from the real prototype under the full expected range of normal conditions. Keep originals immutable and store metadata (batch, lighting, product type, camera settings, date/site).

Define the smallest useful region of interest and input resolution. If the object position is fixed, crop it before inference instead of asking the network to search the whole frame.

Label/prepare data for Sensor fusion classifier. Split train/validation/final-test sets by batch/site when possible.

Train locally on the workstation. Start with a compact CNN or keypoint/OCR pipeline; only move to a detector/segmenter if the simpler formulation cannot meet the requirement.

Quantize to INT8 and compare final-test metrics before and after quantization. Save the calibration image list.

Analyze/generate the model for STM32U5 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Metal/non-metal class).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Recycling plants). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Final-test images come from a different batch/day/site where possible.

Measure false accept and false reject separately; do not report accuracy alone.

Test lighting high/low limits, dirty lens, small position shifts and camera restart.

Measure trigger-to-output latency under worst-case processing load.

Run at least a multi-hour or multi-thousand-cycle soak test.

### Cost-down / upgrade decision

If measured CPU/RAM usage has large margin, evaluate a smaller U5 SKU or lower STM32 family during a formal cost-down phase. Move upward to H7 only when sample rate, channels, DSP or model size cannot meet timing with margin.

## 82. AI contamination detector for recycling

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Waste processing |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB camera |
| AI formulation | Anomaly/foreign-object detector |
| Product output | Contamination flag |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Use conveyor encoder/trigger and collect dirty, crushed and mixed materials.

### Offline step-by-step development

Write a one-page requirement for “AI contamination detector for recycling”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera. Fix sensor mounting before collecting the main dataset.

Capture raw images from the real prototype under the full expected range of normal conditions. Keep originals immutable and store metadata (batch, lighting, product type, camera settings, date/site).

Define the smallest useful region of interest and input resolution. If the object position is fixed, crop it before inference instead of asking the network to search the whole frame.

Label/prepare data for Anomaly/foreign-object detector. Split train/validation/final-test sets by batch/site when possible.

Train locally on the workstation. Start with a compact CNN or keypoint/OCR pipeline; only move to a detector/segmenter if the simpler formulation cannot meet the requirement.

Quantize to INT8 and compare final-test metrics before and after quantization. Save the calibration image list.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Contamination flag).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Waste processing). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Final-test images come from a different batch/day/site where possible.

Measure false accept and false reject separately; do not report accuracy alone.

Test lighting high/low limits, dirty lens, small position shifts and camera restart.

Measure trigger-to-output latency under worst-case processing load.

Run at least a multi-hour or multi-thousand-cycle soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

# Category — Waste management

## 83. AI smart trash-bin sensor

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Commercial buildings/cities |
| Cost-first MCU | STM32U5 — STM32U5 cost-first target |
| Sensors / inputs | ToF/ultrasonic + optional camera |
| AI formulation | Fill/anomaly classifier |
| Product output | Fill level / blockage |

Use U575/U585/U595-class parts as starting points. Best for low-power sensor AI, audio/vibration, simple fusion and small models. Prototype with B-U585I-IOT02A when its onboard sensors are useful.

Cost-control strategy: Use ToF/ultrasonic for fill and only add camera AI if material classification is required.

### Offline step-by-step development

Write a one-page requirement for “AI smart trash-bin sensor”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using ToF/ultrasonic + optional camera. Fix sensor mounting before collecting the main dataset.

Record synchronized raw sensor streams across the full healthy operating envelope; log sample rate, sensor orientation, machine/load/state and temperature.

Segment data into fixed windows and evaluate both engineered features (RMS, kurtosis, FFT bands, temperature trends) and a small 1D CNN/MLP/anomaly model.

Prepare labels or normal/anomaly targets for Fill/anomaly classifier. Keep entire machines/sites out of the training set for a stronger final test where possible.

Train locally, quantize, and verify that the model is learning the intended physical condition rather than a shortcut such as RPM or background noise.

Reproduce exactly the same preprocessing in C on the MCU and compare window-by-window outputs against the offline Python reference.

Analyze/generate the model for STM32U5 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Fill level / blockage).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Commercial buildings/cities). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test on machines/sessions not represented in training.

Sweep operating speed/load/temperature and verify the model does not confuse operating condition with failure.

Compare MCU preprocessing/inference with the offline Python reference on identical windows.

Test sensor disconnect, saturation and mounting changes.

Run a long-duration data-logging and inference soak test.

### Cost-down / upgrade decision

If measured CPU/RAM usage has large margin, evaluate a smaller U5 SKU or lower STM32 family during a formal cost-down phase. Move upward to H7 only when sample rate, channels, DSP or model size cannot meet timing with margin.

# Category — Energy

## 84. AI solar-panel defect camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Solar farms/maintenance |
| Cost-first MCU | STM32N6 — STM32N6 accelerated target |
| Sensors / inputs | RGB/thermal camera |
| AI formulation | Defect/anomaly detection |
| Product output | Panel defect flag |

Use STM32N657-class devices when the workload genuinely needs detection/segmentation/pose/depth/tracking or more complex variable scenes. Prototype on STM32N6570-DK. Re-evaluate H7 only if the problem can be constrained substantially.

Cost-control strategy: Capture environmental/weather/thermal variation; validate false alarms under normal aging/soiling.

### Offline step-by-step development

Write a one-page requirement for “AI solar-panel defect camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB/thermal camera. Fix sensor mounting before collecting the main dataset.

Collect representative video/images at the final mounting height and field of view. Include occlusion, backlight, low light, reflections and difficult negatives.

Label the minimum classes/targets needed for Defect/anomaly detection; do not add classes that the product does not use.

Train locally with a model sized for the selected MCU. For N6 targets, use compact detector/segmentation/pose models; for H7, simplify resolution/classes and use fixed zones where possible.

Quantize to INT8 and evaluate per-condition accuracy, not only global mAP/accuracy.

If range sensing is present, synchronize camera frames with ToF/radar/IMU timestamps and tune fusion logic offline from recorded sessions.

Analyze/generate the model for STM32N6 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Panel defect flag).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Solar farms/maintenance). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test each major environmental condition separately: day/night, backlight, occlusion, crowd density, dust/weather where applicable.

Measure missed-object/person rate and nuisance-alarm rate by scenario.

Measure end-to-end latency including camera exposure/capture, inference, fusion and output.

Force sensor disconnection/obstruction and verify a clear degraded/fault state.

Run a controlled pilot in advisory mode before enabling operational actions.

### Cost-down / upgrade decision

N6 is selected because this product needs more complex variable-scene perception. Cost-down is possible only if the requirement can be constrained: fixed zones, fewer classes, lower resolution, reduced frame rate, external range sensor, or a simpler classifier. Re-test H7 only after that reformulation.

## 85. AI wind-turbine monitoring camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Wind farms |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB camera + vibration optional |
| AI formulation | Visual anomaly + condition model |
| Product output | Damage/anomaly event |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Capture environmental/weather/thermal variation; validate false alarms under normal aging/soiling.

### Offline step-by-step development

Write a one-page requirement for “AI wind-turbine monitoring camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera + vibration optional. Fix sensor mounting before collecting the main dataset.

Collect representative video/images at the final mounting height and field of view. Include occlusion, backlight, low light, reflections and difficult negatives.

Label the minimum classes/targets needed for Visual anomaly + condition model; do not add classes that the product does not use.

Train locally with a model sized for the selected MCU. For N6 targets, use compact detector/segmentation/pose models; for H7, simplify resolution/classes and use fixed zones where possible.

Quantize to INT8 and evaluate per-condition accuracy, not only global mAP/accuracy.

If range sensing is present, synchronize camera frames with ToF/radar/IMU timestamps and tune fusion logic offline from recorded sessions.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Damage/anomaly event).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Wind farms). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test each major environmental condition separately: day/night, backlight, occlusion, crowd density, dust/weather where applicable.

Measure missed-object/person rate and nuisance-alarm rate by scenario.

Measure end-to-end latency including camera exposure/capture, inference, fusion and output.

Force sensor disconnection/obstruction and verify a clear degraded/fault state.

Run a controlled pilot in advisory mode before enabling operational actions.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

# Category — Electrical maintenance

## 86. AI electrical-panel inspection camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Factories/data centers |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB/thermal camera |
| AI formulation | Hotspot/visual anomaly classifier |
| Product output | Panel anomaly |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Use thermal/RGB calibration and fixed viewing geometry where possible.

### Offline step-by-step development

Write a one-page requirement for “AI electrical-panel inspection camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB/thermal camera. Fix sensor mounting before collecting the main dataset.

Capture raw images from the real prototype under the full expected range of normal conditions. Keep originals immutable and store metadata (batch, lighting, product type, camera settings, date/site).

Define the smallest useful region of interest and input resolution. If the object position is fixed, crop it before inference instead of asking the network to search the whole frame.

Label/prepare data for Hotspot/visual anomaly classifier. Split train/validation/final-test sets by batch/site when possible.

Train locally on the workstation. Start with a compact CNN or keypoint/OCR pipeline; only move to a detector/segmenter if the simpler formulation cannot meet the requirement.

Quantize to INT8 and compare final-test metrics before and after quantization. Save the calibration image list.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Panel anomaly).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Factories/data centers). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Final-test images come from a different batch/day/site where possible.

Measure false accept and false reject separately; do not report accuracy alone.

Test lighting high/low limits, dirty lens, small position shifts and camera restart.

Measure trigger-to-output latency under worst-case processing load.

Run at least a multi-hour or multi-thousand-cycle soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

# Category — Utilities

## 87. AI meter-reading camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Utilities/factories |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB camera |
| AI formulation | Digit/needle recognition |
| Product output | Meter reading |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Constrain meter type and camera angle; use keypoints/ROI before OCR.

### Offline step-by-step development

Write a one-page requirement for “AI meter-reading camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera. Fix sensor mounting before collecting the main dataset.

Capture raw images from the real prototype under the full expected range of normal conditions. Keep originals immutable and store metadata (batch, lighting, product type, camera settings, date/site).

Define the smallest useful region of interest and input resolution. If the object position is fixed, crop it before inference instead of asking the network to search the whole frame.

Label/prepare data for Digit/needle recognition. Split train/validation/final-test sets by batch/site when possible.

Train locally on the workstation. Start with a compact CNN or keypoint/OCR pipeline; only move to a detector/segmenter if the simpler formulation cannot meet the requirement.

Quantize to INT8 and compare final-test metrics before and after quantization. Save the calibration image list.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Meter reading).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Utilities/factories). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Final-test images come from a different batch/day/site where possible.

Measure false accept and false reject separately; do not report accuracy alone.

Test lighting high/low limits, dirty lens, small position shifts and camera restart.

Measure trigger-to-output latency under worst-case processing load.

Run at least a multi-hour or multi-thousand-cycle soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

# Category — Industrial monitoring

## 88. AI pipe/gauge-reading camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Industrial facilities |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB camera |
| AI formulation | Gauge/valve state recognition |
| Product output | Reading/state |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Prefer sensor fusion and anomaly models where labeled faults are rare.

### Offline step-by-step development

Write a one-page requirement for “AI pipe/gauge-reading camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera. Fix sensor mounting before collecting the main dataset.

Record synchronized raw sensor streams across the full healthy operating envelope; log sample rate, sensor orientation, machine/load/state and temperature.

Segment data into fixed windows and evaluate both engineered features (RMS, kurtosis, FFT bands, temperature trends) and a small 1D CNN/MLP/anomaly model.

Prepare labels or normal/anomaly targets for Gauge/valve state recognition. Keep entire machines/sites out of the training set for a stronger final test where possible.

Train locally, quantize, and verify that the model is learning the intended physical condition rather than a shortcut such as RPM or background noise.

Reproduce exactly the same preprocessing in C on the MCU and compare window-by-window outputs against the offline Python reference.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Reading/state).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Industrial facilities). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test on machines/sessions not represented in training.

Sweep operating speed/load/temperature and verify the model does not confuse operating condition with failure.

Compare MCU preprocessing/inference with the offline Python reference on identical windows.

Test sensor disconnect, saturation and mounting changes.

Run a long-duration data-logging and inference soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

# Category — Industrial retrofit

## 89. AI analog-dial monitoring unit

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Legacy factories |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB camera |
| AI formulation | Needle/keypoint angle model |
| Product output | Numeric reading |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Fix camera relative to legacy dial and convert detected needle angle to engineering units deterministically.

### Offline step-by-step development

Write a one-page requirement for “AI analog-dial monitoring unit”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera. Fix sensor mounting before collecting the main dataset.

Capture raw images from the real prototype under the full expected range of normal conditions. Keep originals immutable and store metadata (batch, lighting, product type, camera settings, date/site).

Define the smallest useful region of interest and input resolution. If the object position is fixed, crop it before inference instead of asking the network to search the whole frame.

Label/prepare data for Needle/keypoint angle model. Split train/validation/final-test sets by batch/site when possible.

Train locally on the workstation. Start with a compact CNN or keypoint/OCR pipeline; only move to a detector/segmenter if the simpler formulation cannot meet the requirement.

Quantize to INT8 and compare final-test metrics before and after quantization. Save the calibration image list.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Numeric reading).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Legacy factories). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Final-test images come from a different batch/day/site where possible.

Measure false accept and false reject separately; do not report accuracy alone.

Test lighting high/low limits, dirty lens, small position shifts and camera restart.

Measure trigger-to-output latency under worst-case processing load.

Run at least a multi-hour or multi-thousand-cycle soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

# Category — Industrial process

## 90. AI tank/hopper level camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Food/chemical/agriculture |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | RGB/ToF |
| AI formulation | Level geometry/classifier |
| Product output | Fill level / blockage |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Use geometry/ToF first; AI handles irregular surfaces, foam or blockage.

### Offline step-by-step development

Write a one-page requirement for “AI tank/hopper level camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB/ToF. Fix sensor mounting before collecting the main dataset.

Capture raw images from the real prototype under the full expected range of normal conditions. Keep originals immutable and store metadata (batch, lighting, product type, camera settings, date/site).

Define the smallest useful region of interest and input resolution. If the object position is fixed, crop it before inference instead of asking the network to search the whole frame.

Label/prepare data for Level geometry/classifier. Split train/validation/final-test sets by batch/site when possible.

Train locally on the workstation. Start with a compact CNN or keypoint/OCR pipeline; only move to a detector/segmenter if the simpler formulation cannot meet the requirement.

Quantize to INT8 and compare final-test metrics before and after quantization. Save the calibration image list.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Fill level / blockage).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Food/chemical/agriculture). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Final-test images come from a different batch/day/site where possible.

Measure false accept and false reject separately; do not report accuracy alone.

Test lighting high/low limits, dirty lens, small position shifts and camera restart.

Measure trigger-to-output latency under worst-case processing load.

Run at least a multi-hour or multi-thousand-cycle soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

# Category — Construction

## 91. AI construction safety camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Construction companies |
| Cost-first MCU | STM32N6 — STM32N6 accelerated target |
| Sensors / inputs | RGB camera |
| AI formulation | Person/PPE/zone detector |
| Product output | Safety event |

Use STM32N657-class devices when the workload genuinely needs detection/segmentation/pose/depth/tracking or more complex variable scenes. Prototype on STM32N6570-DK. Re-evaluate H7 only if the problem can be constrained substantially.

Cost-control strategy: Collect outdoor light/weather/PPE variability; treat alerts as advisory unless safety-certified.

### Offline step-by-step development

Write a one-page requirement for “AI construction safety camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera. Fix sensor mounting before collecting the main dataset.

Collect representative video/images at the final mounting height and field of view. Include occlusion, backlight, low light, reflections and difficult negatives.

Label the minimum classes/targets needed for Person/PPE/zone detector; do not add classes that the product does not use.

Train locally with a model sized for the selected MCU. For N6 targets, use compact detector/segmentation/pose models; for H7, simplify resolution/classes and use fixed zones where possible.

Quantize to INT8 and evaluate per-condition accuracy, not only global mAP/accuracy.

If range sensing is present, synchronize camera frames with ToF/radar/IMU timestamps and tune fusion logic offline from recorded sessions.

Analyze/generate the model for STM32N6 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Safety event).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Construction companies). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test each major environmental condition separately: day/night, backlight, occlusion, crowd density, dust/weather where applicable.

Measure missed-object/person rate and nuisance-alarm rate by scenario.

Measure end-to-end latency including camera exposure/capture, inference, fusion and output.

Force sensor disconnection/obstruction and verify a clear degraded/fault state.

Run a controlled pilot in advisory mode before enabling operational actions.

### Cost-down / upgrade decision

N6 is selected because this product needs more complex variable-scene perception. Cost-down is possible only if the requirement can be constrained: fixed zones, fewer classes, lower resolution, reduced frame rate, external range sensor, or a simpler classifier. Re-test H7 only after that reformulation.

# Category — Construction/mining

## 92. AI hard-hat/vest detector

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Construction/mining |
| Cost-first MCU | STM32N6 — STM32N6 accelerated target |
| Sensors / inputs | RGB camera |
| AI formulation | PPE detector |
| Product output | PPE compliance |

Use STM32N657-class devices when the workload genuinely needs detection/segmentation/pose/depth/tracking or more complex variable scenes. Prototype on STM32N6570-DK. Re-evaluate H7 only if the problem can be constrained substantially.

Cost-control strategy: Collect PPE variation, dust and occlusion.

### Offline step-by-step development

Write a one-page requirement for “AI hard-hat/vest detector”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera. Fix sensor mounting before collecting the main dataset.

Collect representative video/images at the final mounting height and field of view. Include occlusion, backlight, low light, reflections and difficult negatives.

Label the minimum classes/targets needed for PPE detector; do not add classes that the product does not use.

Train locally with a model sized for the selected MCU. For N6 targets, use compact detector/segmentation/pose models; for H7, simplify resolution/classes and use fixed zones where possible.

Quantize to INT8 and evaluate per-condition accuracy, not only global mAP/accuracy.

If range sensing is present, synchronize camera frames with ToF/radar/IMU timestamps and tune fusion logic offline from recorded sessions.

Analyze/generate the model for STM32N6 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (PPE compliance).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Construction/mining). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test each major environmental condition separately: day/night, backlight, occlusion, crowd density, dust/weather where applicable.

Measure missed-object/person rate and nuisance-alarm rate by scenario.

Measure end-to-end latency including camera exposure/capture, inference, fusion and output.

Force sensor disconnection/obstruction and verify a clear degraded/fault state.

Run a controlled pilot in advisory mode before enabling operational actions.

### Cost-down / upgrade decision

N6 is selected because this product needs more complex variable-scene perception. Cost-down is possible only if the requirement can be constrained: fixed zones, fewer classes, lower resolution, reduced frame rate, external range sensor, or a simpler classifier. Re-test H7 only after that reformulation.

# Category — Construction/ports

## 93. AI crane-zone safety detector

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Construction/ports |
| Cost-first MCU | STM32N6 — STM32N6 accelerated target |
| Sensors / inputs | RGB + ToF/radar optional |
| AI formulation | Person/vehicle detection + zone logic |
| Product output | Crane-zone warning |

Use STM32N657-class devices when the workload genuinely needs detection/segmentation/pose/depth/tracking or more complex variable scenes. Prototype on STM32N6570-DK. Re-evaluate H7 only if the problem can be constrained substantially.

Cost-control strategy: Fuse camera semantics with range sensing for large moving equipment.

### Offline step-by-step development

Write a one-page requirement for “AI crane-zone safety detector”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB + ToF/radar optional. Fix sensor mounting before collecting the main dataset.

Collect representative video/images at the final mounting height and field of view. Include occlusion, backlight, low light, reflections and difficult negatives.

Label the minimum classes/targets needed for Person/vehicle detection + zone logic; do not add classes that the product does not use.

Train locally with a model sized for the selected MCU. For N6 targets, use compact detector/segmentation/pose models; for H7, simplify resolution/classes and use fixed zones where possible.

Quantize to INT8 and evaluate per-condition accuracy, not only global mAP/accuracy.

If range sensing is present, synchronize camera frames with ToF/radar/IMU timestamps and tune fusion logic offline from recorded sessions.

Analyze/generate the model for STM32N6 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Crane-zone warning).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Construction/ports). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test each major environmental condition separately: day/night, backlight, occlusion, crowd density, dust/weather where applicable.

Measure missed-object/person rate and nuisance-alarm rate by scenario.

Measure end-to-end latency including camera exposure/capture, inference, fusion and output.

Force sensor disconnection/obstruction and verify a clear degraded/fault state.

Run a controlled pilot in advisory mode before enabling operational actions.

### Cost-down / upgrade decision

N6 is selected because this product needs more complex variable-scene perception. Cost-down is possible only if the requirement can be constrained: fixed zones, fewer classes, lower resolution, reduced frame rate, external range sensor, or a simpler classifier. Re-test H7 only after that reformulation.

# Category — Mining

## 94. AI mining equipment proximity unit

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Mines |
| Cost-first MCU | STM32N6 — STM32N6 accelerated target |
| Sensors / inputs | RGB + ToF/radar + IMU |
| AI formulation | Person/vehicle detection + distance fusion |
| Product output | Proximity risk |

Use STM32N657-class devices when the workload genuinely needs detection/segmentation/pose/depth/tracking or more complex variable scenes. Prototype on STM32N6570-DK. Re-evaluate H7 only if the problem can be constrained substantially.

Cost-control strategy: Use rugged sensors and range fusion; include dust/low-light data.

### Offline step-by-step development

Write a one-page requirement for “AI mining equipment proximity unit”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB + ToF/radar + IMU. Fix sensor mounting before collecting the main dataset.

Collect representative video/images at the final mounting height and field of view. Include occlusion, backlight, low light, reflections and difficult negatives.

Label the minimum classes/targets needed for Person/vehicle detection + distance fusion; do not add classes that the product does not use.

Train locally with a model sized for the selected MCU. For N6 targets, use compact detector/segmentation/pose models; for H7, simplify resolution/classes and use fixed zones where possible.

Quantize to INT8 and evaluate per-condition accuracy, not only global mAP/accuracy.

If range sensing is present, synchronize camera frames with ToF/radar/IMU timestamps and tune fusion logic offline from recorded sessions.

Analyze/generate the model for STM32N6 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Proximity risk).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Mines). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test each major environmental condition separately: day/night, backlight, occlusion, crowd density, dust/weather where applicable.

Measure missed-object/person rate and nuisance-alarm rate by scenario.

Measure end-to-end latency including camera exposure/capture, inference, fusion and output.

Force sensor disconnection/obstruction and verify a clear degraded/fault state.

Run a controlled pilot in advisory mode before enabling operational actions.

### Cost-down / upgrade decision

N6 is selected because this product needs more complex variable-scene perception. Cost-down is possible only if the requirement can be constrained: fixed zones, fewer classes, lower resolution, reduced frame rate, external range sensor, or a simpler classifier. Re-test H7 only after that reformulation.

# Category — Ports

## 95. AI port container inspection camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Ports/shipping |
| Cost-first MCU | STM32N6 — STM32N6 accelerated target |
| Sensors / inputs | RGB camera |
| AI formulation | Container ID/damage/anomaly detector |
| Product output | Container status |

Use STM32N657-class devices when the workload genuinely needs detection/segmentation/pose/depth/tracking or more complex variable scenes. Prototype on STM32N6570-DK. Re-evaluate H7 only if the problem can be constrained substantially.

Cost-control strategy: Collect containers in varied weather/lighting; separate ID/OCR from damage detection.

### Offline step-by-step development

Write a one-page requirement for “AI port container inspection camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera. Fix sensor mounting before collecting the main dataset.

Collect representative video/images at the final mounting height and field of view. Include occlusion, backlight, low light, reflections and difficult negatives.

Label the minimum classes/targets needed for Container ID/damage/anomaly detector; do not add classes that the product does not use.

Train locally with a model sized for the selected MCU. For N6 targets, use compact detector/segmentation/pose models; for H7, simplify resolution/classes and use fixed zones where possible.

Quantize to INT8 and evaluate per-condition accuracy, not only global mAP/accuracy.

If range sensing is present, synchronize camera frames with ToF/radar/IMU timestamps and tune fusion logic offline from recorded sessions.

Analyze/generate the model for STM32N6 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Container status).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Ports/shipping). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test each major environmental condition separately: day/night, backlight, occlusion, crowd density, dust/weather where applicable.

Measure missed-object/person rate and nuisance-alarm rate by scenario.

Measure end-to-end latency including camera exposure/capture, inference, fusion and output.

Force sensor disconnection/obstruction and verify a clear degraded/fault state.

Run a controlled pilot in advisory mode before enabling operational actions.

### Cost-down / upgrade decision

N6 is selected because this product needs more complex variable-scene perception. Cost-down is possible only if the requirement can be constrained: fixed zones, fewer classes, lower resolution, reduced frame rate, external range sensor, or a simpler classifier. Re-test H7 only after that reformulation.

# Category — Rail

## 96. AI railway track/equipment inspection unit

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Rail operators |
| Cost-first MCU | STM32N6 — STM32N6 accelerated target |
| Sensors / inputs | RGB camera + IMU |
| AI formulation | Defect/anomaly detection |
| Product output | Track/equipment anomaly |

Use STM32N657-class devices when the workload genuinely needs detection/segmentation/pose/depth/tracking or more complex variable scenes. Prototype on STM32N6570-DK. Re-evaluate H7 only if the problem can be constrained substantially.

Cost-control strategy: Use synchronized vehicle motion/IMU and record track conditions over long distances.

### Offline step-by-step development

Write a one-page requirement for “AI railway track/equipment inspection unit”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera + IMU. Fix sensor mounting before collecting the main dataset.

Collect representative video/images at the final mounting height and field of view. Include occlusion, backlight, low light, reflections and difficult negatives.

Label the minimum classes/targets needed for Defect/anomaly detection; do not add classes that the product does not use.

Train locally with a model sized for the selected MCU. For N6 targets, use compact detector/segmentation/pose models; for H7, simplify resolution/classes and use fixed zones where possible.

Quantize to INT8 and evaluate per-condition accuracy, not only global mAP/accuracy.

If range sensing is present, synchronize camera frames with ToF/radar/IMU timestamps and tune fusion logic offline from recorded sessions.

Analyze/generate the model for STM32N6 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Track/equipment anomaly).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Rail operators). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test each major environmental condition separately: day/night, backlight, occlusion, crowd density, dust/weather where applicable.

Measure missed-object/person rate and nuisance-alarm rate by scenario.

Measure end-to-end latency including camera exposure/capture, inference, fusion and output.

Force sensor disconnection/obstruction and verify a clear degraded/fault state.

Run a controlled pilot in advisory mode before enabling operational actions.

### Cost-down / upgrade decision

N6 is selected because this product needs more complex variable-scene perception. Cost-down is possible only if the requirement can be constrained: fixed zones, fewer classes, lower resolution, reduced frame rate, external range sensor, or a simpler classifier. Re-test H7 only after that reformulation.

# Category — Fleet safety

## 97. AI bus/truck blind-spot camera

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Commercial fleets |
| Cost-first MCU | STM32N6 — STM32N6 accelerated target |
| Sensors / inputs | RGB + ToF/radar |
| AI formulation | Person/cyclist/vehicle detector |
| Product output | Blind-spot warning |

Use STM32N657-class devices when the workload genuinely needs detection/segmentation/pose/depth/tracking or more complex variable scenes. Prototype on STM32N6570-DK. Re-evaluate H7 only if the problem can be constrained substantially.

Cost-control strategy: Use RGB/IR + range/vehicle signals; validate latency across vibration/day/night.

### Offline step-by-step development

Write a one-page requirement for “AI bus/truck blind-spot camera”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB + ToF/radar. Fix sensor mounting before collecting the main dataset.

Collect representative video/images at the final mounting height and field of view. Include occlusion, backlight, low light, reflections and difficult negatives.

Label the minimum classes/targets needed for Person/cyclist/vehicle detector; do not add classes that the product does not use.

Train locally with a model sized for the selected MCU. For N6 targets, use compact detector/segmentation/pose models; for H7, simplify resolution/classes and use fixed zones where possible.

Quantize to INT8 and evaluate per-condition accuracy, not only global mAP/accuracy.

If range sensing is present, synchronize camera frames with ToF/radar/IMU timestamps and tune fusion logic offline from recorded sessions.

Analyze/generate the model for STM32N6 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Blind-spot warning).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Commercial fleets). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test each major environmental condition separately: day/night, backlight, occlusion, crowd density, dust/weather where applicable.

Measure missed-object/person rate and nuisance-alarm rate by scenario.

Measure end-to-end latency including camera exposure/capture, inference, fusion and output.

Force sensor disconnection/obstruction and verify a clear degraded/fault state.

Run a controlled pilot in advisory mode before enabling operational actions.

### Cost-down / upgrade decision

N6 is selected because this product needs more complex variable-scene perception. Cost-down is possible only if the requirement can be constrained: fixed zones, fewer classes, lower resolution, reduced frame rate, external range sensor, or a simpler classifier. Re-test H7 only after that reformulation.

## 98. AI fleet driver-monitoring unit

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Truck/bus/taxi companies |
| Cost-first MCU | STM32N6 — STM32N6 accelerated target |
| Sensors / inputs | Cabin RGB/IR camera |
| AI formulation | Face/eye/head/phone detector |
| Product output | Drowsy/distracted state |

Use STM32N657-class devices when the workload genuinely needs detection/segmentation/pose/depth/tracking or more complex variable scenes. Prototype on STM32N6570-DK. Re-evaluate H7 only if the problem can be constrained substantially.

Cost-control strategy: Use RGB/IR + range/vehicle signals; validate latency across vibration/day/night.

### Offline step-by-step development

Write a one-page requirement for “AI fleet driver-monitoring unit”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using Cabin RGB/IR camera. Fix sensor mounting before collecting the main dataset.

Collect representative video/images at the final mounting height and field of view. Include occlusion, backlight, low light, reflections and difficult negatives.

Label the minimum classes/targets needed for Face/eye/head/phone detector; do not add classes that the product does not use.

Train locally with a model sized for the selected MCU. For N6 targets, use compact detector/segmentation/pose models; for H7, simplify resolution/classes and use fixed zones where possible.

Quantize to INT8 and evaluate per-condition accuracy, not only global mAP/accuracy.

If range sensing is present, synchronize camera frames with ToF/radar/IMU timestamps and tune fusion logic offline from recorded sessions.

Analyze/generate the model for STM32N6 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Drowsy/distracted state).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Truck/bus/taxi companies). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test each major environmental condition separately: day/night, backlight, occlusion, crowd density, dust/weather where applicable.

Measure missed-object/person rate and nuisance-alarm rate by scenario.

Measure end-to-end latency including camera exposure/capture, inference, fusion and output.

Force sensor disconnection/obstruction and verify a clear degraded/fault state.

Run a controlled pilot in advisory mode before enabling operational actions.

### Cost-down / upgrade decision

N6 is selected because this product needs more complex variable-scene perception. Cost-down is possible only if the requirement can be constrained: fixed zones, fewer classes, lower resolution, reduced frame rate, external range sensor, or a simpler classifier. Re-test H7 only after that reformulation.

# Category — Logistics fleets

## 99. AI commercial vehicle cargo monitor

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Logistics fleets |
| Cost-first MCU | STM32H7 — STM32H7 cost-first target |
| Sensors / inputs | Cargo-area RGB camera + door sensor |
| AI formulation | Cargo presence/change detection |
| Product output | Cargo event/state |

Use H723/H743/H747/H750-class parts depending memory/peripherals. Best for fixed-camera inference, DSP, simple OCR and controlled vision. Prototype camera work with STM32H747I-DISCO + B-CAMS-OMV.

Cost-control strategy: Use door sensor + fixed cargo camera to detect loading/unloading events.

### Offline step-by-step development

Write a one-page requirement for “AI commercial vehicle cargo monitor”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using Cargo-area RGB camera + door sensor. Fix sensor mounting before collecting the main dataset.

Capture raw images from the real prototype under the full expected range of normal conditions. Keep originals immutable and store metadata (batch, lighting, product type, camera settings, date/site).

Define the smallest useful region of interest and input resolution. If the object position is fixed, crop it before inference instead of asking the network to search the whole frame.

Label/prepare data for Cargo presence/change detection. Split train/validation/final-test sets by batch/site when possible.

Train locally on the workstation. Start with a compact CNN or keypoint/OCR pipeline; only move to a detector/segmenter if the simpler formulation cannot meet the requirement.

Quantize to INT8 and compare final-test metrics before and after quantization. Save the calibration image list.

Analyze/generate the model for STM32H7 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Cargo event/state).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Logistics fleets). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Final-test images come from a different batch/day/site where possible.

Measure false accept and false reject separately; do not report accuracy alone.

Test lighting high/low limits, dirty lens, small position shifts and camera restart.

Measure trigger-to-output latency under worst-case processing load.

Run at least a multi-hour or multi-thousand-cycle soak test.

### Cost-down / upgrade decision

Keep H7 if the product can use fixed geometry, a small ROI and modest inference rate. Upgrade to STM32N6 when field requirements demand full-scene detection/segmentation/tracking, significantly larger models or higher frame rate. Do not upgrade just because a larger model is easier to train on a PC.

# Category — Industrial operations

## 100. AI machine-operator monitoring unit

| Item | Design choice |
| --- | --- |
| Primary buyers/installations | Industrial equipment manufacturers |
| Cost-first MCU | STM32N6 — STM32N6 accelerated target |
| Sensors / inputs | RGB camera + optional ToF |
| AI formulation | Operator pose/action/zone model |
| Product output | Operator state/safety event |

Use STM32N657-class devices when the workload genuinely needs detection/segmentation/pose/depth/tracking or more complex variable scenes. Prototype on STM32N6570-DK. Re-evaluate H7 only if the problem can be constrained substantially.

Cost-control strategy: Model operator state/pose in defined machine zones; avoid identity unless required.

### Offline step-by-step development

Write a one-page requirement for “AI machine-operator monitoring unit”: installation geometry, input conditions, exact output, maximum response time, acceptable false alarm/miss rate, and offline interfaces.

Build the bench prototype using RGB camera + optional ToF. Fix sensor mounting before collecting the main dataset.

Collect representative video/images at the final mounting height and field of view. Include occlusion, backlight, low light, reflections and difficult negatives.

Label the minimum classes/targets needed for Operator pose/action/zone model; do not add classes that the product does not use.

Train locally with a model sized for the selected MCU. For N6 targets, use compact detector/segmentation/pose models; for H7, simplify resolution/classes and use fixed zones where possible.

Quantize to INT8 and evaluate per-condition accuracy, not only global mAP/accuracy.

If range sensing is present, synchronize camera frames with ToF/radar/IMU timestamps and tune fusion logic offline from recorded sessions.

Analyze/generate the model for STM32N6 using STM32Cube AI Studio / ST Edge AI Core on the offline workstation. Record RAM, Flash/external-memory use, unsupported operators and measured target latency.

Integrate firmware: sensor acquisition, preprocessing, inference, output decoding, thresholds/hysteresis, self-test, watchdog, configuration storage, diagnostics and the product output (Operator state/safety event).

Run a bench soak test and compare MCU decisions with the offline reference on recorded test vectors. Fix any preprocessing mismatch before field testing.

Run a controlled field/pilot test with the intended buyers/installations (Industrial equipment manufacturers). Log false positives, false negatives, sensor faults and timing.

Freeze a release bundle: firmware binary/ELF, model, calibration/configuration, source commit, tool versions, hashes, test report and known limitations. Then design the cost-reduced custom PCB around the measured memory/peripheral requirements.

### Validation before a pilot sale

Test each major environmental condition separately: day/night, backlight, occlusion, crowd density, dust/weather where applicable.

Measure missed-object/person rate and nuisance-alarm rate by scenario.

Measure end-to-end latency including camera exposure/capture, inference, fusion and output.

Force sensor disconnection/obstruction and verify a clear degraded/fault state.

Run a controlled pilot in advisory mode before enabling operational actions.

### Cost-down / upgrade decision

N6 is selected because this product needs more complex variable-scene perception. Cost-down is possible only if the requirement can be constrained: fixed zones, fewer classes, lower resolution, reduced frame rate, external range sensor, or a simpler classifier. Re-test H7 only after that reformulation.

# 3. Offline Production Programming and End-of-Line Test

Expose SWD test pads/header on every custom PCB. Keep boot/reset access and a serial/service interface.

Build approved binaries on a controlled offline engineering PC; never build production firmware directly on the factory station.

Copy only the locked release package to the offline factory PC.

Use STM32CubeProgrammer CLI + ST-LINK in a pogo fixture for erase/program/verify and approved device configuration.

Program unique serial/calibration data using a documented format. Security fuses/credentials require a separate provisioning design and access control.

Boot the unit and execute automated sensor self-tests and a known AI inference test vector.

Record serial number, PCB revision, firmware/model versions, calibration and test result.

Quarantine failures; do not rework by manually changing thresholds without an engineering change record.

# 4. Common Model Recipes

| Problem | Start with | Avoid at first |
| --- | --- | --- |
| Fixed visual PASS/FAIL | Small CNN / MobileNet-style classifier | YOLO on a full frame |
| Rare unknown visual defect | Anomaly model + deterministic ROI | Dozens of synthetic defect classes |
| Vibration/audio anomaly | Feature MLP or 1D CNN | Large spectrogram model before proving features |
| Known gauge/dial | Keypoint/needle angle + deterministic conversion | General-purpose OCR/vision model |
| People/vehicles in variable scene | Compact detector on N6 | Trying to force a large detector onto H7 |
| Depth/proximity | ToF/radar + simple semantic AI fusion | Monocular depth if a cheap range sensor solves it |

## Official sources to archive for offline work

STM32H7 series: https://www.st.com/en/microcontrollers-microprocessors/stm32h7-series.html

STM32U5 series: https://www.st.com/en/microcontrollers-microprocessors/stm32u5-series.html

STM32N6 series: https://www.st.com/en/microcontrollers-microprocessors/stm32n6-series.html

STM32Cube AI Studio documentation: https://wiki.st.com/stm32mcu/wiki/AI:STM32Cube_AI_Studio_documentation

ST Edge AI Core getting started: https://dev.st.com/edge-ai-docs/general/latest/en/docs/tutorials/st-edge-ai-core-getting-started.html

STM32 camera/DCMI application note AN5020: https://www.st.com/content/st_com/en/technical-documents/an5020-introduction-to-digital-camera-interface-dcmi-for-stm32-mcus-stmicroelectronics.html

STM32H747I-DISCO: https://www.st.com/en/evaluation-tools/stm32h747i-disco.html

B-CAMS-OMV camera bundle: https://www.st.com/en/evaluation-tools/b-cams-omv.html

B-U585I-IOT02A STM32U5 IoT Discovery kit: https://www.st.com/en/evaluation-tools/b-u585i-iot02a.html

End of handbook. Use the product sections as engineering starting points; every final design must be validated against the exact MCU datasheet, board documentation, current errata, environmental requirements, regulatory requirements, and measured field performance.