STM32N6 Offline Industrial AI
Development Guide

Three mass-product project paths for factories and business environments

Selected MCU platform: STM32N6 / STM32N657
Development target: STM32N6570-DK
Development mode: offline-first after initial software/package download
Prepared: 17 October 2026

Projects

1. AI Visual Quality Inspection Station
2. AI Machine Health / Predictive Maintenance Node
3. AI Forklift & Warehouse Proximity Warning Unit

# Executive Summary

This guide is designed for a team that wants to build practical STM32N6-based industrial AI products but currently has no embedded programmer/writer and little or no embedded lab equipment. It deliberately avoids cloud dependence after the initial download/setup stage.

Important starting point: For the prototype stage, buy the STM32N6570-DK. It already contains an on-board STLINK-V3EC debugger/programmer, so you do not need a separate writer to flash or debug the board. A separate ST-LINK probe becomes useful when you move to your own custom PCB.

## The three selected projects

| Project | Primary sensing | AI problem | Why it is commercially different |
| --- | --- | --- | --- |
| 1. AI Visual Quality Inspection Station | RGB camera + controlled illumination + product trigger | classification / object detection / segmentation / visual anomaly detection | Improves outgoing product quality and reduces manual inspection. |
| 2. AI Machine Health Node | vibration + microphone + temperature | audio/vibration classification, anomaly detection, sensor fusion | Monitors the machine itself and targets downtime/maintenance cost. |
| 3. AI Forklift & Warehouse Warning Unit | RGB camera + multizone ToF + optional IMU | person/vehicle detection + tracking + distance/risk fusion | Targets logistics and workplace collision awareness instead of product quality or maintenance. |

## Recommended order

Build Project 1 first, Project 2 second, and Project 3 third. Project 1 teaches the STM32N6 camera/NPU pipeline. Project 2 teaches high-rate sensor acquisition and non-vision AI. Project 3 combines vision and sensor fusion and is therefore the most system-complex.

## What “offline mode” means in this guide

Internet is used once to download installers, firmware packages, documentation, model-zoo source, pretrained starting models, Python packages, and optional container images.

After that preparation, coding, compilation, model training, quantization, C-code generation, flashing, debugging, logging, and testing can be done on the local workstation and boards without Internet access.

Cloud benchmarking and developer-cloud services are not required; ST Edge AI Core can be installed locally and used on the workstation.

Your datasets, labels, model checkpoints, firmware, and production binaries remain on your local storage.

# 1. Minimum Hardware When You Currently Have No Embedded Tools

## 1.1 Buy these first

| Item | Qty | Purpose | Priority |
| --- | --- | --- | --- |
| STM32N6570-DK Discovery kit | 2 | Primary development platform; second board is a spare/reference unit. Includes camera, external flash/PSRAM, microphone, display, Ethernet and on-board STLINK-V3EC. | Essential |
| USB cables appropriate for board/debug ports | 2-3 | Power, ST-LINK, virtual COM and data connection. | Essential |
| Digital multimeter | 1 | Voltage, continuity, power and wiring checks. | Essential |
| Breadboard / terminal breakout / jumper wires | 1 set | Fast sensor and I/O experiments. | Essential |
| Adjustable bench power supply with current limit | 1 | Safer testing of custom I/O boards and later custom PCBs. | Strongly recommended |
| 2- or 4-channel oscilloscope | 1 | Clock, SPI/I2C, sensor, power and signal-integrity debugging. | Strongly recommended |
| USB logic analyzer | 1 | SPI/I2C/UART debugging. | Recommended |
| Soldering iron + hot air + ESD mat | 1 set | Sensor breakout, connectors, prototype PCB rework. | Recommended |
| STLINK-V3SET or STLINK-V3MINIE | 1 | External SWD/JTAG programmer/debugger for your future custom PCB. Not needed for the DK prototype. | Buy before custom PCB |

## 1.2 Why the STM32N6570-DK is the correct starting point

It contains the STM32N657X0 with the Neural-ART accelerator.

It includes an IMX335 camera module, 1-Gbit Octo-SPI flash and 256-Mbit Hexadeca-SPI PSRAM, so you can prototype real vision workloads without first designing high-speed memory hardware.

It has an on-board STLINK-V3EC debugger/programmer and Virtual COM port. This removes the immediate need for an external flash writer.

It includes a digital microphone for the machine-health/audio project.

It has Ethernet, USB, microSD, Arduino/STMod+ expansion and a display, which are useful for offline data capture and diagnostics.

## 1.3 Extra sensors for the three projects

| Sensor / module | Project | Why |
| --- | --- | --- |
| Controlled LED illumination + diffuser | Project 1 | Stable lighting is often more important than the neural-network choice in industrial visual inspection. |
| Photoelectric / proximity trigger sensor | Project 1 | Triggers capture only when a part is in a known inspection position. |
| IIS3DWB vibration sensor or evaluation breakout | Project 2 | Industrial 3-axis digital vibration sensing with flat response to 6 kHz and high output data rate. |
| STTS22H temperature sensor or evaluation probe | Project 2 | Adds temperature context to vibration/audio anomalies. |
| Digital microphone | Project 2 | The DK already has one; a final product may use a mechanically isolated industrial microphone. |
| VL53L8CX ToF board/module | Project 3 | 8x8 depth zones, multitarget ranging, up to about 4 m for near-field distance confirmation. |
| Optional IMU | Project 3 | Provides forklift/device motion and turn/acceleration context. |

# 2. Build an Offline-First Development Workstation

## 2.1 Recommended PC

| Component | Recommended | Minimum practical |
| --- | --- | --- |
| OS | Windows 11 64-bit | Windows 10/11 64-bit or Linux for parts of the toolchain |
| CPU | 8+ modern cores | 4 cores |
| RAM | 32 GB | 16 GB |
| Storage | 1 TB NVMe with at least 300 GB reserved for SDKs/datasets | 500 GB SSD |
| GPU | NVIDIA GPU with 8-16 GB VRAM for local training | CPU training works for small models but is much slower |
| USB | Several direct USB ports | Powered hub if necessary |

## 2.2 Download once before going offline

STM32CubeIDE - install the latest version that supports STM32N6.

STM32CubeMX - useful for peripheral configuration and generated initialization code.

STM32CubeProgrammer - GUI and CLI tool for ST-LINK/bootloader programming, external flash access and production automation.

STM32CubeN6 MCU package - board support, HAL/LL drivers and example projects.

ST Edge AI Core - local CLI used to analyze a trained model, generate optimized embedded code and validate it on STM32N6.

A local snapshot/ZIP or Git clone of STMicroelectronics/stm32ai-modelzoo-services and any example/model files you plan to use.

Python environment installers plus a local wheelhouse of all Python packages needed by the model-zoo/training workflow.

Local annotation software. Good offline choices include CVAT deployed locally with Docker or Label Studio installed locally.

Datasets and any pretrained model weights you will use as starting points. Download them before disconnecting the workstation.

Official PDF documentation: STM32N6 reference manual RM0486, STM32N657 datasheet, STM32N6570-DK user/data brief, AN6265, AN5967, AN6000, STM32CubeProgrammer user manual, ST Edge AI documentation, and the latest STM32N6 errata.

## 2.3 Make the Python environment reproducible offline

Do this while Internet is still available. The exact Python version and package versions should follow the model-zoo release you download.

Example preparation concept (run while online):

python -m venv C:\stm32n6\venv
C:\stm32n6\venv\Scripts\activate
pip install -r requirements.txt
pip freeze > C:\stm32n6\offline_bundle\requirements-lock.txt
pip download -r requirements.txt -d C:\stm32n6\offline_bundle\wheelhouse

Later on an offline PC, install from the wheelhouse instead of PyPI:

pip install --no-index --find-links C:\stm32n6\offline_bundle\wheelhouse -r requirements-lock.txt

Version discipline: Do not randomly upgrade Python, STM32Cube, ST Edge AI Core or model-zoo packages in the middle of a product milestone. Freeze a known-working toolchain and archive its installers.

## 2.4 Suggested local directory

D:\IndustrialAI\
  tools\
  docs\
  stm32cube_n6\
  edge_ai_core\
  model_zoo\
  datasets\
    project1_quality\
    project2_machine_health\
    project3_forklift\
  models\
  firmware\
  releases\
  production_programming\
  backups\

# 3. First STM32N6 Bring-Up - No External Writer Required

## 3.1 Connection path

PC
  | USB
  v
On-board STLINK-V3EC
  | SWD/JTAG internally on DK
  v
STM32N657
  |
  +--> external Octo-SPI flash
  +--> external PSRAM
  +--> camera / microphone / display

## 3.2 First-day validation sequence

Install STM32CubeIDE and STM32CubeProgrammer including ST-LINK drivers.

Connect only the Discovery kit. Do not add external sensors yet.

Open STM32CubeProgrammer and confirm ST-LINK connection and device detection.

Update ST-LINK firmware if the tools request it while you still have Internet/download access.

Import a simple STM32CubeN6 board example. Build it with no changes.

Program the example using the DK on-board ST-LINK and verify that the board starts correctly.

Open the Virtual COM port and verify UART logging. From this point forward, keep a serial log in every project.

Run an official camera example and confirm stable capture/display before adding AI.

Run an official STM32N6 AI/model-zoo deployment example before training your own model.

## 3.3 STM32N6 flash/boot implication

STM32N6 systems commonly use external flash for boot/application/model storage. The Discovery kit already includes external Octo-SPI flash. STM32CubeProgrammer supports STM32N6 external-flash programming through ST-LINK using an external loader, or through BootROM using USB/UART and a partition/TSV workflow. Start with the ST-provided board loaders and examples; do not design your own external-flash loader until you move to a custom PCB.

## 3.4 When you eventually need an external programmer

For a custom PCB, expose a proper SWD/STDC14-style debug connector or pogo-pad footprint. A standalone STLINK-V3SET or STLINK-V3MINIE can program/debug the STM32 target through SWD. During production you can later automate STM32CubeProgrammer CLI from an offline manufacturing PC.

# 4. Common Offline AI-to-STM32N6 Workflow

REAL DATA
  -> local dataset
  -> local annotation
  -> local training/fine-tuning
  -> evaluation
  -> quantization (usually INT8)
  -> ST Edge AI Core analysis
  -> optimized embedded network/code
  -> STM32CubeIDE firmware build
  -> on-board ST-LINK programming
  -> target validation
  -> production release

## 4.1 Dataset discipline

Create separate train, validation and final test sets. Never tune using the final test set.

Capture data under the real lighting, vibration, noise, temperature, motion and mounting conditions expected in the customer site.

Keep a dataset manifest: source machine/line, date, operating condition, labeler, sensor serial, firmware version and environment.

Store raw data separately from processed/augmented data so that preprocessing can be reproduced.

Maintain a “difficult cases” folder for false positives and false negatives found during field testing.

## 4.2 Model selection rule

| Problem | Good first model family | Why |
| --- | --- | --- |
| Visual pass/fail classification | MobileNet/EfficientNet-class compact CNN | Simple, efficient baseline if a single centered part fills most of the image. |
| Object detection | Small YOLO / SSD family supported by STM32N6 model zoo | Provides class + bounding box for missing/wrong parts or persons. |
| Pixel-level defect/zone understanding | DeepLab-style semantic segmentation or supported instance segmentation | Useful when exact defect/region shape matters. |
| Unknown defect / machine anomaly | Compact autoencoder / one-class or anomaly scoring pipeline | Useful when fault classes are incomplete. |
| Audio event / machine sound | YAMNet-like or compact spectrogram CNN | Converts local sound windows into event/fault probabilities. |
| Vibration time series | 1D CNN or spectrogram CNN | Captures frequency/time patterns in bearings, motors and pumps. |

## 4.3 Quantization and STM32 deployment

For STM32N6, a common deployment target is a quantized model (for example TFLite INT8 or ONNX QDQ, depending on the service/model). ST Edge AI Core can be installed locally; it can analyze compatibility/memory, generate optimized C code and validate on the STM32N6570-DK. The STM32 model-zoo includes STM32N6570-DK deployment paths for object detection, semantic/instance segmentation, depth estimation, face detection and audio-event detection.

## 4.4 Offline release package for every model

Raw model/checkpoint and exported ONNX/TFLite model.

Quantized model plus calibration dataset list.

Class names and preprocessing parameters.

Exact training configuration and random seed.

Validation metrics before and after quantization.

ST Edge AI analysis report: RAM, external memory, operators, warnings.

Firmware commit ID and compiler/tool versions.

Known limitations and test cases.

# 5. Project 1 - AI Visual Quality Inspection Station

Goal: a compact product mounted over or beside a production station that makes a local PASS/FAIL decision and optionally identifies the defect type. This is the best first STM32N6 project because it directly uses the board camera pipeline and NPU.

## 5.1 Typical buyers and use cases

Electronics: missing connector, wrong component, reversed part, missing screw.

Packaging: missing cap, damaged seal, wrong label, wrong orientation.

Machining/plastics: scratch, crack, deformation, incomplete molding.

Food: surface defect, incorrect shape/color, foreign item in an inspection zone.

## 5.2 Prototype architecture

Product arrives
     |
Photoelectric trigger
     |
RGB camera + fixed LED illumination
     |
STM32N657 / Neural-ART
     |
AI result: PASS / FAIL / defect type
     |
+--> local display/log
+--> PLC digital output / Ethernet message
+--> reject actuator command (through proper industrial interface)

## 5.3 Prototype hardware

| Block | Prototype choice | Production direction |
| --- | --- | --- |
| Compute | STM32N6570-DK | Custom STM32N657 board after performance is proven |
| Camera | DK IMX335 module | Industrial lens/sensor selected for field of view and working distance |
| Lighting | External diffuse LED ring/bar | Enclosed fixed-current LED illumination |
| Trigger | Photoelectric sensor / simple proximity switch | 24 V industrial sensor through isolated/conditioned input |
| Output | DK GPIO + safe lab interface | 24 V digital output, Ethernet/RS-485/fieldbus gateway as required |
| Storage | microSD / PC serial logs | External flash + optional SD/service port |

## 5.4 Offline development steps

Mechanical fixture first. Fix camera distance, angle, lens and product position. Avoid handheld data collection for the final dataset.

Build stable lighting. Lock exposure/white-balance behavior where possible. Capture “good” products across realistic material and lot variation.

Collect defective examples. Start with 500-2,000 images per important class if feasible, but quality/variation matters more than a magic number.

Label locally. For simple pass/fail, folder-level classification labels may be enough. For missing/wrong components use bounding boxes. For scratches/regions use segmentation masks.

Train locally on the workstation. Start with the smallest model that reaches the required accuracy; do not start with the largest model.

Quantize locally. Re-check accuracy after quantization and inspect failure cases, especially tiny defects and dark/reflective surfaces.

Use ST Edge AI Core locally to analyze compatibility and memory, then generate/integrate the network into the STM32N6 application.

Run inference from a static image first, then live camera frames, then hardware-triggered frames.

Add post-processing: confidence thresholds, region-of-interest rules, debounce across 2-3 frames if the line is moving, and PASS/FAIL logic.

Add offline logging of image ID, inference result, confidence, firmware version and operator disposition so false decisions can be retrained later.

## 5.5 Firmware tasks

Camera initialization and frame buffer management.

DCMIPP/ISP preprocessing: crop/resize/color conversion consistent with training.

Neural-ART inference scheduling and model memory placement.

Post-processing and deterministic PASS/FAIL state machine.

Trigger input and output timing.

Local health checks: camera disconnected, overtemperature, flash error, low supply.

Watchdog and fail-safe output state.

UART/USB/Ethernet service diagnostics kept available even in offline customer networks.

## 5.6 Validation targets

| Metric | Prototype target | Production consideration |
| --- | --- | --- |
| False reject rate | Measure separately by product SKU/lot | Too many false rejects destroy operator trust. |
| False accept rate | Define very low limit for critical defects | Usually the most important quality metric. |
| Inference + decision latency | Comfortably below takt-time budget | Include camera exposure and I/O latency, not only NPU time. |
| Lighting robustness | Test shift, dust, aging and reflections | Industrial enclosure/illumination design is part of the ML system. |
| Recovery | Automatic after reset/power dip | Watchdog and known-safe output state required. |

## 5.7 Mass-production transition

Freeze the optical stack before shrinking the electronics.

Design the custom PCB only after the model meets latency/memory targets on the DK.

Keep external boot flash and, if needed, external RAM sized with margin for model updates.

Provide a production programming/debug header or pogo pads.

Create a golden-image programming package containing bootloader, application, model and factory configuration.

Add serial-number/lot tracking so production failures can be traced to PCB and firmware batches.

# 6. Project 2 - AI Machine Health / Predictive Maintenance Node

Goal: a small node attached to a motor, pump, fan, compressor, gearbox or similar machine. It learns healthy behavior and detects developing anomalies from vibration, sound and temperature. This is deliberately different from Project 1 because the main AI input is time-series/acoustic data rather than camera images.

## 6.1 Prototype architecture

IIS3DWB vibration sensor ----
                            \nDigital microphone ----------> STM32N657 -> feature extraction / AI -> health score / fault class
                            /
STTS22H temperature --------
                              |
                              +--> local log / LED / Ethernet / RS-485 gateway

## 6.2 Why these sensors

IIS3DWB is designed for industrial vibration monitoring, with a flat bandwidth up to 6 kHz and output data rate up to 26.7 kHz. This is appropriate for bearings, motors and gearboxes.

A microphone adds airborne acoustic information that may reveal rubbing, cavitation, squeal or impact patterns that are weak in the mounted accelerometer.

Temperature provides operating context and helps separate “hot but normal” from a combined heat/vibration abnormal condition.

## 6.3 Data-collection plan

Choose one machine class first, e.g., a small induction motor + bearing test rig. Do not train one model on every type of machine at the beginning.

Define operating regimes: off, idle/no-load, normal load, high load, acceleration/deceleration, and known harmless transients.

Record synchronized windows of vibration, microphone and temperature. Store raw data with machine RPM/load if available.

Create controlled abnormal conditions on a safe test bench: imbalance, looseness, misalignment, worn bearing or simulated impact. Never intentionally damage an operating customer machine.

Segment signals into fixed windows (for example 0.5-2 s depending on the machine and model).

For vibration/audio, evaluate both raw/1D models and spectrogram-based CNNs. Spectrograms are easier to inspect and explain to engineers.

Train a supervised fault classifier if you have representative fault data. If faults are rare, train a normal/anomaly model and output an anomaly score rather than pretending to diagnose every fault.

## 6.4 Offline signal pipeline

Sensor samples
  -> de-mean / calibration
  -> optional band-pass / resampling
  -> fixed time window
  -> FFT / log-mel spectrogram OR raw 1D tensor
  -> quantized neural network
  -> class probabilities / anomaly score
  -> temporal smoothing
  -> health state

## 6.5 Suggested AI outputs

| Output | Meaning |
| --- | --- |
| Health score 0-100 | High-level maintenance indicator; should be trendable over days/weeks. |
| Normal / Warning / Alarm | Stable operational state for PLC/maintenance system. |
| Fault family probability | Example: imbalance / bearing-like / looseness / cavitation, only if training data supports those classes. |
| Confidence / OOD flag | Indicates that current signal does not resemble known training conditions. |
| Trend features | RMS, spectral-band energy and model score for engineering diagnosis. |

## 6.6 Firmware tasks

High-rate SPI acquisition from vibration sensor with DMA/FIFO handling.

Microphone capture and synchronized timestamping.

Temperature sampling at a slower rate.

Window manager with no data loss.

Local DSP preprocessing if required by the network.

AI inference and multi-sensor score fusion.

Ring-buffer history to preserve several seconds before/after an alarm.

Nonvolatile configuration: sensor orientation, thresholds, machine ID.

Communication service for offline factory networks: UART/RS-485/Ethernet depending on product SKU.

## 6.7 Validation targets

| Test | What to verify |
| --- | --- |
| Repeatability | Same healthy machine under same load gives stable score over hours/days. |
| Operating-range robustness | Load and RPM changes do not become false failures if those regimes are normal. |
| Mounting sensitivity | Bolt/magnet/adhesive mounting changes frequency response; define a production mounting method. |
| Early-fault detection | Warning should rise before catastrophic failure, but not on every transient. |
| Sensor fault | Disconnected/saturated sensor must be reported as sensor failure, not machine failure. |
| Temperature extremes | Electronics and model behavior remain stable over required environment. |

## 6.8 Mass-production transition

Mechanical coupling is part of the sensor design. Specify mounting torque, surface preparation and orientation.

Use a smaller custom board with STM32N657 only if the AI workload truly needs N6; for very simple vibration models, a smaller STM32 may later reduce cost. Keep the N6 prototype to prove the algorithm first.

Factory-calibrate sensor offsets and write calibration coefficients during end-of-line programming.

Create a vibration self-test / known-excitation production test if possible.

Ship machine-specific model/config packs without changing the hardware SKU where practical.

# 7. Project 3 - AI Forklift & Warehouse Proximity Warning Unit

Goal: a retrofit unit mounted on a forklift, tug, AGV or warehouse vehicle. It detects people/vehicles with the camera and uses a near-field ToF sensor to strengthen distance/risk estimates. It can drive local warning indicators and send events to a fleet/warehouse system.

Safety boundary: Treat the first product as an assistive warning/awareness device, not as a safety-certified protective device or emergency-stop system. If you later claim functional-safety performance, the entire hardware/software/system architecture and certification plan must be redesigned for the applicable standards.

## 7.1 Prototype architecture

RGB camera -----------
                       \
VL53L8CX ToF ------------> STM32N657 -> person/vehicle detection + tracking + distance fusion -> risk zone
                       /
Optional IMU ----------
                         |
                         +--> buzzer / warning LEDs
                         +--> event log
                         +--> Ethernet/CAN/RS-485 gateway as product requires

## 7.2 What AI adds

Camera AI answers: “Is that object a person, forklift, pallet, cart or irrelevant background?”

Tracking answers: “Is it the same person and are we closing distance?”

ToF answers: “Is there a near object in this direction and approximately how far away is it?”

Optional IMU answers: “Is the vehicle turning, accelerating or stationary?”

Fusion converts those signals into a stable warning level rather than triggering from one noisy sensor.

## 7.3 Offline dataset plan

Mount the camera at the intended height and field of view before collecting the main dataset.

Record empty aisles, workers in different PPE/clothing, crouching/bending people, pallet stacks, forklifts, carts, reflective floors, low light and backlight.

Label people and relevant vehicle classes. If the product only needs person warning, keep the class set small at first.

Collect difficult negatives: posters with people, mannequins, reflections, TV screens, high-visibility jackets hanging on racks.

Record ToF data and video with synchronized timestamps for risk-fusion tuning.

Keep site-separated test data. A model trained and tested on the same warehouse can look better than it really is.

## 7.4 Warning logic example

| State | Example rule | Output |
| --- | --- | --- |
| Clear | No relevant tracked person in warning region | No warning |
| Awareness | Person detected; trajectory not closing or distance uncertain | Visual indicator |
| Warning | Person detected + near ToF return / closing track | Buzzer + amber/red indicator |
| Critical proximity | Strong person detection + near distance + closing motion | Highest local warning; log event |
| Sensor degraded | Camera blocked, ToF unavailable or model not running | Fault indicator; never silently report “clear” |

## 7.5 Firmware tasks

Camera capture + NPU person/vehicle detection.

Object tracking between frames.

VL53L8CX I2C data acquisition and zone mapping.

Calibration mapping between camera image regions and ToF zones.

Optional IMU acquisition and motion state.

Risk-state machine with hysteresis so warnings do not chatter.

Camera/ToF fault detection and degraded-mode behavior.

Event ring buffer with timestamp and reason code.

Local configuration for warning zones, mounting side and vehicle type.

## 7.6 Validation targets

| Test | What to verify |
| --- | --- |
| Person recall | Standing, walking, crouching, partly occluded and varied clothing/PPE. |
| False warning rate | Pallets, posts, signage, reflections and static objects should not create constant alarms. |
| Range behavior | Known test distances across ToF field of view and lighting conditions. |
| Motion behavior | Turning/bumps/vibration do not destabilize tracking. |
| Environmental | Dust, rain/splash if applicable, darkness/backlight, cold/heat. |
| Fault behavior | Blocked lens/disconnected sensor creates fault state, not false “safe” state. |

# 8. Moving From Discovery Kit to Your Own STM32N6 PCB

Do not begin with a custom STM32N6 PCB. The N6 design includes high-speed camera and external-memory interfaces, power sequencing and boot requirements. Prove the AI workload, sensor choices and memory requirements on the Discovery kit first.

## 8.1 Custom-board blocks

| Block | Design requirement |
| --- | --- |
| STM32N657 | Choose package based on required interfaces and PCB capability. |
| Boot flash | External serial flash sized for bootloader, application, AI model(s), assets and update margin. |
| External RAM | Add only if internal SRAM and model buffering require it; high-speed layout is critical. |
| Power | Follow STM32N6 power-domain and sequencing guidance in AN5967/AN6000 and datasheet. |
| Clock/reset | Follow reference design, boot mode and reset recommendations. |
| Camera | MIPI CSI-2/high-speed routing and sensor clock/power require careful PCB/layout work. |
| Debug/programming | Expose SWD, reset, target voltage reference and ground. Prefer STDC14 or pogo-pad footprint. |
| Industrial I/O | Use proper level shifting, isolation, ESD/surge protection and 24 V front-end design as required. |
| EMC/ESD | Design for the product environment from the start, not as a final patch. |

## 8.2 Hardware design documents to keep offline

RM0486 STM32N6 reference manual.

The exact STM32N657 device datasheet for your package.

AN5967 - hardware-development guidance for STM32N6.

AN6000 - discrete power-supply guidance.

STM32N6570-DK schematics/BOM/layout files where ST makes them available.

Latest errata sheet for the silicon revision you buy.

## 8.3 First custom PCB should be a carrier, not a full redesign

A good intermediate step is to keep the STM32N6570-DK as compute and build a custom sensor/I/O carrier board. This lets you validate 24 V inputs, RS-485/CAN, lights, triggers, ToF, vibration sensors and enclosure wiring before taking on the high-speed STM32N6 PCB itself.

# 9. Offline Programming, Debugging and Production Flashing

## 9.1 Prototype stage

Developer PC (offline)
  -> STM32CubeIDE build
  -> ELF/BIN/ST image
  -> on-board STLINK-V3EC
  -> STM32N6570-DK

## 9.2 Custom PCB / pilot build

Offline engineering PC
  -> STM32CubeProgrammer GUI/CLI
  -> STLINK-V3SET or STLINK-V3MINIE
  -> SWD pogo/header
  -> STM32N657 custom board
  -> external flash loader programs boot/application/model

## 9.3 Production line concept

Create a locked release directory. Never program directly from a developer build folder.

Generate a manifest containing binary filenames, expected hashes, firmware/model versions and target board revision.

Use a pogo-pin fixture or robust connector so every unit is wired identically.

Use STM32CubeProgrammer CLI scripts for erase/program/verify and option-byte operations that are part of the approved manufacturing flow.

Program a unique serial number / calibration record. Secure provisioning must follow a formal security design; do not improvise OTP/fuse programming.

Boot the product and run an automated self-test: sensor IDs, RAM/flash check, camera frame, microphone/vibration data, network/I/O, inference test vector.

Store the production result in a local factory database or CSV if the production network is intentionally offline.

Print/laser the serial number and link it to board revision, test result and firmware/model version.

## 9.4 What the writer/programmer does

The programmer is not an AI-specific device. It is simply the hardware bridge that lets the PC access the MCU debug/programming interface. The DK has this bridge on-board. Your custom PCB does not, so you connect an external ST-LINK probe to SWD/JTAG or use an approved bootloader interface. STM32CubeProgrammer can program STM32N6 through ST-LINK and supports N6 external-flash workflows.

# 10. Practical 12-Week Offline Prototype Plan

| Weeks | Project 1 - Quality | Project 2 - Machine Health | Project 3 - Forklift/Warehouse |
| --- | --- | --- | --- |
| 1-2 | Toolchain, DK bring-up, camera examples, lighting fixture | Toolchain, DK bring-up, microphone + vibration sensor communication | Toolchain, DK bring-up, camera + ToF communication |
| 3-4 | Collect/label first dataset; train baseline | Build test rig; collect healthy regimes | Collect/label people/warehouse data; synchronize ToF |
| 5-6 | Quantize + first N6 deployment | Build feature pipeline + baseline model | Quantize + first person detector on N6 |
| 7-8 | Trigger/I/O + real moving line mockup | Deploy vibration/audio model to N6 | Tracking + ToF fusion + warning state machine |
| 9-10 | False accept/reject testing; hard negatives | Fault/anomaly testing; mounting sensitivity | False-warning testing; occlusion/lighting/motion tests |
| 11-12 | Enclosure/carrier design + release candidate | Enclosure/mounting + release candidate | Vehicle-mount prototype + release candidate |

## 10.1 Do not run all three in parallel at the beginning

If the team is new to embedded development, finish a full loop on Project 1 first: dataset -> model -> STM32N6 inference -> I/O -> field test. Reusing that engineering discipline will make Projects 2 and 3 much faster.

# 11. “Ready to Start” Checklist

## 11.1 Offline software bundle

☐ STM32CubeIDE installer archived

☐ STM32CubeMX installer archived

☐ STM32CubeProgrammer installer archived

☐ STM32CubeN6 firmware package archived

☐ ST Edge AI Core installer/package archived

☐ STM32 model-zoo repository snapshot archived

☐ Pretrained model files archived

☐ Python installer + wheelhouse + requirements lock archived

☐ Offline annotation tool installer/container archived

☐ Official PDFs and latest errata archived

☐ Checksums generated for the bundle

☐ Backup copy stored on a second drive

## 11.2 Prototype hardware bundle

☐ 2 x STM32N6570-DK

☐ USB cables and stable USB/power setup

☐ Digital multimeter

☐ LED lighting + fixture materials for Project 1

☐ IIS3DWB vibration evaluation hardware for Project 2

☐ STTS22H temperature evaluation hardware for Project 2

☐ VL53L8CX ToF evaluation hardware for Project 3

☐ Basic breadboard/wiring/terminal parts

☐ Bench supply and oscilloscope if budget allows

☐ STLINK-V3SET/V3MINIE scheduled for purchase before custom PCB

## 11.3 Engineering exit criteria before custom PCB

☐ Model accuracy validated on a genuinely separate test set

☐ Quantized model accuracy accepted

☐ ST Edge AI memory and latency fit confirmed on STM32N6570-DK

☐ Sensor drivers stable for multi-hour run

☐ Watchdog/recovery tested

☐ Power-cycle and fault-state behavior tested

☐ Production I/O requirements frozen

☐ External flash/RAM size frozen with update margin

☐ Enclosure/optics/mounting concept validated

☐ Custom-board debug/programming connector defined

# 12. Which Project Should Become Your First Product?

| Criterion | Project 1 Quality Inspection | Project 2 Machine Health | Project 3 Forklift/Warehouse |
| --- | --- | --- | --- |
| Fastest STM32N6 demo | Best | Medium | Medium |
| Uses N6 camera/NPU strongly | Excellent | Moderate | Excellent |
| Needs mechanical/optical control | High | Medium | High |
| Needs difficult field dataset | Medium-High | High | High |
| Regulatory/safety burden | Moderate | Moderate | High if marketed as safety device |
| Broad factory demand | Very high | Very high | High |
| Potential units per customer | Several to dozens per line/site | Dozens to hundreds per site | Several to hundreds per fleet/site |
| Recommended first product | YES | Second | Third |

Recommendation: start commercial discovery around Project 1 while technically prototyping Project 2 in parallel only after your first engineers can reliably build, flash and debug STM32N6 firmware. Treat Project 3 as the most advanced product because sensor fusion, vehicle mounting and safety expectations increase system complexity.

# 13. Official References Used for This Guide

[R1] STM32N6570-DK Data Brief - Board features; on-board STLINK-V3EC; flash/PSRAM; camera/audio/communications.
https://www.st.com/resource/en/data_brief/stm32n6570-dk.pdf

[R2] AN6265 - Getting started with STM32N6 in STM32CubeIDE - STM32CubeIDE/CubeN6/CubeMX N6 development workflow.
https://www.st.com/resource/en/application_note/an6265-getting-started-with-stm32n6-mcus-in-stm32cubeide-stmicroelectronics.pdf

[R3] ST Edge AI Core Getting Started - Local CLI can analyze models, generate optimized C and validate on STM32N6570-DK.
https://dev.st.com/edge-ai-docs/general/latest/en/docs/tutorials/st-edge-ai-core-getting-started.html

[R4] STM32 AI Model Zoo Services - Supported use cases and STM32N6570-DK model/deployment examples.
https://github.com/STMicroelectronics/stm32ai-modelzoo-services

[R5] STM32CubeProgrammer documentation - Programming/debug/bootloader/external-memory capabilities.
https://dev.st.com/stm32cube-docs/prog/latest/en/index.html

[R6] STM32N6 external flash programming - N6 programming through ST-LINK or BootROM/external flash workflow.
https://dev.st.com/stm32cube-docs/prog/2.23.0/en/docs/markup/CubeProg_UserManual/External_flash_memory_window_for_STM32N6.html

[R7] AN5967 - STM32N6 hardware development - Power, clock, boot, reset, debug and hardware-reference guidance.
https://www.st.com/resource/en/application_note/an5967-getting-started-with-hardware-development-for-stm32n6-mcus-stmicroelectronics.pdf

[R8] AN6000 - STM32N6 discrete power supply - Power architecture and boot/power sequencing guidance.
https://www.st.com/resource/en/application_note/an6000-how-to-build-the-discrete-power-supply-for-stm32n6-mcus-stmicroelectronics.pdf

[R9] STM32N6 documentation portal - Current reference manual, datasheet, errata and application notes.
https://www.st.com/en/microcontrollers-microprocessors/stm32n6-series/documentation.html

[R10] IIS3DWB product page - Industrial wide-band vibration sensor; up to 6 kHz bandwidth and 26.7 kHz ODR.
https://www.st.com/en/mems-and-sensors/iis3dwb.html

[R11] STTS22H product page - Factory-calibrated digital temperature sensor.
https://www.st.com/en/mems-and-sensors/stts22h.html

[R12] VL53L8CX product page - 8x8 multizone ToF; multitarget ranging up to 400 cm.
https://www.st.com/en/imaging-and-photonics-solutions/vl53l8cx.html

[R13] STLINK-V3SET product page - Standalone SWD/JTAG programmer/debugger for custom STM32 boards.
https://www.st.com/en/development-tools/stlink-v3set.html

[R14] STLINK-V3MINIE product page - Compact standalone STM32 SWD/JTAG programmer/debugger.
https://www.st.com/en/development-tools/stlink-v3minie.html

Version note: ST continuously updates STM32CubeProgrammer, STM32CubeIDE, ST Edge AI, firmware packages and errata. For an offline product program, archive the exact validated versions and documentation used for each release. Before a new production revision, reconnect a controlled engineering machine and review current errata/tool releases.